Ambient Air Quality Data of Bengaluru Stations For the month of July, 2021

AQI Bulletin

Karnataka State Pollution Control Board No.49, Parisara Bhavan, Church Street, Bengaluru-560 001.

Website: http://kspcb.karnataka.gov.in

I N D E X

| Sl. No. | Content | Page No. |
| --- | --- | --- |
| 1 | Introduction | 1 |
| 2 | Site of CAAQM Stations operated by KSPCB in Bengaluru | 1 |
| 3 | Parameter-wise data tables of CAAQM Stations | 2,3,4,5 |
| 4 | Daily AQI values of CAAQMS in Bengaluru-(July-2021) | 6 |
| 5 | AQI Trend Bengaluru (July 2021) | 7 |
| 6 | Concentration ranges of Ambient Air Quality Parameters of Bengaluru Stations | 8 |
| 7 | Air Quality Index, | 8 |
| 8 | Meteorological parameters, Windrose diagrams | 9 |
| 9 | Broad guidelines for Public | 11 |
| 10 | Annexure |  |
|  | National Ambient Air Quality Standard (2009) |  |
|  | List of Monitoring Stations with parameters |  |

Introduction: Ambient Air Quality of Bengaluru CAAQM Stations

Bengaluru is known for its cool and pleasant climate, because of its high elevation of around  800-900 meters above sea level. The annual average temperate ranges from 23 to 26  throughout the year. However, in view of rapid urbanization, Industrialisation and increase  of vehicles, the air quality is getting disturbed compared to past years. Hence, in order to  determine the pollutants, its nature, quality and quantity in the Ambient Air and source of  emission generated, the KSPCB has installed 7 CAAQM (Continuous Ambient Air quality  monitoring Stations) in Bengaluru and the monitoring is done on 24 hours basis for PM10,  PM2.5, SO2, NO2, Ammonia, O3, CO and Benzene and the compiled statistical Data is sent to  CPCB, New Delhi electronically and also the data is published in the Board Web Site.

Site map of CAAQM stations operated by KSPCB in Bengaluru

CAAQM Stations: Hebbal, Jayanagar, KAVIKA, NIMHANS, Silkboard, Nisarga Bhavan (Basaveshwaranagar), City Railway Station(CRS)

1

Parameter-wise data tables of CAAQM Stations

I) Hebbal

II) Jayanagar

2

III) KAVIKA

IV) NIMHANS

3

V) Silk board

VI) Nisarga Bhavan

4

VII) City Railway Station.

5

Daily AQI values of CAAQM Stations in Bengaluru(July-2021)

6

AQI Trend Bengaluru, July 2021

Hebbal, Jayanagar, KAVIKA, NIMHANS, Silkboard, Nisarga Bhavan (Basaveshwaranagar),  City Railway Station(CRS)

Data Analysis of Ambient Air Quality:

❖ Particulate Matter (PM10): Recorded within permissible limit at Hebbal, Jayanagar,  KAVIKA, NIMHANS, Nisarga Bhavan and City Railway Station as per NAAQS 2009  standards. However there is a slight increase in PM10 values was observed in last 6 days  at Silkboard station and that maybe due to high vehicular movement at that particular  period of time.

❖ Particulate Matter (PM2.5): Recorded within permissible limit in all the five monitored  station viz., Hebbal, Jayanagar, KAVIKA, NIMHANS & Silkboard as per NAAQS 2009  standards.

❖ Sulphur Dioxide(SO2): Recorded within permissible limit in all stations across Bengaluru  as per NAAQS 2009 standards.

❖ Nitrogen Dioxide(NO2): Recorded within permissible limit in all stations across  Bengaluru as per NAAQS 2009 standards.

❖ Ammonia(NH3): Observed within permissible limit in all the five monitored stations viz.,  Hebbal, Jayanagar, KAVIKA, NIMHANS & Silk board as per NAAQS 2009 standards.

❖ Carbon Monoxide(CO): Observed 8-hourly concentration values within the permissible  limit in all stations across Bengaluru as per NAAQS 2009 Standard.

❖ Ozone(O3): Observed within permissible limit in all the five monitored stations viz.,  Hebbal, Jayanagar, KAVIKA, NIMHANS & Silk board as per NAAQS 2009 standards.  7

Concentration ranges of Ambient Air Quality Parameters of Bengaluru Stations

The concentration ranges for pollutants of CAAQM stations having 24 hourly standard limits  are presented in below table based on detailed tabulated date.

| Table-1 Range of 24-hourly Averages for Notified Parameters monitored in July 2021, Bengaluru |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameters | Hebbal |  | Jayanagar |  | KAVIKA |  | NIMHANS |  | Silk board |  | Nisarga Bhavan |  | City Railway station |  |
|  | MIN | MAX | MIN | MAX | MIN | MAX | MIN | MAX | MIN | MAX | MIN | MAX | MIN | MAX |
| PM10(µg/m3) | 13 | 44 | 14 | 56 | 21 | 86 | 14 | 52 | 25 | 139 | 33.31 | 54.83 | 38.22 | 73.53 |
| PM2.5(µg/m3) | 4 | 20 | 7.0 | 22.0 | 9 | 56 | 7 | 25 | 6 | 21 | - | - | - | - |
| SO2(µg/m3) | 3.8 | 7.9 | 3.5 | 8.0 | 3.1 | 17.6 | 5.1 | 9.5 | 2.5 | 8.7 | 3.20 | 12.95 | 5.3 | 7.82 |
| NO2(µg/m3) | 1.1 | 1.4 | 14.9 | 34.0 | 9.8 | 20.9 | 7.9 | 17.2 | 12.9 | 51.1 | 4.61 | 15.98 | 11.23 | 36.18 |
| NH3(µg/m3) | 4.0 | 7.7 | 7.9 | 12.7 | 18.9 | 32.9 | 1.7 | 10.0 | 9.3 | 21.2 | - | - | - | - |
| Note: * Insufficient Data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CO, Ozone and Benzene not included as there is no 24 hourly permissible limits in NAAQM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Air Quality Index(AQI)

AQI of Bengaluru was found largely lying in Good &Satisfactory at all locations of  Bengaluru. However, there was slight increase in PM10 values in the last six days monitored  at Silk board and that may be due high vehicular movement in that particular period. Moderate rainfall during July-2021 has also attributed for improved air quality in Bengaluru  city to some extent.

| Table-2 AQI Values of CAAQM stations in Bengaluru for the month of July 2021 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AQI Categories | Range | Hebbal | Jayanagar | KAVIKA | NIMHANS | Silkboard | Nisarga Bhavan | City Railway station |
| Good | (0-50) | 31 | 3 | 19 | 27 | 5 | 24 | 3 |
| Satisfactory | (51-100) | - | 1 | 8 | 1 | 5 | 7 | 28 |
| Moderate | (101-200) | - | - | - | - | 7 | - | - |
| Poor | 201-300 | - | - | - | - | - | - | - |
| Very Poor | 301-400 | - | - | - | - | - | - | - |
| Severe | (> 401) | - | - | - | - | - | - | - |

Meteorological Parameters

Daily average wind speed was observed in the range 0.3 m/s – 2.5 m/s. Monthly average  temperature was 23.71˚C with minimum daily average as 18.8˚C and maximum as 29˚C  recorded. Monthly average relative humidity was 70.45% with maximum daily average as 89 % and minimum as 33.2% recorded.

| Table-4 Monthly Range and Average for Metrological Parameters in Bengaluru, July 2021 |  |  |  |
| --- | --- | --- | --- |
| Parameters(Unit)* | Average | Maximum | Minimum |
| Wind Speed(m/s) | 1.33 | 2.5 | 0.3 |
| Temperature(oC) | 23.71 | 29 | 18.8 |
| Relative Humidity(%) | 70.45 | 89 | 33.2 |

* Data of 6 Stations 8

Windrose diagrams: The graphical charts that characterise the speed and direction of wind  at the CAAQM Stations.

| 9 |  |
| --- | --- |

NATIONAL AMBIENT AIR QUALITY STANDARDS Concentration in Ambient Air

Sl.

No. Pollutants

Time

Weighted  Average

Industrial,  Residential  Rural and  other Areas

Ecologically  Sensitive Area  (Notified by  Central

Government)

Methods of Measurement

| 1 | Sulphur Dioxide (SO2) µg/m3 | Annual * | 50 | 20 | -Improved west and Gaeke Method – Ultraviolet Fluorescence |
| --- | --- | --- | --- | --- | --- |
|  |  | 24 Hours** | 80 | 80 |  |
| 2 | Nitrogen Dioxide (NO2) µg/m3 | Annual * | 40 | 30 | -Jacob & Hochheiser Modified (NaOH NaAsO2) Method -Gas phase Chemiluminescence |
|  |  | 24 Hours** | 80 | 80 |  |
| 3 | Particulate Mater (Size less than 10 µm) or PM10 µg/m3 | Annual * | 60 | 60 | -Gravimetric -TECOM -Beta attenuation |
|  |  | 24 Hours** | 100 | 100 |  |
| 4 | Particulate Mater (Size less than 10 µm) or PM 2.5 µg/m3 | Annual * | 40 | 40 | -Gravimetric -TECOM -Beta attenuation |
|  |  | 24 Hours** | 60 | 60 |  |
| 5 | Ozone (O3) µg/m3 | 8 Hours * | 100 | 100 | -UV Photometric -Chemical Method |
|  |  | 1 Hours** | 180 | 180 |  |
| 6 | Lead (Pb) µg/m3 | Annual * | 0.5 | 0.5 | -AAs/ICP Method after sampling on EPM 2000 or equivalent filter paper -ED-XRF using Teflon filter |
|  |  | 24 Hours** | 1 | 1 |  |
| 7 | Carbon Monoxide (CO) µg/m3 | 8 Hours * | 02 | 02 | -Non dispersive Infrared (NDIR) -Spectroscopy |
|  |  | 1 Hours** | 04 | 04 |  |
| 8 | Ammonia (NH3) µg/m3 | Annual * | 100 | 100 | -Chemiluminescence -Indophenol Blue Method |
|  |  | 24 Hours** | 400 | 400 |  |
| 9 | Benzene (C6H6) µg/m3 | Annual * | 05 | 05 | -Gas Chromatography (GC) based continuous analyzer -Adsorption and desorption followed by GC analysis |
| 10 | Benzo (a) Pyrene (BaP) µg/m3 | Annual * | 01 | 01 | -Solvent extraction followed by HPLC/GC analysis |
| 11 | Arsenic (As) µg/m3 | Annual * | 06 | 06 | -AAS/ICP Method after sampling on EPM 2000 or equivalent filter paper |
| 12 | Nickel (Ni) ng/m3 | Annual * | 20 | 20 | -AAS/ICP Method after sampling on EPM 2000 or equivalent filter paper |

CAAQM STATIONS

| Sl. No. | Stations | Types of activities around location (Residential/ Commercial/ Traffic/Industrial) | Parameters Monitored |
| --- | --- | --- | --- |
| 1 | Hebbal | Sensitive | SO2, NO2, PM10, PM2.5, NH3, CO, O3, Benzene& Meteorological parameters |

10

| 2 | Jayanagar | Commercial | SO2, NO2, PM10, PM2.5, NH3, CO, O3, Benzene& Meteorological parameters |
| --- | --- | --- | --- |
| 3 | KAVIKA | Commercial | SO2, NO2, PM10, PM2.5, NH3, CO, O3, Benzene& Meteorological parameters |
| 4 | NIMHANS | Sensitive | SO2, NO2, PM10, PM2.5, NH3, CO, O3, Benzene& Meteorological parameters |
| 5 | Silkboard | Residential cum Commercial | SO2, NO2, PM10, PM2.5, NH3, CO, O3, Benzene& Meteorological parameters |
| 6 | Nisarga Bhavan | Residential | SO2, NO2, PM10, CO |
| 7 | City Railway Station | Commercial | SO2, NO2, PM10, CO & Meteorological parameters |

INFERENCE: The AQI of CAAQMS in Bengaluru was Good and Satisfactory. Broad guidelines for Public

AQI is an initiative intended to enhance public awareness and involvement in efforts to  improve air quality. People can contribute by maintaining vehicles properly (e.g. get PUC  checks, replace car air filter, maintain right tyres pressure), following lane discipline & speed  limits, avoiding prolong idling and turning off engines at red traffic signals. The following  are some of the best practices that are to be followed to maintain/ improve the Air Quality.

1) Avoid using private vehicles, bikes and instead use public transports viz., Public Buses and Metro services.

2) Encourage carpool and use smaller vehicles (e.g. avoid SUVs).

3) Construction projects shall compulsorily put up enclosures and barriers around their project and carry out regular water sprinkling to suppress dust. Air purifier can also be  installed to mitigate dust pollution.

4) Road dust management by using mechanized road sweeping and water sprinkling  system, etc., The Civic Bodies shall regularly remove the silt and muck dumped on  the roadside and pavements, besides levelling & Asphalting of Roads and filling up of  potholes should be taken up on top priority.

5) Unnecessary parking of vehicles on roadside junctions and circles should be avoided of around 50 to 100 meters.

6) Avoid open burning of garbage wastes, tree leaves, branches, trash, tyres etc., especially near roadsides, lakes & water bodies, open ground, vacant land & Parks.  ====********==== 11

Plant Trees, Save Environment.