KARNATAKA STATE POLLUTION CONTROL BOARD No.49, Parisara Bhavan, Church Street, Bengaluru-560 001.

Air Quality Monitoring Cell

Air Quality Index of CAAQM Stations monitored in Bengaluru city-November-2022 Air Quality Index-(AQI)

Air Quality Index is a tool for effective communication of air quality status to people in  terms, which are easy to understand. It transforms complex air quality data of various  pollutants into a single number (index value), nomenclature and colour.

There are six AQI categories, namely Good, Satisfactory, Moderate, Poor,  Very Poor, and Severe. Each of these categories is decided based on ambient  concentration values of air pollutants and their likely health impacts (known as health  breakpoints). AQ sub-index and health breakpoints are evolved for eight pollutants (PM10,  PM2.5, NO2, SO2, CO, O3, NH3, and Pb) for which short-term (upto 24-hours)

National Ambient Air Quality Standards are prescribed.

Based on the measured ambient concentrations of a pollutant, sub-index is  calculated, which is a linear function of concentration (e.g. the sub-index for PM2.5 will  be 51 at concentration 31 µg/m3, 100 at concentration 60 µg/m3, and 75 at concentration  of 45(µg/m3). The worst sub-index determines the overall AQI. AQI categories and  health breakpoints for the eight pollutants are as follow:

The Karnataka State Pollution Control Board has installed 7 Continuous  Ambient Air Quality Monitoring Stations (CAAQMs) covering Residential,  Commercial, Mixed and Silence Zones. The Air Quality is being monitored round the  clock, 24 hours a day for the Primary and secondary pollutants as well as Meteorological

parameters. The Air Quality monitored during the month of November-2022 is as  shown below:

| Daily AQI Values of Bengaluru (November-2022) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date/Name of station | City Railway Station, Majestic | Nisarga Bhavan, Saneguruvanahalli | Veterinary College, Hebbal | Shalini Ground, Jayanagar | KAVIKA, Mysore Road | NIMHANS | H.S.R Layout, Near Central Silkboard |
| 01-11-2022 | 74 | 50 | 47 | 53 | 80 | 33 | 52 |
| 02-11-2022 | 73 | 43 | 40 | 39 | 74 | 27 | 63 |
| 03-11-2022 | 73 | 46 | 53 | 47 | 91 | 33 | 60 |
| 04-11-2022 | 74 | 50 | 54 | 44 | 81 | 30 | 45 |
| 05-11-2022 | 90 | 47 | 61 | 71 | 83 | 40 | 45 |
| 06-11-2022 | 78 | 48 | 91 | 99 | 153 | 74 | 105 |
| 07-11-2022 | 86 | 47 | 101 | 102 | 209 | 93 | 119 |
| 08-11-2022 | 103 | 46 | 86 | 99 | 178 | 83 | 109 |
| 09-11-2022 | 104 | 48 | 103 | 106 | 210 | 97 | 126 |
| 10-11-2022 | 95 | 53 | 80 | 88 | 139 | 61 | 90 |
| 11-11-2022 | 89 | 60 | 139 | 110 | 234 | 92 | 127 |
| 12-11-2022 | 78 | 49 | 38 | 40 | 79 | 46 | 32 |
| 13-11-2022 | 78 | 49 | 46 | 28 | * | 23 | 24 |
| 14-11-2022 | 90 | 45 | 53 | 46 | 84 | 36 | 37 |
| 15-11-2022 | 105 | 51 | 99 | 97 | 230 | 89 | 109 |
| 16-11-2022 | 75 | 48 | 147 | 119 | 280 | 108 | 138 |
| 11-11-2022 | 77 | 56 | 103 | 109 | 240 | 103 | 139 |
| 18-11-2022 | 85 | 44 | 92 | 105 | 209 | 94 | 131 |
| 19-11-2022 | 90 | 36 | 93 | 103 | 200 | 92 | 126 |
| 20-11-2022 | 93 | 39 | 106 | 125 | 202 | 99 | 137 |
| 21-11-2022 | 84 | 40 | 78 | 93 | 223 | 78 | 113 |
| 22-11-2022 | 73 | 40 | 35 | 54 | 105 | 34 | 62 |
| 23-11-2022 | 77 | 45 | 27 | 41 | * | 34 | 58 |
| 24-11-2022 | 83 | 44 | 63 | 52 | 146 | 48 | 69 |
| 25-11-2022 | 93 | 41 | 77 | 68 | 107 | 54 | 73 |
| 26-11-2022 | 81 | 42 | 80 | 89 | 133 | 65 | 84 |
| 27-11-2022 | 78 | 41 | 77 | 80 | 145 | 66 | 107 |
| 28-11-2022 | 93 | 40 | 143 | 147 | 301 | 110 | 150 |
| 29-11-2022 | 92 | 38 | 159 | 166 | 291 | 113 | 164 |
| 30-11-2022 | 86 | 40 | 106 | 121 | 280 | 104 | 141 |
| Min | 73 | 36 | 27 | 28 | 74 | 23 | 24 |
| Max | 105 | 60 | 159 | 166 | 301 | 113 | 164 |

* Data not available

| Good | Satisfactory | Moderate | Poor | Very Poor | Severe |
| --- | --- | --- | --- | --- | --- |
| (0-50) | (51-100) | (101-200) | (201-300) | (301-400) | (>401) |

Possible Health Impacts

| Good | Minimal Impact |
| --- | --- |
| Satisfactory | Minor breathing discomfort to sensitive people |
| Moderate | Breathing discomfort to the people with lungs, asthma and heart diseases |
| Poor | Breathing discomfort to most people on prolonged exposure |
| Very Poor | Respiratory illness on prolonged exposure |
| Severe | Affects healthy people and seriously impacts those with existing diseases |

Daily AQI of CAAQM Stations

| City Railway Station-Nov-2022 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Date | NO2 ug/m3 | SO2 ug/m3 | CO mg/m3 | PM10 ug/m3 | AQI |
| 01-11-2022 | 23.6 | 17.4 | 1.47 | 61.4 | 74 |
| 02-11-2022 | 23.6 | 17.4 | 1.46 | 53.7 | 73 |
| 03-11-2022 | 23.6 | 17.4 | 1.44 | 63.7 | 73 |
| 04-11-2022 | 23.6 | 17.4 | 1.48 | 64.9 | 74 |
| 05-11-2022 | 23.5 | 17.4 | 1.79 | 72.8 | 90 |
| 06-11-2022 | 23.6 | 17.4 | 1.29 | 77.5 | 78 |
| 07-11-2022 | 23.4 | 17.4 | 1.52 | 85.7 | 86 |
| 08-11-2022 | 23.7 | 17.4 | 2.22 | 93.0 | 103 |
| 09-11-2022 | 23.6 | 17.3 | 2.21 | 105.8 | 104 |
| 10-11-2022 | 23.5 | 17.4 | 1.89 | 86.4 | 95 |
| 11-11-2022 | 23.6 | 17.3 | 1.77 | 89.1 | 89 |
| 12-11-2022 | 23.6 | 17.4 | 1.55 | 51.4 | 78 |
| 13-11-2022 | 23.6 | 17.3 | 1.55 | 60.4 | 78 |
| 14-11-2022 | 23.6 | 17.4 | 1.76 | 89.6 | 90 |
| 15-11-2022 | 23.6 | 17.4 | 1.47 | 108.1 | 105 |
| 16-11-2022 | 23.6 | 20.6 | 1.37 | 74.7 | 75 |
| 17-11-2022 | 23.5 | 23.3 | 1.54 | 66.6 | 77 |
| 18-11-2022 | 23.5 | 23.3 | 1.59 | 85.3 | 85 |
| 19-11-2022 | 23.5 | 23.3 | 1.66 | 89.5 | 90 |
| 20-11-2022 | 23.6 | 23.4 | 1.62 | 93.2 | 93 |
| 21-11-2022 | 23.6 | 23.3 | 1.53 | 83.6 | 84 |
| 22-11-2022 | 23.6 | 23.4 | 1.45 | 61.8 | 73 |
| 23-11-2022 | 23.6 | 23.4 | 1.53 | 53.6 | 77 |
| 24-11-2022 | 23.6 | 23.3 | 1.66 | 60.2 | 83 |

25-11-2022 23.5 23.4 1.85 65.9 93

| 26-11-2022 | 23.5 | 23.4 | 1.61 | 78.2 | 81 |
| --- | --- | --- | --- | --- | --- |
| 27-11-2022 | 23.6 | 23.4 | 1.56 | 76.4 | 78 |
| 28-11-2022 | 23.6 | 23.3 | 1.86 | 91.6 | 93 |
| 29-11-2022 | 23.7 | 23.4 | 1.56 | 91.7 | 92 |
| 30-11-2022 | 23.6 | 23.4 | 1.61 | 85.7 | 86 |
| Minimum | 23.4 | 17.3 | 1.29 | 51.4 | * |
| Maximum | 23.7 | 23.4 | 2.22 | 108.1 | * |
| Average | 23.6 | 20.3 | 1.63 | 77.4 | * |

| Saneguruvanahalli-Nov-2022 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | NO2 ug/m3 | SO2 ug/m3 | CO mg/m3 | PM10 ug/m3 | TEMP oC | HR % | WS m/s | WD degree | SR W/m2 | AQI |
| 01-11-2022 | 20.8 | 12.2 | 0.74 | 50.4 | 24.53 | 75.60 | 0.12 | 257.90 | 247.65 | 50 |
| 02-11-2022 | 21.2 | 12.8 | 0.76 | 42.6 | 24.46 | 75.06 | 0.12 | 275.12 | 210.83 | 43 |
| 03-11-2022 | 24.6 | 8.5 | 0.77 | 46.2 | 24.88 | 73.19 | 0.12 | 200.54 | 556.58 | 46 |
| 04-11-2022 | 23.2 | 12.1 | 0.82 | 49.8 | 24.95 | 79.56 | 0.15 | 199.60 | 598.03 | 50 |
| 05-11-2022 | 22.1 | 7.0 | 0.75 | 47.0 | 24.67 | 78.96 | 0.17 | 189.59 | 379.71 | 47 |
| 06-11-2022 | 21.0 | 8.6 | 0.80 | 47.6 | 24.57 | 78.71 | 0.14 | 199.78 | 250.31 | 48 |
| 07-11-2022 | 21.7 | 7.0 | 0.81 | 46.7 | 24.70 | 78.85 | 0.14 | 197.74 | 302.14 | 47 |
| 08-11-2022 | 19.8 | 4.7 | 0.78 | 46.2 | 24.70 | 79.18 | 0.13 | 226.99 | 292.46 | 46 |
| 09-11-2022 | 23.6 | 10.3 | 0.96 | 43.4 | 25.11 | 77.67 | 0.14 | 181.29 | 576.98 | 48 |
| 10-11-2022 | 19.6 | 7.3 | 0.73 | 53.3 | 24.72 | 83.82 | 0.11 | 272.68 | 371.95 | 53 |
| 11-11-2022 | 20.4 | 6.5 | 0.76 | 59.5 | 24.88 | 80.75 | 0.15 | 235.18 | 527.21 | 60 |
| 12-11-2022 | 19.6 | 6.1 | 0.82 | 49.2 | 25.00 | 81.93 | 0.13 | 192.33 | 627.76 | 49 |
| 13-11-2022 | 17.9 | 3.9 | 0.80 | 49.1 | 24.73 | 84.05 | 0.17 | 189.19 | 427.70 | 49 |
| 14-11-2022 | 20.6 | 10.3 | 0.85 | 45.4 | 24.66 | 80.94 | 0.16 | 185.73 | 303.05 | 45 |
| 15-11-2022 | 19.4 | 5.6 | 0.75 | 50.5 | 24.52 | 74.26 | 0.15 | 200.73 | 236.08 | 51 |
| 16-11-2022 | 20.3 | 7.1 | 0.74 | 47.7 | 24.51 | 71.30 | 0.20 | 179.61 | 231.38 | 48 |
| 17-11-2022 | 21.1 | 5.8 | 1.12 | 46.5 | 24.55 | 71.90 | 0.18 | 246.07 | 249.79 | 56 |
| 18-11-2022 | 22.3 | 6.4 | 0.74 | 43.8 | 25.38 | 73.85 | 0.45 | 158.39 | 630.66 | 44 |
| 19-11-2022 | 21.4 | 6.0 | 0.72 | 36.2 | 24.89 | 74.20 | 0.64 | 56.20 | 285.98 | 36 |
| 20-11-2022 | 20.4 | 5.7 | 0.71 | 39.1 | 24.91 | 74.25 | 0.61 | 71.53 | 272.02 | 39 |
| 21-11-2022 | 22.2 | 13.6 | 0.70 | 39.8 | 25.28 | 76.21 | 0.58 | 138.77 | 289.64 | 40 |
| 22-11-2022 | 24.0 | 8.4 | 0.74 | 40.4 | 25.68 | 76.74 | 0.61 | 120.33 | 430.05 | 40 |
| 23-11-2022 | 24.9 | 7.0 | 0.78 | 45.1 | 25.89 | 75.70 | 0.68 | 37.37 | 426.38 | 45 |
| 24-11-2022 | 27.2 | 9.2 | 0.87 | 41.8 | 25.15 | 75.23 | 0.60 | 27.46 | 579.04 | 44 |
| 25-11-2022 | 22.9 | 7.0 | 0.81 | 40.7 | 25.12 | 87.33 | 0.48 | 22.99 | 545.19 | 41 |
| 26-11-2022 | 22.3 | 6.2 | 0.76 | 42.2 | 25.07 | 86.39 | 0.77 | 44.44 | 374.36 | 42 |
| 27-11-2022 | 20.5 | 6.1 | 0.76 | 41.4 | 25.03 | 85.44 | 0.78 | 109.09 | 365.98 | 41 |
| 28-11-2022 | 21.9 | 5.0 | 0.75 | 40.3 | 24.94 | 85.00 | 0.73 | 108.62 | 323.21 | 40 |

29-11-2022 21.0 4.8 0.79 32.8 25.04 84.57 0.73 108.21 360.00 38

| 30-11-2022 | 22.1 | 4.8 | 0.73 | 39.5 | 25.06 | 84.39 | 0.73 | 107.90 | 345.95 | 40 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Minimum | 17.9 | 3.9 | 0.70 | 32.8 | 24.46 | 71.30 | 0.11 | 22.99 | 210.83 | * |
| Maximum | 27.2 | 13.6 | 1.12 | 59.5 | 25.89 | 87.33 | 0.78 | 275.12 | 630.66 | * |
| Average | 21.7 | 7.5 | 0.79 | 44.8 | 24.92 | 78.83 | 0.36 | 158.05 | 387.27 | * |

| Hebbal-November-2022 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | CO (mg/m³) | Ozone (µg/m³) | NO2 (µg/m³) | NH3 (µg/m³) | SO2 (µg/m³) | PM2.5 (µg/m³) | PM10 (µg/m³) | BEN. (µg/m³) | AT (°C) | RH (%) | WS (m/s) | WD (deg) | BP (mmHg) | AQI |
| 01-11-2022 | 0.41 | 15.4 | 22.1 | 11.3 | 3.5 | 25.8 | 47.3 | 0.1 | 20.3 | 83 | 0.9 | 108 | 711 | 47 |
| 02-11-2022 | 0.40 | 5.9 | 24.7 | 8.9 | 3.5 | 18.5 | 39.5 | 0.1 | 20.0 | 84 | 0.9 | 114 | 713 | 40 |
| 03-11-2022 | 0.46 | 4.4 | 25.8 | 8.1 | 3.3 | 23.7 | 53.3 | 0.1 | 21.4 | 81 | 0.9 | 104 | 715 | 53 |
| 04-11-2022 | 0.51 | 5.5 | 26.6 | 7.9 | 3.0 | 21.8 | 53.8 | 0.1 | 22.4 | 76 | 0.9 | 110 | 713 | 54 |
| 05-11-2022 | 0.44 | 8.2 | 26.5 | 8.5 | 4.0 | 30.3 | 61.0 | 0.1 | 22.2 | 76 | 0.9 | 112 | 713 | 61 |
| 06-11-2022 | 0.41 | 11.4 | 24.0 | 9.6 | 3.8 | 54.5 | 85.7 | 0.1 | 21.6 | 79 | 0.8 | 101 | 712 | 91 |
| 07-11-2022 | 0.51 | 11.0 | 25.8 | 7.9 | 4.1 | 56.5 | 101.8 | 0.1 | 22.1 | 66 | 0.8 | 98 | 712 | 101 |
| 08-11-2022 | 0.41 | 13.7 | 26.3 | 8.8 | 3.1 | 50.3 | 86.0 | 0.1 | 22.6 | 66 | 0.8 | 101 | 712 | 86 |
| 09-11-2022 | 0.48 | 11.0 | 24.8 | 8.8 | 3.5 | 59.4 | 103.8 | 0.1 | 22.1 | 66 | 0.8 | 102 | 712 | 103 |
| 10-11-2022 | 0.38 | 8.8 | 23.8 | 8.4 | 3.3 | 47.7 | 77.8 | 0.1 | 22.0 | 71 | 0.9 | 118 | 711 | 80 |
| 11-11-2022 | 0.37 | 19.2 | 21.3 | 9.6 | 3.5 | 71.7 | 102.1 | 0.1 | 19.8 | 79 | 1.1 | 151 | 710 | 139 |
| 12-11-2022 | 0.37 | 4.8 | 23.5 | 7.3 | 3.1 | 22.8 | 37.0 | 0.1 | 19.4 | 90 | 0.9 | 122 | 710 | 38 |
| 13-11-2022 | 0.46 | 2.2 | 23.8 | 8.1 | 2.7 | 21.5 | 45.5 | 0.1 | 21.0 | 82 | 0.9 | 104 | 711 | 46 |
| 14-11-2022 | 0.44 | 4.4 | 25.5 | 7.5 | 2.8 | 26.8 | 53.0 | 0.1 | 21.6 | 80 | 0.9 | 105 | 712 | 53 |
| 15-11-2022 | 0.47 | 6.7 | 26.0 | 8.1 | 3.4 | 59.5 | 96.7 | 0.1 | 22.2 | 76 | 0.8 | 101 | 711 | 99 |
| 16-11-2022 | 0.36 | 11.7 | 22.9 | 8.9 | 3.4 | 74.0 | 111.8 | 0.1 | 21.9 | 73 | 0.9 | 105 | 711 | 147 |
| 17-11-2022 | 0.44 | 14.3 | 28.3 | 8.3 | 3.5 | 60.9 | 103.8 | 0.1 | 20.3 | 61 | 0.8 | 101 | 711 | 103 |
| 18-11-2022 | 0.44 | 12.1 | 29.1 | 7.4 | 3.6 | 50.3 | 91.7 | 0.1 | 20.3 | 59 | 0.8 | 95 | 711 | 92 |
| 19-11-2022 | 0.40 | 12.6 | 29.7 | 8.7 | 3.5 | 52.8 | 93.3 | 0.1 | 20.1 | 64 | 0.8 | 98 | 710 | 93 |
| 20-11-2022 | 0.35 | 18.8 | 21.5 | 9.6 | 3.5 | 61.9 | 94.7 | 0.1 | 18.3 | 67 | 0.9 | 119 | 709 | 106 |
| 21-11-2022 | 0.35 | 18.1 | 18.7 | 8.8 | 3.6 | 46.6 | 74.3 | 0.1 | 21.1 | 63 | 1.0 | 126 | 709 | 78 |
| 22-11-2022 | 0.28 | 2.9 | 18.0 | 9.3 | 3.5 | 19.8 | 35.1 | 0.1 | 20.1 | 83 | 0.9 | 120 | 709 | 35 |
| 23-11-2022 | 0.36 | 3.8 | 13.8 | 10.4 | 3.4 | 14.9 | 27.1 | 0.1 | 20.8 | 86 | 1.0 | 129 | 709 | 27 |
| 24-11-2022 | 0.48 | 2.2 | 28.6 | 8.9 | 3.5 | 35.5 | 63.1 | 0.1 | 20.6 | 87 | 0.9 | 114 | 710 | 63 |
| 25-11-2022 | 0.58 | 2.8 | 31.3 | 7.9 | 2.8 | 35.8 | 76.7 | 0.1 | 22.0 | 76 | 0.9 | 110 | 711 | 77 |
| 26-11-2022 | 0.43 | 4.2 | 30.1 | 8.2 | 3.1 | 41.1 | 80.3 | 0.1 | 22.7 | 72 | 0.9 | 110 | 709 | 80 |
| 27-11-2022 | 0.43 | 6.2 | 24.9 | 8.2 | 3.3 | 45.7 | 76.5 | 0.1 | 23.4 | 72 | 0.9 | 118 | 708 | 77 |
| 28-11-2022 | 0.43 | 10.3 | 26.9 | 13.1 | 3.5 | 72.8 | 113.6 | 0.1 | 22.9 | 72 | 0.9 | 104 | 708 | 143 |
| 29-11-2022 | 0.39 | 22.9 | 29.9 | 26.0 | 3.3 | 77.8 | 119.6 | 0.1 | 21.7 | 69 | 0.9 | 115 | 709 | 159 |
| 30-11-2022 | 0.49 | 9.7 | 33.8 | 16.7 | 4.1 | 61.8 | 108.4 | 0.1 | 21.0 | 67 | 0.9 | 106 | 711 | 106 |
| Average | 0.42 | 9.5 | 25.3 | 9.6 | 3.4 | 44.7 | 77.1 | 0.1 | 21.3 | 74 | 0.9 | 111 | 711 | * |
| Minimum | 0.28 | 2.2 | 13.8 | 7.3 | 2.7 | 14.9 | 27.1 | 0.1 | 18.3 | 59 | 0.8 | 95 | 708 | * |
| Maximum | 0.58 | 22.9 | 33.8 | 26.0 | 4.1 | 77.8 | 119.6 | 0.1 | 23.4 | 90 | 1.1 | 151 | 715 | * |

| Jayanagar-November-2022 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | CO mg/m³ | Ozone µg/m³ | NO2 µg/m³ | NH3 µg/m³ | SO2 µg/m³ | PM2.5 µg/m³ | PM10 µg/m³ | BEN µg/m³ | AT °C | RH % | WS m/s | WD deg. | BP mmHg | AQI |
| 01-11-2022 | 0.50 | 22.5 | 22.9 | 16.2 | 8.1 | 22.8 | 52.8 | 2.27 | 22.0 | 79 | 0.4 | 121 | 710 | 53 |
| 02-11-2022 | 0.73 | 22.7 | 25.0 | 16.6 | 8.5 | 12.0 | 39.2 | 2.48 | 21.3 | 81 | 0.5 | 129 | 709 | 39 |
| 03-11-2022 | 0.90 | 23.0 | 25.1 | 16.2 | 9.1 | 17.7 | 47.1 | 2.49 | 22.7 | 79 | 0.4 | 124 | 710 | 47 |
| 04-11-2022 | 0.54 | 23.1 | 21.6 | 16.2 | 8.5 | 14.3 | 43.9 | 2.14 | 23.1 | 77 | 0.5 | 133 | 711 | 44 |
| 05-11-2022 | 0.66 | 27.1 | 19.9 | 16.5 | 8.1 | 21.3 | 71.2 | 1.99 | 25.6 | 66 | 0.6 | 137 | 712 | 71 |
| 06-11-2022 | 0.59 | 25.4 | 18.6 | 16.6 | 7.5 | 48.1 | 99.3 | 1.85 | 24.0 | 70 | 0.5 | 127 | 712 | 99 |
| 07-11-2022 | 0.65 | 19.1 | 21.1 | 16.3 | 5.8 | 48.7 | 103.6 | 2.10 | 23.5 | 61 | 0.5 | 140 | 711 | 102 |
| 08-11-2022 | 0.63 | 26.6 | 22.3 | 16.5 | 5.4 | 44.0 | 98.7 | 2.21 | 24.2 | 63 | 0.5 | 133 | 711 | 99 |
| 09-11-2022 | 0.70 | 28.6 | 23.6 | 16.2 | 5.4 | 54.9 | 108.3 | 2.34 | 23.7 | 62 | 0.5 | 148 | 711 | 106 |
| 10-11-2022 | 0.65 | 28.5 | 22.5 | 16.4 | 6.1 | 40.6 | 87.9 | 2.23 | 23.9 | 65 | 0.6 | 111 | 710 | 88 |
| 11-11-2022 | 0.67 | 28.7 | 24.2 | 16.0 | 6.1 | 63.1 | 102.5 | 2.40 | 21.1 | 76 | 0.6 | 105 | 710 | 110 |
| 12-11-2022 | 0.80 | 21.3 | 27.5 | 16.3 | 6.3 | 16.0 | 28.1 | 2.73 | 20.6 | 89 | 0.4 | 127 | 710 | 40 |
| 13-11-2022 | 0.44 | 19.0 | 22.4 | 16.4 | 5.7 | 9.2 | 24.4 | 2.04 | 21.5 | 83 | 0.4 | 128 | 712 | 28 |
| 14-11-2022 | 0.54 | 19.0 | 15.6 | 16.7 | 5.6 | 18.3 | 45.7 | 0.28 | 23.0 | 74 | 0.5 | 133 | 712 | 46 |
| 15-11-2022 | 0.71 | 19.2 | 22.6 | 16.3 | 5.8 | 45.2 | 97.4 | 0.40 | 23.5 | 71 | 0.4 | 146 | 711 | 97 |
| 16-11-2022 | 0.74 | 19.1 | 22.6 | 16.5 | 5.2 | 65.8 | 114.5 | 0.41 | 23.0 | 69 | 0.4 | 134 | 710 | 119 |
| 17-11-2022 | 0.65 | 27.0 | 22.9 | 16.3 | 5.7 | 62.5 | 112.9 | 0.37 | 21.9 | 57 | 0.4 | 149 | 710 | 109 |
| 18-11-2022 | 0.65 | 28.4 | 22.8 | 16.4 | 5.4 | 55.0 | 108.0 | 0.36 | 22.3 | 54 | 0.5 | 117 | 710 | 105 |
| 19-11-2022 | 0.66 | 28.5 | 21.0 | 16.6 | 4.7 | 56.0 | 104.0 | 0.37 | 21.5 | 58 | 0.5 | 119 | 709 | 103 |
| 20-11-2022 | 0.60 | 28.8 | 24.4 | 16.3 | 5.4 | 67.6 | 116.8 | 0.34 | 19.9 | 61 | 0.5 | 145 | 708 | 125 |
| 21-11-2022 | 0.54 | 28.2 | 22.1 | 16.0 | 5.7 | 52.0 | 93.2 | 0.30 | 22.7 | 58 | 0.5 | 136 | 708 | 93 |
| 22-11-2022 | 0.72 | 28.1 | 31.3 | 15.1 | 6.0 | 25.1 | 54.1 | 0.40 | 21.9 | 78 | 0.4 | 142 | 708 | 54 |
| 23-11-2022 | 0.62 | 24.4 | 26.8 | 16.1 | 5.1 | 20.8 | 41.3 | 0.34 | 22.3 | 83 | 0.4 | 174 | 709 | 41 |
| 24-11-2022 | 0.52 | 22.9 | 26.0 | 15.9 | 5.2 | 31.3 | 45.9 | 0.29 | 21.0 | 90 | 0.4 | 158 | 710 | 52 |
| 25-11-2022 | 0.48 | 23.0 | 22.8 | 16.2 | 5.8 | 31.5 | 68.0 | 0.27 | 23.1 | 75 | 0.5 | 123 | 711 | 68 |
| 26-11-2022 | 0.54 | 22.9 | 18.4 | 17.1 | 5.7 | 40.5 | 89.2 | 0.30 | 24.1 | 70 | 0.5 | 130 | 709 | 89 |
| 27-11-2022 | 0.40 | 23.0 | 22.0 | 16.4 | 5.3 | 44.3 | 79.8 | 0.22 | 24.0 | 71 | 0.5 | 146 | 708 | 80 |
| 28-11-2022 | 0.66 | 23.3 | 21.9 | 16.4 | 5.6 | 74.2 | 119.8 | 0.37 | 23.7 | 71 | 0.5 | 144 | 709 | 147 |
| 29-11-2022 | 0.63 | 23.3 | 21.7 | 16.0 | 5.5 | 79.8 | 133.1 | 0.35 | 23.1 | 66 | 0.5 | 123 | 709 | 166 |
| 30-11-2022 | 0.50 | 22.9 | 24.4 | 16.2 | 5.3 | 66.4 | 117.5 | 0.28 | 22.6 | 63 | 0.5 | 127 | 710 | 121 |
| Average | 0.62 | 24.2 | 22.9 | 16.3 | 6.1 | 41.6 | 81.6 | 1.16 | 22.7 | 71 | 0.5 | 134 | 710 | * |
| Maximum | 0.90 | 28.8 | 31.3 | 17.1 | 9.1 | 79.8 | 133.1 | 2.73 | 25.6 | 90 | 0.6 | 174 | 712 | * |
| Minimum | 0.40 | 19.0 | 15.6 | 15.1 | 4.7 | 9.2 | 24.4 | 0.22 | 19.9 | 54 | 0.4 | 105 | 708 | * |

| Kavika-November-2022 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | CO mg/m³ | Ozone µg/m³ | NO2 µg/m³ | NH3 µg/m³ | SO2 µg/m³ | PM2.5 µg/m³ | PM10 µg/m³ | BEN µg/m³ | AT °C | RH % | WS m/s | WD deg. | BP mmHg | AQI |
| 01-11-2022 | 1.07 | 5.0 | 13.6 | 39.7 | 5.2 | 44.3 | 79.8 | 0.86 | 23.4 | 44 | 0.5 | 58 | 720 | 80 |
| 02-11-2022 | 1.21 | 6.2 | 11.7 | 45.0 | 5.8 | 36.4 | 73.8 | 1.01 | 22.6 | 43 | 0.5 | 58 | 719 | 74 |
| 03-11-2022 | 1.40 | 4.5 | 11.9 | 48.2 | 7.0 | 47.5 | 90.5 | 1.30 | 23.6 | 43 | 0.6 | 63 | 720 | 91 |
| 04-11-2022 | 1.12 | 5.1 | 11.6 | 41.1 | 5.9 | 40.9 | 80.8 | 0.98 | 24.2 | 43 | 0.6 | 67 | 721 | 81 |
| 05-11-2022 | 0.98 | 4.9 | 12.3 | 37.6 | 6.0 | 47.4 | 82.8 | 0.82 | 24.7 | 43 | 0.6 | 70 | 723 | 83 |
| 06-11-2022 | 1.11 | 3.7 | 14.1 | 36.5 | 6.5 | 75.8 | 118.1 | 0.76 | 24.1 | 44 | 0.6 | 66 | 723 | 153 |
| 07-11-2022 | 1.46 | 4.5 | 21.0 | 54.2 | 5.6 | 92.7 | 148.2 | 0.99 | 24.3 | 44 | 0.6 | 66 | 722 | 209 |
| 08-11-2022 | 1.27 | 4.2 | 17.0 | 48.4 | 5.3 | 83.4 | 130.1 | 1.08 | 25.5 | 43 | 0.6 | 64 | 721 | 178 |
| 09-11-2022 | 1.20 | 5.4 | 20.0 | 51.2 | 5.7 | 93.0 | 140.0 | 1.04 | 25.2 | 43 | 0.6 | 69 | 721 | 210 |
| 10-11-2022 | 1.10 | 4.4 | 17.4 | 43.3 | 5.9 | 71.7 | 119.5 | 0.70 | 24.5 | 44 | 0.5 | 60 | 720 | 139 |
| 11-11-2022 | 1.23 | 4.3 | 19.6 | 42.9 | 5.7 | 100.3 | 140.9 | 0.45 | 22.2 | 44 | 0.5 | 55 | 720 | 234 |
| 12-11-2022 | 1.54 | 5.5 | 13.4 | 51.2 | 6.7 | 47.6 | 62.9 | 0.83 | 21.8 | 44 | 0.5 | 55 | 720 | 79 |
| 13-11-2022 | - | - | - | - | - | - | - | - | - | - | - | - | - | * |
| 14-11-2022 | 0.94 | 8.4 | 9.4 | 45.9 | 5.9 | 50.3 | 74.4 | - | 22.7 | 44 | 0.6 | 68 | 721 | 84 |
| 15-11-2022 | 1.23 | 7.4 | 16.1 | 48.1 | 5.7 | 99.0 | 144.6 | - | 24.5 | 43 | 0.6 | 69 | 721 | 230 |
| 16-11-2022 | 1.27 | 6.9 | 20.0 | 47.5 | 6.4 | 114.0 | 161.8 | - | 24.7 | 43 | 0.6 | 73 | 720 | 280 |
| 17-11-2022 | 1.43 | 6.0 | 24.6 | 52.3 | 6.6 | 102.0 | 154.9 | - | 24.1 | 44 | 0.6 | 69 | 720 | 240 |
| 18-11-2022 | 1.39 | 5.9 | 24.3 | 63.4 | 5.9 | 92.6 | 136.3 | 0.06 | 23.7 | 44 | 0.6 | 66 | 720 | 209 |
| 19-11-2022 | 1.35 | 5.3 | 24.0 | 60.4 | 4.0 | 90.1 | 133.8 | 0.08 | 23.0 | 44 | 0.6 | 65 | 719 | 200 |
| 20-11-2022 | 1.15 | 4.3 | 23.5 | 46.4 | 6.0 | 90.7 | 129.7 | 0.06 | 22.0 | 44 | 0.6 | 70 | 718 | 202 |
| 21-11-2022 | 1.13 | 5.6 | 21.9 | 48.4 | 5.6 | 96.8 | 82.0 | 0.06 | 24.1 | 44 | 0.6 | 78 | 718 | 223 |
| 22-11-2022 | 1.28 | 6.2 | 8.2 | 24.8 | 5.4 | 43.0 | 107.6 | 0.07 | 22.9 | 44 | 0.6 | 69 | 719 | 105 |
| 23-11-2022 | 1.10 | 6.4 | 5.8 | 5.6 | 5.4 | - | - | 0.06 | 23.3 | 44 | 0.7 | 88 | 719 | * |
| 24-11-2022 | 1.28 | 6.0 | 4.4 | 3.4 | 6.6 | 73.7 | 97.7 | 0.07 | 22.3 | 44 | 0.6 | 76 | 720 | 146 |
| 25-11-2022 | 1.05 | 5.9 | 4.4 | 2.9 | 6.4 | 62.1 | 107.7 | 0.06 | 23.9 | 43 | 0.6 | 73 | 721 | 107 |
| 26-11-2022 | 1.06 | 5.6 | 3.2 | 3.4 | 6.3 | 69.9 | 116.3 | 0.06 | 24.9 | 43 | 0.6 | 73 | 719 | 133 |
| 27-11-2022 | 0.82 | 5.0 | 3.0 | 3.1 | 6.1 | 73.5 | 98.3 | 0.05 | 25.1 | 43 | 0.6 | 78 | 718 | 145 |
| 28-11-2022 | 1.16 | 4.5 | 5.5 | 5.0 | 6.3 | 121.5 | 158.3 | 0.06 | 24.9 | 43 | 0.7 | 86 | 719 | 301 |
| 29-11-2022 | 1.30 | 4.7 | 6.7 | 5.2 | 5.9 | 117.4 | 163.4 | 0.07 | 24.3 | 44 | 0.6 | 76 | 720 | 291 |
| 30-11-2022 | 1.25 | 3.9 | 6.9 | 6.0 | 6.0 | 114.0 | 154.2 | 0.07 | 23.5 | 44 | 0.6 | 69 | 720 | 280 |
| Average | 1.20 | 5.4 | 13.6 | 34.9 | 5.9 | 78.3 | 117.4 | 0.47 | 23.8 | 44 | 0.6 | 69 | 720 | * |
| Maximum | 1.54 | 8.4 | 24.6 | 63.4 | 7.0 | 121.5 | 163.4 | 1.30 | 25.5 | 44 | 0.7 | 88 | 723 | * |
| Minimum | 0.82 | 3.7 | 3.0 | 2.9 | 4.0 | 36.4 | 62.9 | 0.05 | 21.8 | 43 | 0.5 | 55 | 718 | * |

| NIMHANS-November-2022 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | CO mg/m³ | Ozone µg/m³ | NO2 µg/m³ | NH3 µg/m³ | SO2 µg/m³ | PM2.5 µg/m³ | PM10 µg/m³ | BEN µg/m³ | AT °C | RH % | WS m/s | WD deg. | BP mmHg | AQI |
| 01-11-2022 | 0.16 | 22.2 | 19.9 | 2.1 | 4.6 | 15 | 33 | 0.29 | 21.4 | 60 | 1.0 | 105 | 709 | 33 |
| 02-11-2022 | 0.40 | 16.1 | 21.4 | 2.0 | 3.8 | 12 | 24 | 0.28 | 20.9 | 61 | 1.1 | 102 | 708 | 27 |
| 03-11-2022 | 0.45 | 13.8 | 23.8 | 2.0 | 4.3 | 15 | 33 | 0.32 | 21.9 | 60 | 0.9 | 115 | 709 | 33 |
| 04-11-2022 | 0.52 | 13.5 | 21.5 | 2.1 | 5.1 | 14 | 30 | 0.29 | 22.4 | 59 | 1.0 | 129 | 710 | 30 |
| 05-11-2022 | 0.46 | 17.3 | 21.0 | 2.0 | 4.9 | 18 | 40 | 0.26 | 23.0 | 55 | 1.1 | 130 | 711 | 40 |
| 06-11-2022 | 0.42 | 27.6 | 20.3 | 2.0 | 4.9 | 31 | 74 | 0.32 | 22.5 | 56 | 0.9 | 120 | 711 | 74 |
| 07-11-2022 | 0.43 | 28.3 | 28.1 | 2.1 | 5.8 | 39 | 93 | 0.40 | 22.7 | 47 | 0.8 | 126 | 711 | 93 |
| 08-11-2022 | 0.43 | 25.6 | 25.1 | 2.1 | 4.5 | 35 | 83 | 0.37 | 23.4 | 48 | 0.9 | 123 | 710 | 83 |
| 09-11-2022 | 0.59 | 26.3 | 27.6 | 2.1 | 5.3 | 41 | 97 | 0.42 | 22.8 | 48 | 0.8 | 126 | 710 | 97 |
| 10-11-2022 | 0.33 | 30.8 | 19.4 | 2.1 | 4.6 | 26 | 61 | 0.28 | 22.8 | 50 | 1.3 | 100 | 709 | 61 |
| 11-11-2022 | 0.47 | 37.7 | 19.7 | 2.1 | 3.9 | 39 | 92 | 0.33 | 20.7 | 57 | 1.8 | 80 | 709 | 92 |
| 12-11-2022 | 0.92 | 16.2 | 21.1 | 2.1 | 4.5 | 12 | 23 | 0.31 | 20.4 | 66 | 1.3 | 96 | 709 | 46 |
| 13-11-2022 | 0.36 | 11.4 | 17.9 | 2.1 | 4.3 | 11 | 23 | 0.30 | 21.4 | 62 | 1.1 | 129 | 710 | 23 |
| 14-11-2022 | 0.40 | 14.1 | 21.0 | 2.1 | 4.4 | 16 | 36 | 0.30 | 22.4 | 56 | 1.0 | 126 | 711 | 36 |
| 15-11-2022 | 0.58 | 22.5 | 26.3 | 2.0 | 5.2 | 38 | 89 | 0.36 | 22.9 | 55 | 0.7 | 130 | 710 | 89 |
| 16-11-2022 | 0.45 | 30.4 | 25.9 | 2.0 | 5.1 | 47 | 112 | 0.36 | 22.5 | 53 | 0.7 | 123 | 709 | 108 |
| 17-11-2022 | 0.55 | 31.2 | 33.7 | 2.0 | 4.6 | 43 | 104 | 0.43 | 21.4 | 43 | 0.7 | 123 | 709 | 103 |
| 18-11-2022 | 0.48 | 30.9 | 33.0 | 2.1 | 4.8 | 37 | 94 | 0.40 | 21.3 | 44 | 0.8 | 122 | 709 | 94 |
| 19-11-2022 | 0.50 | 29.5 | 32.1 | 2.0 | 5.0 | 37 | 92 | 0.40 | 20.7 | 45 | 0.8 | 129 | 709 | 92 |
| 20-11-2022 | 0.39 | 35.4 | 23.2 | 2.1 | 4.5 | 39 | 99 | 0.30 | 19.7 | 46 | 0.9 | 130 | 707 | 99 |
| 21-11-2022 | 0.30 | 37.0 | 20.5 | 2.1 | 4.1 | 31 | 78 | 0.23 | 22.1 | 44 | 1.1 | 121 | 707 | 78 |
| 22-11-2022 | 0.56 | 16.4 | 20.9 | 2.2 | 3.8 | 14 | 34 | 0.24 | 21.4 | 60 | 1.0 | 106 | 708 | 34 |
| 23-11-2022 | 0.50 | 17.0 | 19.5 | 2.1 | 3.8 | 13 | 34 | 0.26 | 22.2 | 61 | 1.1 | 142 | 708 | 34 |
| 24-11-2022 | 0.56 | 13.1 | 23.6 | 2.1 | 4.2 | 19 | 48 | 0.30 | 21.0 | 66 | 0.7 | 141 | 709 | 48 |
| 25-11-2022 | 0.47 | 16.0 | 20.3 | 2.1 | 3.4 | 22 | 54 | 0.26 | 22.4 | 58 | 1.0 | 140 | 710 | 54 |
| 26-11-2022 | 0.55 | 16.9 | 22.3 | 2.0 | 3.5 | 26 | 65 | 0.27 | 22.9 | 55 | 0.8 | 137 | 708 | 65 |
| 27-11-2022 | 0.42 | 22.0 | 18.5 | 2.1 | 4.0 | 27 | 66 | 0.24 | 23.4 | 55 | 0.6 | 139 | 707 | 66 |
| 28-11-2022 | 0.64 | 31.4 | 23.3 | 2.1 | 4.0 | 46 | 115 | 0.33 | 23.4 | 54 | 0.7 | 148 | 708 | 110 |
| 29-11-2022 | 0.44 | 36.8 | 22.5 | 2.1 | 4.4 | 48 | 119 | 0.30 | 22.4 | 51 | 0.9 | 126 | 709 | 113 |
| 30-11-2022 | 0.67 | 33.2 | 23.9 | 2.1 | 4.1 | 43 | 106 | 0.32 | 21.6 | 50 | 0.8 | 121 | 709 | 104 |
| Average | 0.48 | 24.0 | 23.2 | 2.1 | 4.5 | 28 | 68 | 0.32 | 22.0 | 54 | 0.9 | 123 | 709 | * |
| Maximum | 0.92 | 37.7 | 33.7 | 2.2 | 5.8 | 48 | 119 | 0.43 | 23.4 | 66 | 1.8 | 148 | 711 | * |
| Minimum | 0.16 | 11.4 | 17.9 | 2.0 | 3.4 | 11 | 23 | 0.23 | 19.7 | 43 | 0.6 | 80 | 707 | * |

| Silkboard-November-2022 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | CO mg/m³ | Ozone µg/m³ | NO2 µg/m³ | NH3 µg/m³ | SO2 µg/m³ | PM2.5 µg/m³ | PM10 µg/m³ | BEN µg/m³ | AT °C | RH % | WS m/s | WD deg. | BP mmHg | AQI |
| 01-11-2022 | 0.56 | 26.4 | 23.8 | 12.2 | 5.9 | 21.0 | 51.8 | 0.14 | 21.1 | 79 | 1.0 | 119 | 689 | 52 |
| 02-11-2022 | 0.59 | 22.0 | 21.0 | 15.2 | 5.8 | 13.3 | 63.3 | 0.15 | 20.7 | 81 | 1.1 | 118 | 689 | 63 |
| 03-11-2022 | 0.48 | 22.7 | 23.8 | 11.2 | 5.6 | 15.1 | 60.2 | 0.12 | 21.3 | 81 | 1.1 | 109 | 689 | 60 |
| 04-11-2022 | 0.47 | 23.0 | 24.9 | 10.5 | 5.6 | 12.6 | 45.0 | 0.12 | 22.3 | 78 | 1.2 | 92 | 690 | 45 |
| 05-11-2022 | 0.47 | 25.8 | 26.0 | 9.3 | 5.6 | 18.6 | 44.8 | 0.12 | 22.8 | 73 | 1.2 | 92 | 691 | 45 |
| 06-11-2022 | 0.54 | 31.3 | 26.0 | 9.8 | 5.7 | 44.5 | 106.9 | 0.14 | 22.6 | 73 | 1.1 | 105 | 691 | 105 |
| 07-11-2022 | 0.95 | 30.0 | 23.6 | 13.4 | 5.7 | 53.8 | 129.1 | 0.24 | 22.7 | 62 | 0.9 | 113 | 690 | 119 |
| 08-11-2022 | 0.84 | 28.3 | 22.4 | 16.8 | 5.7 | 47.3 | 113.3 | 0.21 | 23.4 | 64 | 1.0 | 105 | 690 | 109 |
| 09-11-2022 | 1.12 | 28.4 | 25.6 | 13.0 | 5.7 | 58.0 | 139.3 | 0.27 | 22.7 | 64 | 0.9 | 105 | 690 | 126 |
| 10-11-2022 | 0.52 | 31.5 | 24.0 | 12.9 | 5.6 | 37.7 | 90.4 | 0.13 | 22.7 | 67 | 1.3 | 107 | 689 | 90 |
| 11-11-2022 | 0.70 | 34.8 | 22.5 | 19.7 | 5.7 | 58.5 | 140.5 | 0.18 | 20.6 | 76 | 1.6 | 133 | 689 | 127 |
| 12-11-2022 | 0.64 | 22.5 | 16.9 | 22.5 | 5.6 | 11.8 | 29.0 | 0.16 | 20.1 | 88 | 1.1 | 121 | 689 | 32 |
| 13-11-2022 | 0.47 | 21.3 | 19.5 | 17.1 | 5.5 | 8.9 | 21.5 | 0.12 | 21.1 | 83 | 1.0 | 89 | 690 | 24 |
| 14-11-2022 | 0.52 | 25.6 | 19.1 | 18.6 | 5.7 | 15.5 | 37.2 | 0.13 | 22.3 | 73 | 1.1 | 96 | 690 | 37 |
| 15-11-2022 | 0.74 | 31.4 | 19.8 | 20.8 | 5.7 | 47.6 | 114.2 | 0.18 | 22.9 | 71 | 0.9 | 100 | 690 | 109 |
| 16-11-2022 | 0.92 | 28.8 | 16.4 | 28.1 | 5.7 | 65.4 | 157.0 | 0.23 | 22.7 | 68 | 0.8 | 143 | 689 | 138 |
| 17-11-2022 | 1.16 | 27.6 | 16.6 | 42.0 | 5.8 | 66.0 | 158.6 | 0.29 | 20.9 | 59 | 0.9 | 162 | 689 | 139 |
| 18-11-2022 | 1.08 | 32.1 | 23.9 | 40.0 | 5.7 | 60.8 | 145.9 | 0.27 | 21.2 | 56 | 0.8 | 123 | 689 | 131 |
| 19-11-2022 | 0.90 | 31.7 | 15.7 | 33.6 | 5.8 | 57.9 | 138.8 | 0.23 | 20.7 | 59 | 0.9 | 110 | 689 | 126 |
| 20-11-2022 | 0.91 | 30.0 | 15.5 | 35.4 | 5.7 | 64.6 | 155.0 | 0.23 | 19.5 | 61 | 0.9 | 177 | 688 | 137 |
| 21-11-2022 | 0.76 | 33.5 | 18.6 | 29.1 | 5.9 | 49.8 | 119.5 | 0.19 | 22.2 | 58 | 1.1 | 149 | 688 | 113 |
| 22-11-2022 | 0.82 | 23.1 | 15.3 | 25.1 | 5.7 | 25.7 | 61.7 | 0.20 | 21.3 | 79 | 0.9 | 152 | 688 | 62 |
| 23-11-2022 | 1.05 | 21.9 | 13.4 | 31.3 | 5.7 | 24.2 | 58.0 | 0.26 | 21.9 | 82 | 1.0 | 195 | 688 | 58 |
| 24-11-2022 | 0.71 | 24.6 | 22.1 | 16.5 | 5.6 | 28.7 | 68.8 | 0.18 | 20.8 | 87 | 0.9 | 95 | 689 | 69 |
| 25-11-2022 | 0.65 | 26.9 | 21.6 | 18.8 | 5.7 | 30.5 | 73.3 | 0.17 | 22.3 | 76 | 1.0 | 93 | 689 | 73 |
| 26-11-2022 | 0.56 | 28.3 | 24.2 | 13.5 | 5.7 | 34.9 | 83.7 | 0.14 | 23.1 | 71 | 1.0 | 94 | 688 | 84 |
| 27-11-2022 | 0.45 | 33.3 | 27.2 | 10.1 | 5.7 | 45.9 | 110.3 | 0.11 | 23.3 | 72 | 0.9 | 98 | 688 | 107 |
| 28-11-2022 | 0.79 | 38.8 | 25.1 | 15.4 | 5.9 | 72.7 | 174.5 | 0.20 | 23.6 | 70 | 0.9 | 108 | 688 | 150 |
| 29-11-2022 | 0.81 | 38.8 | 25.0 | 17.6 | 5.7 | 79.3 | 190.2 | 0.20 | 22.4 | 67 | 1.0 | 115 | 689 | 164 |
| 30-11-2022 | 0.82 | 37.8 | 22.7 | 19.3 | 5.7 | 67.5 | 161.9 | 0.21 | 21.7 | 64 | 1.0 | 107 | 689 | 141 |
| Average | 0.73 | 28.7 | 21.4 | 20.0 | 5.7 | 41.3 | 101.5 | 0.18 | 21.9 | 71 | 1.0 | 118 | 689 | * |
| Maximum | 1.16 | 38.8 | 27.2 | 42.0 | 5.9 | 79.3 | 190.2 | 0.29 | 23.6 | 88 | 1.6 | 195 | 691 | * |
| Minimum | 0.45 | 21.3 | 13.4 | 9.3 | 5.5 | 8.9 | 21.5 | 0.11 | 19.5 | 56 | 0.8 | 89 | 688 | * |

e

u

l

a

V

I

Q

A

AQI Trend in CAAQMs in Bengaluru-November-2022

400

350

City Railway Station, Majestic

300

Nisarga Bhavan, Saneguruvanahalli

250

200

Veterinary College, Hebbal

150

Shalini Ground, Jayanagar

100

KAVIKA, Mysore Road

50

NIMHANS

0

H.S.R Layout, Near Central Silkboard

INFERENCE:

The Air Quality monitored through the 7 CAAQMs (Continuous Ambient Air  Quality Monitoring Stations) installed in Bengaluru viz., City  Railway Station, Hebbal, Saneguruvanahalli, NIMHANS, Jayanagar, KAVIKA-Mysore  Road and Central Silk Board-HSR Layout, for the month of November-2022 reveals  that the Air quality is in Good, Satisfactory and moderate in almost all days over the  number of days monitored. Further, the AQI in few stations have shown Moderate  which may be due to high vehicular movement at that particular period of time. The  AQI in KAVIKA, Mysore road reached from moderate to Poor which may be due to  heavy traffic movements and waste burning observed on road side.

However, the air quality is going to improve on the changed weather conditions  and effective traffic management over the days to come.

-------*****------