Air Quality Index

Air Quality Index is a tool for effective communication of air quality status to  people in terms, which are easy to understand. It transforms complex air quality data of  various pollutants into a single number (index value), nomenclature and colour.

There are six AQI categories, namely Good, Satisfactory, Moderate, Poor,  Very Poor, and Severe. Each of these categories is decided based on ambient  concentration values of air pollutants and their likely health impacts (known as health  breakpoints). AQ sub-index and health breakpoints are evolved for eight pollutants (PM10,  PM2.5, NO2, SO2, CO, O3, NH3, and Pb) for which short-term (upto 24-hours)

National Ambient Air Quality Standards are prescribed.

Based on the measured ambient concentrations of a pollutant, sub-index is  calculated, which is a linear function of concentration (e.g. the sub-index for PM2.5 will  be 51 at concentration 31 µg/m3, 100 at concentration 60 µg/m3, and 75 at concentration  of 45(µg/m3). The worst sub-index determines the overall AQI. AQI categories and  health breakpoints for the eight pollutants are as follow:

The Karnataka State Pollution Control Board has installed 7 Continuous  Ambient Air Quality Monitoring Stations (CAAQMs) covering Residential,  Commercial, Industrial and Silence Zones. The Air Quality is being monitored round  the clock, 24 hours a day for the Primary and secondary pollutants as well as  Meteorological parameters. The Air Quality monitored during the month of October

2022 is as shown below:

| Daily AQI Values of Bengaluru (October-2022) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date/ CAAQM Stations | City Railway Station, Majestic | Nisarga Bhavan, Saneguruvanahalli | Veterinary College, Hebbal | Shalini Ground, Jayanagar | KAVIKA, Mysore Road | NIMHANS | H.S.R Layout, Near Central Silkboard |
| 01-10-2022 | 67 | 49 | 36 | 28 | 58 | 29 | * |
| 02-10-2022 | 72 | 49 | 47 | 62 | 74 | 47 | * |
| 03-10-2022 | 79 | 48 | 55 | 66 | 132 | 64 | * |
| 04-10-2022 | 78 | 45 | 60 | 74 | 91 | 64 | 115 |
| 05-10-2022 | 58 | 45 | 56 | 60 | 85 | 60 | 111 |
| 06-10-2022 | 76 | 45 | * | 44 | 301 | 46 | 114 |
| 07-10-2022 | 78 | 46 | 47 | 41 | 98 | 44 | 107 |
| 08-10-2022 | 68 | 46 | 52 | 66 | 110 | 53 | 108 |
| 09-10-2022 | 69 | 44 | 54 | 45 | 93 | 48 | 71 |
| 10-10-2022 | 77 | 44 | 47 | 34 | 83 | 39 | 55 |
| 11-10-2022 | 68 | 43 | 59 | 45 | 78 | 47 | 62 |
| 12-10-2022 | 68 | 42 | 54 | 42 | 62 | 42 | 85 |
| 13-10-2022 | 71 | 41 | 52 | 43 | 75 | 47 | 57 |
| 14-10-2022 | 74 | 42 | 49 | 43 | 81 | 46 | 69 |
| 15-10-2022 | 73 | 41 | 47 | 28 | 52 | 28 | 42 |
| 16-10-2022 | 89 | 41 | 47 | 30 | 75 | 36 | 43 |
| 17-10-2022 | 92 | 28 | 46 | 37 | 77 | 33 | 45 |
| 18-10-2022 | 95 | 31 | 46 | 32 | 78 | 32 | 70 |
| 19-10-2022 | 105 | 65 | 43 | 48 | 79 | 34 | 72 |
| 20-10-2022 | 102 | 55 | 56 | 49 | 90 | 47 | 70 |
| 21-10-2022 | 93 | 31 | 60 | 59 | 104 | 50 | 96 |
| 22-10-2022 | 113 | 37 | 76 | 102 | 176 | 79 | 129 |
| 23-10-2022 | 55 | 41 | 82 | 89 | 147 | 82 | 154 |
| 24-10-2022 | 97 | 72 | 154 | 254 | 312 | 83 | 320 |
| 25-10-2022 | 109 | 98 | 210 | 312 | 349 | 109 | 301 |
| 26-10-2022 | 109 | 105 | 105 | 204 | 316 | 110 | 168 |
| 27-10-2022 | 93 | 69 | 83 | 84 | 147 | 78 | 129 |
| 28-10-2022 | 84 | 63 | 66 | 75 | 111 | 55 | 92 |
| 29-10-2022 | 71 | 58 | 75 | 69 | 121 | 61 | 66 |
| 30-10-2022 | 80 | 52 | 85 | 79 | 174 | 73 | 96 |
| 31-10-2022 | 90 | 48 | 108 | 111 | 216 | 101 | 131 |
| Min | 55 | 28 | 36 | 28 | 52 | 28 | 42 |
| Max | 113 | 105 | 210 | 312 | 349 | 110 | 320 |

| Good | Satisfactory | Moderate | Poor | Very Poor | Severe |
| --- | --- | --- | --- | --- | --- |
| (0-50) | (51-100) | (101-200) | (201-300) | (301-400) | (>401) |

Possible Health Impacts

| Good | Minimal Impact |
| --- | --- |
| Satisfactory | Minor breathing discomfort to sensitive people |
| Moderate | Breathing discomfort to the people with lungs, asthma and heart diseases |
| Poor | Breathing discomfort to most people on prolonged exposure |
| Very Poor | Respiratory illness on prolonged exposure |
| Severe | Affects healthy people and seriously impacts those with existing diseases |

| HEBBAL-Oct-2022 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | CO (mg/m³) | Ozone (µg/m³) | NO2 (µg/m³) | NH3 (µg/m³) | SO2 (µg/m³) | PM2.5 (µg/m³) | PM10 (µg/m³) | BEN. (µg/m ³) | AT (°C) | RH (%) | WS (m/s) | WD (deg) | BP (mmHg) | AQI |
| 01-10-2022 | 0.42 | 36.4 | 3.7 | 2.8 | 2.9 | 19.8 | 35.8 | 0.1 | 20.0 | 89 | 1.2 | 166 | 709 | 36 |
| 02-10-2022 | 0.54 | 49.4 | 9.6 | 5.2 | 2.6 | 18.3 | 47.0 | 0.1 | 20.2 | 87 | 1.1 | 162 | 708 | 47 |
| 03-10-2022 | 0.37 | 62.5 | 16.0 | 14.4 | 1.9 | 29.1 | 55.3 | 0.1 | 21.3 | 83 | 1.0 | 141 | 707 | 55 |
| 04-10-2022 | 0.34 | 97.6 | 5.7 | 6.7 | 1.2 | 32.6 | 59.6 | 0.1 | 22.5 | 78 | 1.1 | 150 | 708 | 60 |
| 05-10-2022 | 0.33 | 48.0 | 3.8 | 2.8 | 2.6 | 28.4 | 55.9 | 0.1 | 22.8 | 78 | 1.2 | 166 | 707 | 56 |
| 06-10-2022 | - | - | - | - | - | - | - | - | - | - | - | - | - | * |
| 07-10-2022 | 0.31 | 37.0 | 10.5 | 7.7 | 3.5 | 19.7 | 46.8 | 0.1 | 21.5 | 78 | 1.0 | 127 | 707 | 47 |
| 08-10-2022 | 0.33 | 22.4 | 7.1 | 4.3 | 3.2 | 25.3 | 51.9 | 0.1 | 22.9 | 74 | 0.9 | 117 | 707 | 52 |
| 09-10-2022 | 0.32 | 26.5 | 7.6 | 5.9 | 3.1 | 26.5 | 54.1 | 0.1 | 22.5 | 81 | 0.9 | 123 | 708 | 54 |
| 10-10-2022 | 0.37 | 14.3 | 21.7 | 13.2 | 2.8 | 23.4 | 46.8 | 0.1 | 22.9 | 81 | 0.9 | 118 | 708 | 47 |
| 11-10-2022 | 0.43 | 12.0 | 26.3 | 11.8 | 2.5 | 28.4 | 59.3 | 0.1 | 23.1 | 80 | 1.0 | 129 | 709 | 59 |
| 12-10-2022 | 0.42 | 26.2 | 21.8 | 10.4 | 2.7 | 24.8 | 54.1 | 0.1 | 23.0 | 79 | 1.0 | 124 | 709 | 54 |
| 13-10-2022 | 0.43 | 34.2 | 27.9 | 10.6 | 2.4 | 25.8 | 52.2 | 0.1 | 23.1 | 82 | 1.0 | 125 | 710 | 52 |
| 14-10-2022 | 0.44 | 33.6 | 22.3 | 10.4 | 2.7 | 28.4 | 48.8 | 0.1 | 22.0 | 86 | 0.9 | 123 | 709 | 49 |
| 15-10-2022 | 0.45 | 15.0 | 19.1 | 8.6 | 2.5 | 27.0 | 46.9 | 0.1 | 22.9 | 85 | 1.0 | 132 | 707 | 47 |
| 16-10-2022 | 0.44 | 32.2 | 3.7 | 2.8 | 2.2 | 28.2 | 47.4 | 0.1 | 22.5 | 84 | 0.9 | 116 | 711 | 47 |
| 17-10-2022 | 0.48 | 9.5 | 10.1 | 3.5 | 2.4 | 21.4 | 46.2 | 0.1 | 22.6 | 82 | 0.9 | 114 | 713 | 46 |
| 18-10-2022 | 0.49 | 41.7 | 10.9 | 4.8 | 2.5 | 23.5 | 46.2 | 0.1 | 21.6 | 85 | 0.9 | 123 | 711 | 46 |
| 19-10-2022 | 0.44 | 38.8 | 3.7 | 2.8 | 1.6 | 21.2 | 42.5 | 0.1 | 22.9 | 80 | 0.9 | 120 | 718 | 43 |
| 20-10-2022 | 0.47 | 54.3 | 19.7 | 7.7 | 2.1 | 29.4 | 55.5 | 0.1 | 23.1 | 80 | 0.9 | 104 | 716 | 56 |
| 21-10-2022 | 0.43 | 29.4 | 30.9 | 10.0 | 2.5 | 26.5 | 60.4 | 0.1 | 22.8 | 76 | 0.8 | 93 | 713 | 60 |
| 22-10-2022 | 0.45 | - | 26.0 | 11.8 | 2.2 | 43.9 | 76.0 | 0.1 | 23.6 | 69 | 1.0 | 127 | 714 | 76 |
| 23-10-2022 | 0.33 | - | 7.9 | 6.8 | 3.3 | 45.6 | 81.7 | 0.1 | 21.6 | 62 | 1.0 | 143 | 710 | 82 |
| 24-10-2022 | 0.34 | 63.7 | 3.8 | 2.8 | 3.3 | 76.1 | 119.8 | 0.1 | 20.3 | 56 | 1.0 | 134 | 710 | 154 |
| 25-10-2022 | 0.39 | 42.7 | 5.9 | 4.0 | 3.0 | 93.1 | 141.0 | 0.1 | 20.6 | 61 | 0.9 | 117 | 710 | 210 |
| 26-10-2022 | 0.48 | 35.6 | 11.4 | 5.4 | 2.8 | 58.4 | 107.8 | 0.1 | 21.9 | 58 | 0.8 | 100 | 711 | 105 |
| 27-10-2022 | 0.50 | 11.6 | 8.1 | 4.3 | 3.1 | 41.7 | 82.8 | 0.1 | 22.4 | 63 | 0.8 | 96 | 710 | 83 |
| 28-10-2022 | 0.38 | 11.7 | 7.3 | 12.1 | 3.0 | 37.7 | 65.5 | 0.1 | 21.8 | 73 | 0.9 | 116 | 710 | 66 |
| 29-10-2022 | 0.40 | 12.9 | 26.1 | 28.5 | 3.7 | 45.2 | 74.3 | 0.1 | 21.8 | 77 | 0.9 | 109 | 712 | 75 |
| 30-10-2022 | 0.37 | 13.1 | 22.8 | 21.3 | 3.1 | 50.7 | 79.9 | 0.1 | 22.3 | 76 | 0.8 | 98 | 713 | 85 |
| 31-10-2022 | 0.42 | 33.4 | 31.5 | 15.8 | 3.2 | 60.8 | 112.4 | 0.1 | 22.5 | 64 | 0.9 | 106 | 711 | 108 |
| Average | 0.41 | 33.8 | 14.4 | 8.6 | 2.7 | 35.4 | 65.1 | 0.1 | 22.2 | 76 | 1.0 | 124 | 710 | * |
| Minimum | 0.31 | 9.5 | 3.7 | 2.8 | 1.2 | 18.3 | 35.8 | 0.1 | 20.0 | 56 | 0.8 | 93 | 707 | * |
| Maximum | 0.54 | 97.6 | 31.5 | 28.5 | 3.7 | 93.1 | 141.0 | 0.1 | 23.6 | 89 | 1.2 | 166 | 718 | * |

| Jayanagar-Oct-2022 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | CO mg/m³ | Ozone µg/m³ | NO2 µg/m³ | NH3 µg/m³ | SO2 µg/m³ | PM2.5 µg/m³ | PM10 µg/m³ | BEN µg/m³ | AT °C | RH % | WS m/s | WD deg. | BP mmHg | AQI |
| 01-10-2022 | 0.25 | 18.5 | 15.0 | 2.9 | 6.0 | 15.5 | 28.1 | 0.14 | 20.1 | 83 | 1.1 | 235 | 708 | 28 |
| 02-10-2022 | 0.40 | 18.2 | 19.5 | 3.2 | 5.7 | 35.8 | 61.9 | 0.19 | 22.7 | 73 | 1.0 | 225 | 707 | 62 |
| 03-10-2022 | 0.67 | 19.5 | 25.6 | 3.5 | 6.1 | 39.3 | 63.3 | 0.30 | 21.4 | 79 | 0.7 | 246 | 707 | 66 |
| 04-10-2022 | 0.39 | 19.5 | 20.3 | 3.1 | 5.4 | 43.9 | 74.0 | 0.32 | 22.8 | 72 | 0.9 | 231 | 706 | 74 |
| 05-10-2022 | 0.31 | 19.5 | 16.4 | 3.0 | 5.7 | 35.8 | 57.0 | 0.34 | 22.5 | 74 | 1.0 | 228 | 706 | 60 |
| 06-10-2022 | 0.19 | 19.2 | 14.4 | 3.1 | 5.9 | 26.1 | 43.4 | 0.28 | 20.8 | 78 | 1.1 | 222 | 706 | 44 |
| 07-10-2022 | 0.29 | 19.0 | 16.1 | 3.1 | 6.3 | 22.3 | 41.2 | 0.28 | 22.4 | 70 | 1.1 | 216 | 707 | 41 |
| 08-10-2022 | 0.56 | 18.9 | 27.3 | 2.6 | 5.9 | 27.7 | 65.5 | 0.25 | 23.0 | 68 | 0.5 | 189 | 707 | 66 |
| 09-10-2022 | 0.40 | 19.5 | 27.9 | 2.9 | 5.9 | 26.7 | 38.5 | 0.37 | 21.8 | 84 | 0.5 | 186 | 708 | 45 |
| 10-10-2022 | 0.41 | 19.5 | 22.1 | 3.1 | 6.1 | 19.3 | 34.0 | 0.31 | 23.7 | 81 | 0.5 | 198 | 709 | 34 |
| 11-10-2022 | 0.38 | 19.5 | 20.9 | 3.0 | 6.0 | 20.0 | 45.0 | 0.34 | 23.9 | 79 | 0.6 | 188 | 708 | 45 |
| 12-10-2022 | 0.38 | 19.4 | 18.8 | 3.1 | 6.1 | 14.4 | 41.6 | 0.31 | 23.6 | 77 | 0.8 | 190 | 709 | 42 |
| 13-10-2022 | 0.41 | 19.5 | 22.0 | 3.0 | 6.3 | 21.0 | 42.8 | 0.31 | 24.4 | 77 | 0.5 | 192 | 708 | 43 |
| 14-10-2022 | 0.43 | 19.2 | 21.4 | 3.5 | 5.4 | 25.8 | 36.8 | 0.34 | 22.4 | 85 | 0.6 | 205 | 708 | 43 |
| 15-10-2022 | 0.41 | 19.5 | 22.3 | 2.8 | 5.2 | 11.2 | 26.4 | 0.32 | 23.9 | 81 | 0.6 | 186 | 707 | 28 |
| 16-10-2022 | 0.38 | 19.7 | 21.3 | 3.0 | 6.3 | 17.8 | 27.0 | 0.31 | 23.1 | 82 | 0.4 | 175 | 709 | 30 |
| 17-10-2022 | 0.39 | 19.2 | 23.7 | 2.7 | 6.2 | 12.8 | 37.1 | 0.32 | 24.2 | 77 | 0.5 | 171 | 710 | 37 |
| 18-10-2022 | 0.60 | 19.4 | 21.8 | 3.0 | 5.9 | 17.5 | 31.8 | 0.27 | 22.4 | 83 | 0.7 | 225 | 710 | 32 |
| 19-10-2022 | 0.50 | 19.4 | 21.7 | 3.0 | 5.4 | 16.2 | 47.6 | 0.27 | 24.2 | 75 | 0.5 | 188 | 709 | 48 |
| 20-10-2022 | 0.58 | 19.4 | 24.4 | 2.8 | 5.0 | 21.8 | 48.5 | 0.38 | 24.1 | 76 | 0.5 | 150 | 710 | 49 |
| 21-10-2022 | 0.62 | 19.4 | 28.6 | 4.2 | 5.1 | 23.7 | 59.2 | 0.32 | 23.4 | 76 | 0.4 | 135 | 711 | 59 |
| 22-10-2022 | 0.74 | 19.3 | 29.8 | 6.1 | 5.3 | 49.3 | 103.2 | 0.35 | 23.9 | 68 | 0.4 | 143 | 711 | 102 |
| 23-10-2022 | 0.45 | 26.7 | 22.8 | 6.3 | 6.2 | 49.8 | 88.5 | 0.34 | 23.3 | 55 | 0.6 | 196 | 710 | 89 |
| 24-10-2022 | 0.69 | 25.2 | 32.5 | 11.0 | 8.8 | 106.3 | 148.2 | 0.32 | 21.7 | 50 | 0.8 | 237 | 709 | 254 |
| 25-10-2022 | 1.19 | 19.2 | 43.3 | 16.7 | 8.1 | 136.0 | 183.3 | 1.53 | 21.8 | 58 | 0.5 | 225 | 710 | 312 |
| 26-10-2022 | 1.11 | 19.2 | 30.6 | 16.6 | 7.5 | 91.3 | 141.9 | 1.89 | 23.3 | 53 | 0.4 | 142 | 711 | 204 |
| 27-10-2022 | 1.09 | 27.2 | 32.5 | 16.2 | 7.2 | 37.1 | 84.0 | 2.15 | 23.6 | 59 | 0.5 | 140 | 710 | 84 |
| 28-10-2022 | 1.30 | 19.4 | 23.9 | 16.5 | 7.5 | 33.3 | 74.8 | 2.30 | 23.3 | 69 | 0.5 | 119 | 709 | 75 |
| 29-10-2022 | 0.79 | 27.2 | 19.4 | 16.7 | 8.3 | 35.1 | 68.5 | 1.93 | 22.8 | 75 | 0.5 | 125 | 709 | 69 |
| 30-10-2022 | 0.57 | 30.5 | 21.7 | 16.3 | 7.4 | 47.3 | 76.9 | 2.15 | 23.7 | 73 | 0.4 | 113 | 710 | 79 |
| 31-10-2022 | 0.65 | 28.5 | 21.0 | 16.6 | 8.1 | 60.5 | 115.8 | 2.08 | 24.1 | 59 | 0.5 | 114 | 709 | 111 |
| Average | 0.57 | 20.9 | 23.5 | 6.6 | 6.3 | 36.8 | 65.7 | 0.69 | 23.0 | 73 | 0.6 | 185 | 709 | * |
| Maximum | 1.30 | 30.5 | 43.3 | 16.7 | 8.8 | 136.0 | 183.3 | 2.30 | 24.4 | 85 | 1.1 | 246 | 711 | * |
| Minimum | 0.19 | 18.2 | 14.4 | 2.6 | 5.0 | 11.2 | 26.4 | 0.14 | 20.1 | 50 | 0.4 | 113 | 706 | * |

| KAVIKA-Oct-2022 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | CO mg/m³ | Ozone µg/m³ | NO2 µg/m³ | NH3 µg/m³ | SO2 µg/m³ | PM2.5 µg/m³ | PM10 µg/m³ | BEN µg/m³ | AT °C | RH % | WS m/s | WD deg. | BP mmHg | AQI |
| 01-10-2022 | 0.69 | 16.5 | 18.5 | 33.1 | 5.5 | 34.5 | 51.7 | 0.74 | 22.3 | 81 | 1.2 | 256 | 717 | 58 |
| 02-10-2022 | 0.76 | 22.1 | 15.5 | 27.0 | 6.0 | 44.3 | 73.1 | 0.80 | 23.9 | 75 | 1.2 | 262 | 717 | 74 |
| 03-10-2022 | 1.33 | 14.5 | 17.4 | 47.4 | 5.2 | 69.6 | 108.0 | 1.84 | 23.7 | 77 | 1.0 | 252 | 716 | 132 |
| 04-10-2022 | 0.98 | 22.8 | 18.2 | 32.2 | 4.8 | 54.8 | 88.0 | 1.08 | 25.2 | 70 | 1.1 | 247 | 716 | 91 |
| 05-10-2022 | 0.77 | 27.3 | 14.3 | 26.9 | 4.9 | 50.7 | 69.8 | 0.75 | 24.7 | 72 | 1.1 | 255 | 715 | 85 |
| 06-10-2022 | 0.82 | 19.0 | 14.7 | 28.0 | 5.5 | 123.0 | 154.2 | 0.60 | 23.0 | 77 | 1.1 | 246 | 715 | 301 |
| 07-10-2022 | 0.71 | 18.1 | 11.4 | 27.2 | 5.4 | 59.0 | 87.5 | 0.69 | 24.7 | 68 | 1.2 | 250 | 717 | 98 |
| 08-10-2022 | 1.12 | 13.3 | 13.2 | 38.8 | 5.3 | 63.0 | 105.0 | 1.24 | 25.7 | 65 | 0.9 | 190 | 717 | 110 |
| 09-10-2022 | 0.95 | 11.5 | 10.7 | 34.4 | 4.7 | 55.8 | 79.6 | 0.95 | 24.1 | 78 | 0.8 | 189 | 718 | 93 |
| 10-10-2022 | 0.89 | 11.0 | 12.1 | 31.5 | 5.2 | 50.0 | 67.6 | 0.85 | 24.9 | 76 | 0.7 | 194 | 718 | 83 |
| 11-10-2022 | 0.73 | 16.6 | 11.3 | 25.3 | 5.5 | 46.6 | 66.5 | 0.69 | 25.2 | 75 | 0.8 | 179 | 718 | 78 |
| 12-10-2022 | 0.79 | 17.0 | 12.6 | 26.5 | 5.6 | 37.1 | 61.1 | 0.75 | 25.1 | 72 | 0.9 | 192 | 719 | 62 |
| 13-10-2022 | 0.85 | 19.8 | 15.3 | 29.8 | 5.3 | 45.1 | 60.6 | 0.85 | 25.2 | 75 | 0.8 | 181 | 718 | 75 |
| 14-10-2022 | 1.09 | 18.6 | 17.7 | 35.8 | 5.7 | 48.8 | 60.6 | 1.13 | 23.5 | 83 | 0.8 | 205 | 718 | 81 |
| 15-10-2022 | 1.04 | 13.7 | 17.0 | 30.3 | 5.9 | 30.1 | 52.0 | 1.00 | 24.6 | 79 | 0.8 | 197 | 717 | 52 |
| 16-10-2022 | 0.88 | 15.7 | 14.9 | 30.0 | 5.9 | 44.8 | 57.3 | 0.69 | 24.4 | 79 | 0.8 | 173 | 720 | 75 |
| 17-10-2022 | 0.95 | 11.2 | 12.1 | 32.6 | 5.5 | 45.2 | 77.2 | 0.94 | 25.3 | 72 | 0.8 | 151 | 721 | 77 |
| 18-10-2022 | 1.07 | 16.0 | 15.6 | 33.7 | 5.4 | 46.6 | 72.0 | 1.40 | 23.9 | 79 | 0.8 | 218 | 721 | 78 |
| 19-10-2022 | 1.03 | 17.7 | 21.9 | 38.2 | 4.8 | 47.5 | 75.8 | 0.94 | 25.0 | 74 | 0.8 | 190 | 720 | 79 |
| 20-10-2022 | 1.16 | 14.9 | 20.8 | 39.1 | 5.4 | 53.7 | 89.0 | 0.94 | 25.2 | 73 | 0.9 | 157 | 721 | 90 |
| 21-10-2022 | 1.29 | 11.0 | 20.9 | 48.2 | 5.5 | 58.5 | 105.3 | 1.10 | 25.2 | 69 | 0.7 | 103 | 722 | 104 |
| 22-10-2022 | 1.52 | 12.3 | 28.3 | 60.0 | 5.3 | 82.9 | 137.3 | 1.38 | 25.5 | 63 | 0.3 | 23 | 722 | 176 |
| 23-10-2022 | 0.96 | 15.9 | 22.2 | 37.2 | 5.6 | 74.2 | 111.5 | 0.80 | 25.0 | 51 | 0.3 | 17 | 720 | 147 |
| 24-10-2022 | 0.92 | 9.0 | 23.9 | 38.2 | 6.3 | 135.7 | 171.2 | 0.77 | 24.0 | 44 | 0.3 | 14 | 719 | 312 |
| 25-10-2022 | 1.38 | 14.6 | 22.3 | 49.8 | 6.4 | 183.8 | 228.2 | 0.79 | 24.4 | 49 | 0.3 | 12 | 720 | 349 |
| 26-10-2022 | 1.21 | 6.2 | 28.3 | 52.5 | 6.0 | 141.3 | 196.5 | 0.92 | 25.5 | 46 | - | - | 721 | 316 |
| 27-10-2022 | 1.15 | 4.5 | 21.9 | 58.1 | 5.7 | 74.1 | 114.5 | 1.06 | 25.2 | 43 | 0.4 | 78 | 718 | 147 |
| 28-10-2022 | 1.13 | 4.0 | 18.7 | 48.5 | 5.1 | 63.4 | 109.3 | 0.93 | 24.4 | 44 | 0.5 | 61 | 719 | 111 |
| 29-10-2022 | 1.12 | 4.8 | 17.6 | 46.2 | 6.3 | 66.3 | 97.3 | 0.97 | 23.9 | 43 | 0.6 | 66 | 720 | 121 |
| 30-10-2022 | 1.09 | 4.2 | 17.0 | 44.0 | 4.5 | 82.3 | 125.8 | 0.86 | 24.7 | 43 | 0.5 | 60 | 720 | 174 |
| 31-10-2022 | 1.29 | 3.5 | 24.3 | 47.5 | 5.1 | 94.9 | 154.4 | 1.03 | 25.5 | 43 | 0.6 | 62 | 719 | 216 |
| Average | 1.02 | 13.8 | 17.8 | 38.0 | 5.5 | 68.0 | 100.2 | 0.95 | 24.6 | 66 | 0.8 | 156 | 719 | * |
| Maximum | 1.52 | 27.3 | 28.3 | 60.0 | 6.4 | 183.8 | 228.2 | 1.84 | 25.7 | 83 | 1.2 | 262 | 722 | * |
| Minimum | 0.69 | 3.5 | 10.7 | 25.3 | 4.5 | 30.1 | 51.7 | 0.60 | 22.3 | 43 | 0.3 | 12 | 715 | * |

| NIMHANS-Oct-2022 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | CO mg/m³ | Ozone µg/m³ | NO2 µg/m³ | NH3 µg/m³ | SO2 µg/m³ | PM2.5 µg/m³ | PM10 µg/m³ | BEN µg/m³ | AT °C | RH % | WS m/s | WD deg. | BP mmHg | AQI |
| 01-10-2022 | 0.45 | 19.1 | 19.1 | 2.1 | 4.3 | 14 | 29 | 0.37 | 21.3 | 54 | 2.3 | 292 | 706 | 29 |
| 02-10-2022 | 0.49 | 20.8 | 17.6 | 2.1 | 4.6 | 21 | 47 | 0.38 | 22.7 | 47 | 2.1 | 275 | 706 | 47 |
| 03-10-2022 | 0.47 | 15.5 | 22.9 | 2.1 | 5.0 | 28 | 64 | 0.49 | 22.5 | 48 | 1.7 | 273 | 706 | 64 |
| 04-10-2022 | 0.49 | 22.6 | 19.8 | 2.1 | 4.6 | 27 | 64 | 0.42 | 23.7 | 44 | 1.6 | 249 | 705 | 64 |
| 05-10-2022 | 0.48 | 23.7 | 18.0 | 2.1 | 4.9 | 26 | 60 | 0.39 | 23.4 | 46 | 1.7 | 265 | 705 | 60 |
| 06-10-2022 | 0.48 | 20.2 | 17.8 | 2.1 | 4.5 | 20 | 46 | 0.41 | 21.8 | 55 | 1.9 | 281 | 705 | 46 |
| 07-10-2022 | 0.50 | 17.6 | 19.9 | 2.2 | 4.5 | 20 | 44 | 0.38 | 23.3 | 53 | 1.6 | 271 | 706 | 44 |
| 08-10-2022 | 0.51 | 17.9 | 19.7 | 2.1 | 4.9 | 23 | 53 | 0.38 | 23.8 | 52 | 1.0 | 167 | 707 | 53 |
| 09-10-2022 | 0.49 | 14.5 | 21.7 | 2.0 | 4.7 | 21 | 48 | 0.40 | 22.5 | 62 | 0.8 | 197 | 707 | 48 |
| 10-10-2022 | 0.50 | 14.8 | 21.9 | 2.1 | 4.2 | 18 | 39 | 0.38 | 23.5 | 60 | 0.7 | 189 | 708 | 39 |
| 11-10-2022 | 0.47 | 16.2 | 21.8 | 2.0 | 4.4 | 21 | 47 | 0.38 | 23.8 | 59 | 1.1 | 174 | 707 | 47 |
| 12-10-2022 | 0.58 | 15.5 | 22.0 | 2.0 | 5.2 | 19 | 42 | 0.41 | 23.6 | 57 | 0.9 | 195 | 708 | 42 |
| 13-10-2022 | 0.49 | 16.2 | 24.1 | 2.1 | 5.5 | 21 | 47 | 0.39 | 23.9 | 58 | 0.8 | 159 | 707 | 47 |
| 14-10-2022 | 0.62 | 19.3 | 26.3 | 2.1 | 4.8 | 20 | 46 | 0.46 | 22.4 | 63 | 1.0 | 211 | 707 | 46 |
| 15-10-2022 | 0.33 | 17.1 | 22.0 | 2.1 | 4.6 | 12 | 24 | 0.35 | 23.4 | 60 | 1.0 | 174 | 706 | 28 |
| 16-10-2022 | 0.50 | 16.6 | 22.2 | 2.0 | 4.4 | 16 | 36 | 0.40 | 22.9 | 61 | 0.7 | 165 | 708 | 36 |
| 17-10-2022 | 0.56 | 11.0 | 21.4 | 2.0 | 4.7 | 15 | 33 | 0.37 | 23.6 | 58 | 0.9 | 156 | 709 | 33 |
| 18-10-2022 | 0.38 | 18.3 | 18.9 | 2.0 | 5.4 | 15 | 32 | 0.36 | 22.6 | 60 | 1.0 | 260 | 710 | 32 |
| 19-10-2022 | 0.46 | 22.7 | 23.7 | 2.0 | 5.0 | 15 | 34 | 0.36 | 23.6 | 57 | 0.9 | 173 | 708 | 34 |
| 20-10-2022 | 0.46 | 19.9 | 24.5 | 2.1 | 3.6 | 21 | 47 | 0.40 | 23.7 | 57 | 0.9 | 157 | 709 | 47 |
| 21-10-2022 | 0.50 | 15.8 | 25.5 | 2.1 | 4.0 | 22 | 50 | 0.38 | 23.2 | 57 | 0.8 | 137 | 710 | 50 |
| 22-10-2022 | 0.39 | 26.2 | 25.9 | 2.0 | 4.2 | 34 | 79 | 0.41 | 23.5 | 52 | 1.0 | 107 | 710 | 79 |
| 23-10-2022 | 0.30 | 35.3 | 23.4 | 2.0 | 4.9 | 35 | 82 | 0.35 | 22.8 | 43 | 1.1 | 203 | 709 | 82 |
| 24-10-2022 | 0.34 | 34.4 | 24.3 | 2.0 | 5.2 | 35 | 83 | 0.37 | 21.4 | 39 | 1.4 | 232 | 708 | 83 |
| 25-10-2022 | 0.60 | 30.0 | 32.7 | 2.0 | 5.7 | 47 | 114 | 0.58 | 21.8 | 42 | 0.8 | 192 | 709 | 109 |
| 26-10-2022 | 0.60 | 33.9 | 33.4 | 2.0 | 5.4 | 48 | 115 | 0.49 | 22.7 | 41 | 0.6 | 150 | 710 | 110 |
| 27-10-2022 | 0.67 | 24.4 | 27.3 | 2.1 | 5.2 | 33 | 78 | 0.46 | 22.8 | 46 | 0.8 | 143 | 709 | 78 |
| 28-10-2022 | 0.36 | 29.7 | 19.3 | 2.1 | 4.4 | 24 | 55 | 0.31 | 22.6 | 53 | 1.2 | 101 | 708 | 55 |
| 29-10-2022 | 0.35 | 26.2 | 21.3 | 2.1 | 5.1 | 26 | 61 | 0.31 | 22.2 | 57 | 1.0 | 125 | 709 | 61 |
| 30-10-2022 | 0.40 | 28.2 | 20.4 | 2.0 | 4.4 | 31 | 73 | 0.30 | 23.0 | 55 | 0.9 | 112 | 709 | 73 |
| 31-10-2022 | 0.35 | 40.0 | 23.8 | 2.1 | 4.9 | 42 | 101 | 0.35 | 23.4 | 45 | 1.0 | 106 | 708 | 101 |
| Average | 0.47 | 22.1 | 22.7 | 2.1 | 4.8 | 25 | 57 | 0.39 | 22.9 | 53 | 1.1 | 190 | 708 | * |
| Maximum | 0.67 | 40.0 | 33.4 | 2.2 | 5.7 | 48 | 115 | 0.58 | 23.9 | 63 | 2.3 | 292 | 710 | * |
| Minimum | 0.30 | 11.0 | 17.6 | 2.0 | 3.6 | 12 | 24 | 0.30 | 21.3 | 39 | 0.6 | 101 | 705 | * |

| Sanneguruvanahalli-Oct-2022 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | NO2 ug/m3 | SO2 ug/m3 | CO mg/m3 | PM10 ug/m3 | TEMP degree C | HR % | WS m/s | WD degree | SR W/m2 | AQI |
| 01-10-2022 | 17.9 | 6.2 | 0.47 | 48.8 | 25.75 | 77.95 | 0.38 | 62.12 | 119.90 | 49 |
| 02-10-2022 | 17.9 | 4.5 | 0.48 | 48.8 | 25.89 | 78.01 | 0.35 | 52.27 | 106.22 | 49 |
| 03-10-2022 | 20.3 | 4.3 | 0.48 | 48.1 | 25.46 | 82.36 | 0.29 | 129.47 | 111.38 | 48 |
| 04-10-2022 | 18.4 | 3.9 | 0.46 | 45.3 | 24.92 | 82.12 | 0.18 | 194.42 | 118.21 | 45 |
| 05-10-2022 | 18.3 | 2.8 | 0.46 | 45.3 | 24.93 | 81.94 | 0.20 | 215.06 | 94.74 | 45 |
| 06-10-2022 | 18.6 | 4.3 | 0.49 | 45.3 | 24.75 | 84.11 | 0.19 | 200.01 | 112.87 | 45 |
| 07-10-2022 | 19.2 | 3.2 | 0.47 | 45.5 | 24.64 | 84.13 | 0.23 | 148.68 | 104.30 | 46 |
| 08-10-2022 | 18.9 | 6.8 | 0.42 | 45.8 | 24.55 | 84.31 | 0.21 | 149.11 | 122.92 | 46 |
| 09-10-2022 | 18.4 | 6.8 | 0.45 | 44.0 | 24.52 | 83.76 | 0.21 | 172.59 | 75.57 | 44 |
| 10-10-2022 | 19.3 | 4.6 | 0.48 | 44.0 | 24.89 | 81.97 | 0.19 | 211.66 | 75.83 | 44 |
| 11-10-2022 | 21.5 | 4.8 | 0.47 | 43.2 | 24.34 | 84.36 | 0.43 | 53.86 | 45.11 | 43 |
| 12-10-2022 | 20.6 | 3.6 | 0.37 | 42.0 | 25.18 | 78.44 | 0.23 | 140.25 | 116.17 | 42 |
| 13-10-2022 | 20.6 | 3.8 | 0.42 | 41.3 | 24.63 | 83.59 | 0.20 | 177.69 | 68.72 | 41 |
| 14-10-2022 | 22.1 | 5.9 | 0.43 | 41.5 | 24.22 | 84.23 | 0.14 | 192.06 | 107.82 | 42 |
| 15-10-2022 | 20.9 | 3.2 | 0.41 | 41.4 | 24.88 | 83.54 | 0.22 | 133.03 | 47.02 | 41 |
| 16-10-2022 | 19.0 | 2.4 | 0.41 | 41.4 | 24.85 | 83.26 | 0.35 | 46.08 | 42.17 | 41 |
| 17-10-2022 | 22.7 | 5.5 | 0.42 | 20.8 | 24.76 | 70.04 | 0.19 | 209.14 | 43.78 | 28 |
| 18-10-2022 | 24.0 | 4.0 | 0.49 | 30.6 | 24.61 | 70.91 | 0.21 | 133.70 | 73.12 | 31 |
| 19-10-2022 | 23.5 | 4.1 | 0.50 | 65.1 | 25.09 | 71.44 | 0.19 | 145.29 | 97.10 | 65 |
| 20-10-2022 | 22.7 | 4.4 | 0.40 | 56.4 | 26.04 | 72.59 | 0.21 | 138.96 | 117.17 | 55 |
| 21-10-2022 | 21.0 | 5.5 | 0.40 | 30.9 | 24.50 | 71.57 | 0.14 | 188.68 | 107.75 | 31 |
| 22-10-2022 | 19.6 | 3.5 | 0.46 | 37.1 | 24.55 | 70.92 | 0.21 | 144.24 | 41.58 | 37 |
| 23-10-2022 | 18.8 | 4.1 | 0.41 | 40.7 | 24.57 | 70.79 | 0.35 | 45.97 | 40.60 | 41 |
| 24-10-2022 | 21.6 | 11.2 | 0.63 | 72.3 | 25.32 | 71.25 | 0.39 | 46.93 | 133.74 | 72 |
| 25-10-2022 | 25.1 | 14.4 | 0.77 | 97.7 | 25.28 | 71.87 | 0.39 | 154.45 | 96.43 | 98 |
| 26-10-2022 | 26.8 | 15.6 | 1.16 | 107.0 | 25.11 | 73.16 | 0.19 | 105.16 | 108.16 | 105 |
| 27-10-2022 | 23.4 | 7.5 | 0.98 | 69.1 | 24.63 | 75.43 | 0.15 | 191.15 | 113.69 | 69 |
| 28-10-2022 | 21.1 | 7.1 | 0.78 | 62.9 | 24.51 | 75.45 | 0.35 | 216.85 | 99.83 | 63 |
| 29-10-2022 | 21.2 | 9.3 | 0.77 | 58.3 | 24.52 | 75.24 | 0.21 | 186.56 | 110.60 | 58 |
| 30-10-2022 | 20.8 | 7.0 | 0.78 | 52.1 | 24.56 | 75.36 | 0.25 | 196.73 | 98.03 | 52 |
| 31-10-2022 | 22.4 | 5.8 | 0.80 | 48.3 | 24.61 | 75.46 | 0.23 | 202.12 | 139.04 | 48 |
| Minimum | 17.9 | 2.2 | 0.37 | 31.1 | 24.22 | 71.38 | 0.10 | 45.97 | 40.60 | * |
| Maximum | 26.7 | 20.8 | 1.14 | 106.6 | 26.53 | 84.36 | 0.43 | 216.85 | 139.04 | * |
| Average | 20.9 | 5.8 | 0.55 | 50.2 | 25.30 | 80.16 | 0.23 | 148.14 | 93.21 | * |

| City Railway Station-Oct-2022 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Date | NO2 ug/m3 | SO2 ug/m3 | CO mg/m3 | PM10 ug/m3 | AQI |
| 01-10-2022 | 22.5 | 18.7 | 1.33 | 53.1 | 67 |
| 02-10-2022 | 22.5 | 18.6 | 1.44 | 59.1 | 72 |
| 03-10-2022 | 22.5 | 18.6 | 1.58 | 57.0 | 79 |
| 04-10-2022 | 22.4 | 18.6 | 1.56 | 52.7 | 78 |
| 05-10-2022 | 22.5 | 18.6 | 1.11 | 57.8 | 58 |
| 06-10-2022 | 22.5 | 18.6 | 1.52 | 60.7 | 76 |
| 07-10-2022 | 22.4 | 18.6 | 1.55 | 60.4 | 78 |
| 08-10-2022 | 22.4 | 18.6 | 1.21 | 67.9 | 68 |
| 09-10-2022 | 22.6 | 18.6 | 1.37 | 51.3 | 69 |
| 10-10-2022 | 22.5 | 18.7 | 1.54 | 53.6 | 77 |
| 11-10-2022 | 22.6 | 18.6 | 1.35 | 52.5 | 68 |
| 12-10-2022 | 22.5 | 18.6 | 1.36 | 45.0 | 68 |
| 13-10-2022 | 22.4 | 18.6 | 1.41 | 36.0 | 71 |
| 14-10-2022 | 22.5 | 18.6 | 1.47 | 48.1 | 74 |
| 15-10-2022 | 22.5 | 18.6 | 1.23 | 73.0 | 73 |
| 16-10-2022 | 22.5 | 18.7 | 1.58 | 89.3 | 89 |
| 17-10-2022 | 21.4 | 19.1 | 1.84 | 57.7 | 92 |
| 18-10-2022 | 22.6 | 20.1 | 1.89 | 61.3 | 95 |
| 19-10-2022 | 21.0 | 19.6 | 2.43 | 45.1 | 105 |
| 20-10-2022 | 21.4 | 20.7 | 2.13 | 59.6 | 102 |
| 21-10-2022 | 21.3 | 21.6 | 1.85 | 86.5 | 93 |
| 22-10-2022 | 21.3 | 20.6 | 1.82 | 119.4 | 113 |
| 23-10-2022 | 21.3 | 21.0 | 1.61 | 88.2 | 55 |
| 24-10-2022 | 22.6 | 23.1 | 1.54 | 96.9 | 97 |
| 25-10-2022 | 23.9 | 23.9 | 1.94 | 113.2 | 109 |
| 26-10-2022 | 28.7 | 30.2 | 2.07 | 113.3 | 109 |
| 27-10-2022 | 25.5 | 16.3 | 1.85 | 82.8 | 93 |
| 28-10-2022 | 22.0 | 14.2 | 1.68 | 54.6 | 84 |
| 29-10-2022 | 22.9 | 16.4 | 1.41 | 65.7 | 71 |
| 30-10-2022 | 23.7 | 17.3 | 1.42 | 80.2 | 80 |
| 31-10-2022 | 23.6 | 17.3 | 1.46 | 89.6 | 90 |
| Minimum | 20.5 | 14.2 | 1.11 | 36.0 | * |
| Maximum | 28.7 | 30.2 | 2.21 | 119.4 | * |
| Average | 22.6 | 19.3 | 1.56 | 70.9 | * |

| Silkboard-Oct-2022 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | CO mg/m³ | Ozone µg/m³ | NO2 µg/m³ | NH3 µg/m³ | SO2 µg/m³ | PM2.5 µg/m³ | PM10 µg/m³ | BEN µg/m³ | AT °C | RH % | WS m/s | WD deg. | BP mmHg | AQI |
| 01-10-2022 | - | - | - | - | - | - | - | - | - | - | - | - | - | * |
| 02-10-2022 | - | - | - | - | - | - | - | - | - | - | - | - | - | * |
| 03-10-2022 | - | - | - | - | - | - | - | - | - | - | - | - | - | * |
| 04-10-2022 | 0.56 | 24.6 | 16.3 | 53.1 | 5.7 | 31 | 123 | 0.13 | 24.1 | 71 | 1.0 | 245 | 686 | 115 |
| 05-10-2022 | 0.94 | 25.0 | 24.3 | 47.1 | 5.6 | 32 | 116 | 0.23 | 23.9 | 72 | 1.4 | 239 | 686 | 111 |
| 06-10-2022 | 1.04 | 23.1 | 20.1 | 48.6 | 5.5 | 31 | 121 | 0.26 | 23.0 | 74 | 1.4 | 235 | 686 | 114 |
| 07-10-2022 | 0.86 | 22.0 | 10.1 | 27.8 | 5.7 | 27 | 111 | 0.22 | 23.0 | 70 | 1.2 | 212 | 687 | 107 |
| 08-10-2022 | 0.96 | 22.9 | 24.1 | 28.0 | 5.4 | 29 | 112 | 0.24 | 24.1 | 68 | 0.9 | 171 | 687 | 108 |
| 09-10-2022 | 1.02 | 21.3 | 40.8 | 20.4 | 5.5 | 17 | 71 | 0.26 | 22.5 | 82 | 0.8 | 166 | 688 | 71 |
| 10-10-2022 | 0.91 | 21.8 | 30.5 | 14.7 | 5.5 | 20 | 55 | 0.23 | 23.0 | 81 | 0.8 | 163 | 688 | 55 |
| 11-10-2022 | 0.56 | 25.2 | 26.8 | 11.2 | 5.6 | 20 | 62 | 0.14 | 23.9 | 77 | 1.2 | 130 | 688 | 62 |
| 12-10-2022 | 0.87 | 23.1 | 23.4 | 14.1 | 5.4 | 19 | 85 | 0.22 | 23.7 | 76 | 1.1 | 142 | 688 | 85 |
| 13-10-2022 | 0.56 | 26.2 | 27.9 | 12.0 | 5.6 | 21 | 57 | 0.14 | 24.4 | 75 | 1.0 | 106 | 688 | 57 |
| 14-10-2022 | 1.25 | 25.7 | 28.5 | 9.0 | 5.7 | 28 | 69 | 0.31 | 22.7 | 82 | 1.0 | 189 | 687 | 69 |
| 15-10-2022 | 0.76 | 26.2 | 33.9 | 5.0 | 5.7 | 12 | 38 | 0.19 | 23.5 | 79 | 1.1 | 152 | 687 | 42 |
| 16-10-2022 | 0.57 | 26.9 | 34.3 | 5.2 | 5.7 | 15 | 34 | 0.14 | 23.0 | 80 | 0.8 | 124 | 689 | 43 |
| 17-10-2022 | 0.47 | 24.2 | 36.0 | 4.2 | 5.7 | 12 | 34 | 0.12 | 23.7 | 77 | 1.0 | 111 | 689 | 45 |
| 18-10-2022 | 0.99 | 24.1 | 29.1 | 8.6 | 5.7 | 21 | 70 | 0.25 | 23.1 | 79 | 1.1 | 192 | 689 | 70 |
| 19-10-2022 | 0.78 | 26.6 | 31.8 | 7.5 | 5.6 | 19 | 72 | 0.19 | 23.7 | 75 | 1.0 | 156 | 689 | 72 |
| 20-10-2022 | 0.65 | 27.0 | 33.8 | 5.8 | 5.5 | 22 | 70 | 0.16 | 23.8 | 75 | 0.9 | 118 | 689 | 70 |
| 21-10-2022 | 0.63 | 26.0 | 23.9 | 9.3 | 5.7 | 25 | 96 | 0.16 | 23.2 | 74 | 0.9 | 109 | 690 | 96 |
| 22-10-2022 | 0.77 | 28.6 | 12.9 | 16.7 | 5.7 | 47 | 144 | 0.20 | 23.5 | 68 | 0.9 | 161 | 690 | 129 |
| 23-10-2022 | 0.92 | 31.5 | 24.9 | 18.8 | 7.2 | 55 | 181 | 0.23 | 23.0 | 55 | 1.1 | 225 | 689 | 154 |
| 24-10-2022 | 1.07 | 28.6 | 39.5 | 23.2 | 10.1 | 146 | 312 | 0.27 | 21.7 | 52 | 1.2 | 180 | 688 | 320 |
| 25-10-2022 | 1.36 | 29.4 | 28.5 | 39.6 | 9.0 | 121 | 274 | 0.34 | 22.1 | 55 | 0.8 | 163 | 689 | 301 |
| 26-10-2022 | 1.04 | 32.3 | 35.7 | 34.2 | 8.9 | 69 | 202 | 0.26 | 21.6 | 58 | 0.6 | 112 | 690 | 168 |
| 27-10-2022 | 0.91 | 29.3 | 27.3 | 31.3 | 7.0 | 44 | 143 | 0.23 | 22.7 | 60 | 0.9 | 104 | 690 | 129 |
| 28-10-2022 | 0.61 | 29.3 | 21.1 | 20.0 | 5.9 | 31 | 92 | 0.15 | 22.4 | 70 | 1.2 | 119 | 689 | 92 |
| 29-10-2022 | 0.53 | 30.7 | 25.9 | 11.0 | 5.8 | 32 | 66 | 0.13 | 22.0 | 75 | 1.1 | 99 | 689 | 66 |
| 30-10-2022 | 0.60 | 28.6 | 24.5 | 12.2 | 5.8 | 45 | 96 | 0.15 | 22.6 | 75 | 0.9 | 111 | 689 | 96 |
| 31-10-2022 | 0.73 | 32.0 | 23.5 | 13.9 | 5.8 | 56 | 147 | 0.18 | 22.8 | 62 | 0.9 | 128 | 689 | 131 |
| Average | 0.82 | 26.5 | 27.1 | 19.7 | 6.1 | 37 | 109 | 0.21 | 23.1 | 71 | 1.0 | 156 | 688 | * |
| Maximum | 1.36 | 32.3 | 40.8 | 53.1 | 10.1 | 146 | 312 | 0.34 | 24.4 | 82 | 1.4 | 245 | 690 | * |
| Minimum | 0.47 | 21.3 | 10.1 | 4.2 | 5.4 | 12 | 34 | 0.12 | 21.6 | 52 | 0.6 | 99 | 686 | * |

INFERENCE:

The Air Quality monitored through the 7 CAAQMs (Continuous Ambient Air  Quality Monitoring Stations) installed in Bengaluru at City  Railway Station, Hebbal, Saneguruvanahalli, NIMHANS, Jayanagar, KAVIKA-Mysore  Road and Central Silk Board-HSR Layout, for the month of October-2022 reveals that  the Air quality is in Good and Satisfactory in almost all days over the number of days  monitored. However, the Air Quality during the Diwali (Oct-24 to Oct-26) has shown  variations to Poor and Very Poor only in 3 stations viz., Jayanagar, KAVIKA and  Central Silk Board. This is observed in a spike as shown in the above graph.

Further, the Air quality has shown Moderate to Poor in almost all stations at the  month end and this may be due high vehicular movement. It is also observed that on  the onset of winter, the Air quality has changed to moderate in some stations compared  to previous months as this may be also due to one of Environment Factor known as  “Winter Inversion”.