KARNATAKA STATE POLLUTION CONTROL BOARD No.49, ParisaraBhavan, Church Street, Bengaluru-560 001.

Air Quality Monitoring Cell

Air Quality Index of CAAQM Stations monitored in Bengaluru city-January-2023 Air Quality Index-(AQI)

Air Quality Index is a tool for effective communication of air quality status to people  in terms, which are easy to understand. It transforms complex air quality data of  various pollutants into a single number (index value), nomenclature and colour.

There are six AQI categories, namely Good, Satisfactory, Moderate, Poor,  Very Poor, and Severe. Each of these categories is decided based on ambient concentration values of air pollutants and their likely health impacts (known as health  breakpoints). AQ sub-index and health breakpoints are evolved for eight pollutants  (PM10, PM2.5, NO2, SO2, CO, O3, NH3, and Pb) for which short-term (upto 24-hours) National Ambient Air Quality Standards are prescribed.

Based on the measured ambient concentrations of a pollutant, sub-index is  calculated, which is a linear function of concentration (e.g. the sub-index for PM2.5 will  be 51 at concentration 31 µg/m3, 100 at concentration 60 µg/m3, and 75 at  concentration of 45(µg/m3). The worst sub-index determines the overall AQI. AQI  categories and health breakpoints for the eight pollutants are as follow:

The Karnataka State Pollution Control Board has installed 7 Continuous  Ambient Air Quality Monitoring Stations (CAAQMs) covering Residential,  Commercial, Industrial and Silence Zones. The Air Quality is being monitored round

the clock, 24 hours a day for the Primary and secondary pollutants as well as  Meteorological parameters. The Air Quality monitored during the month of January 2023 is as shown below:

| Daily AQI Values of Bengaluru (January-2023) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date / Station Name | City Railway Station, Majestic | Nisarga Bhavan, Saneguruvanahalli | Veterinary College, Hebbal | Shalini Ground, Jayanagar | KAVIKA, Mysore Road | NIMHANS | H.S.R Layout, Near Central Silkboard |
| 01-01-2023 | 107 | 55 | 78 | 78 | 143 | 75 | 78 |
| 02-01-2023 | 107 | 54 | 80 | 83 | 93 | 69 | 67 |
| 03-01-2023 | 107 | 56 | 85 | 83 | 87 | 71 | 74 |
| 04-01-2023 | 107 | 58 | 83 | 92 | 93 | 75 | 79 |
| 05-01-2023 | 107 | 51 | 75 | 90 | 86 | 76 | 74 |
| 06-01-2023 | 107 | 61 | 89 | 106 | 107 | 85 | 89 |
| 07-01-2023 | 107 | 64 | 101 | 108 | 153 | 99 | 105 |
| 08-01-2023 | 107 | 65 | 106 | 160 | 205 | 115 | 149 |
| 09-01-2023 | 107 | 55 | 107 | 105 | 120 | 97 | 103 |
| 10-01-2023 | 107 | 52 | 94 | 101 | 111 | 93 | 103 |
| 11-01-2023 | 94 | 53 | 96 | 99 | 108 | 91 | 100 |
| 12-01-2023 | 94 | 56 | 90 | 94 | 114 | 89 | 88 |
| 13-01-2023 | 88 | 55 | 101 | 104 | 113 | 97 | 91 |
| 14-01-2023 | 100 | 59 | 112 | 101 | 132 | 96 | 103 |
| 15-01-2023 | 82 | 53 | 77 | 81 | 116 | 70 | 77 |
| 16-01-2023 | 87 | 53 | 86 | 88 | 118 | 81 | 77 |
| 17-01-2023 | 94 | 56 | 81 | 157 | 86 | 78 | 66 |
| 18-01-2023 | 94 | 58 | 83 | 90 | 96 | 73 | 74 |
| 19-01-2023 | 94 | 58 | 88 | 89 | 99 | 82 | 83 |
| 20-01-2023 | 95 | 54 | 95 | 96 | 101 | 86 | 87 |
| 21-01-2023 | 99 | 66 | 96 | 93 | 99 | 89 | 85 |
| 22-01-2023 | 94 | 63 | 82 | 90 | * | 75 | 76 |
| 23-01-2023 | 94 | 51 | 87 | 83 | 68 | 73 | 67 |
| 24-01-2023 | 94 | 53 | 71 | 76 | 76 | 64 | 55 |
| 25-01-2023 | 93 | 58 | 93 | 93 | 96 | 80 | 75 |
| 26-01-2023 | 93 | 61 | * | 106 | 94 | 92 | 91 |
| 27-01-2023 | 94 | 53 | 85 | 101 | 102 | 83 | 87 |
| 28-01-2023 | 94 | 49 | 84 | 92 | 105 | 79 | 80 |
| 29-01-2023 | 94 | 58 | 83 | 100 | 104 | 80 | 78 |
| 30-01-2023 | 94 | 44 | 72 | 82 | 100 | 71 | 71 |
| 31-01-2023 | 94 | 54 | 71 | 87 | 112 | 63 | 72 |
| Min | 82 | 44 | 71 | 76 | 68 | 63 | 55 |
| Max | 107 | 66 | 112 | 160 | 205 | 115 | 149 |

* Data Not available

| Good | Satisfactory | Moderate | Poor | Very Poor | Severe |
| --- | --- | --- | --- | --- | --- |
| (0-50) | (51-100) | (101-200) | (201-300) | (301-400) | (>401) |

| City Railway Station-January-2023 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Date | NO2 ug/m3 | SO2 ug/m3 | CO mg/m3 | PM10 ug/m3 | AQI |
| 01-01-2023 | 26.5 | 13.5 | 2.55 | 81.5 | 107 |
| 02-01-2023 | 26.4 | 13.6 | 2.56 | 81.7 | 107 |
| 03-01-2023 | 26.6 | 13.6 | 2.56 | 82.5 | 107 |
| 04-01-2023 | 26.5 | 13.6 | 2.55 | 81.7 | 107 |
| 05-01-2023 | 26.5 | 13.5 | 2.53 | 79.9 | 107 |
| 06-01-2023 | 26.5 | 13.5 | 2.56 | 85.3 | 107 |
| 07-01-2023 | 26.5 | 13.6 | 2.56 | 87.6 | 107 |
| 08-01-2023 | 26.6 | 13.6 | 2.55 | 100.7 | 107 |
| 09-01-2023 | 26.5 | 13.5 | 2.54 | 82.5 | 107 |
| 10-01-2023 | 26.5 | 13.5 | 2.52 | 89.7 | 107 |
| 11-01-2023 | 26.5 | 13.6 | 1.88 | 88.3 | 94 |
| 12-01-2023 | 26.5 | 13.6 | 1.88 | 88.5 | 94 |
| 13-01-2023 | 24.9 | 14.6 | 1.76 | 85.3 | 88 |
| 14-01-2023 | 21.5 | 16.5 | 2.01 | 61.2 | 100 |
| 15-01-2023 | 21.5 | 16.6 | 1.63 | 78.6 | 82 |
| 16-01-2023 | 24.5 | 14.9 | 1.74 | 67.7 | 87 |
| 17-01-2023 | 26.5 | 13.6 | 1.88 | 86.3 | 94 |
| 18-01-2023 | 26.6 | 13.6 | 1.88 | 87.9 | 94 |
| 19-01-2023 | 26.6 | 13.5 | 1.88 | 84.6 | 94 |
| 20-01-2023 | 26.5 | 13.5 | 1.89 | 93.8 | 95 |
| 21-01-2023 | 26.6 | 13.6 | 1.88 | 98.5 | 99 |
| 22-01-2023 | 26.7 | 13.5 | 1.88 | 87.8 | 94 |
| 23-01-2023 | 26.6 | 13.5 | 1.88 | 82.7 | 94 |
| 24-01-2023 | 26.6 | 13.6 | 1.88 | 79.9 | 94 |
| 25-01-2023 | 26.6 | 13.6 | 1.86 | 83.9 | 93 |
| 26-01-2023 | 26.5 | 13.6 | 1.85 | 90.6 | 93 |
| 27-01-2023 | 26.5 | 13.6 | 1.88 | 88.1 | 94 |
| 28-01-2023 | 26.6 | 13.6 | 1.87 | 85.6 | 94 |
| 29-01-2023 | 26.5 | 13.6 | 1.88 | 94.3 | 94 |
| 30-01-2023 | 26.4 | 13.5 | 1.88 | 81.3 | 94 |
| 31-01-2023 | 26.5 | 13.5 | 1.88 | 86.5 | 94 |
| Minimum | 21.5 | 13.5 | 1.63 | 61.2 | * |
| Maximum | 26.7 | 16.6 | 2.54 | 100.7 | * |
| Average | 26.1 | 13.8 | 2.06 | 85.0 | * |

| Saneguruvanahalli-January-2023 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | NO2 ug/m3 | SO2 ug/m3 | CO mg/m3 | PM10 ug/m3 | TEMP degree C | HR % | WS m/s | WD degree | SR W/m2 | AQI |
| 01-01-2023 | 22.9 | 6.5 | 0.84 | 55.4 | 24.98 | 75.85 | 0.20 | 209.71 | 167.99 | 55 |
| 02-01-2023 | 23.2 | 7.3 | 0.80 | 54.2 | 24.90 | 68.97 | 0.36 | 190.76 | 211.12 | 54 |
| 03-01-2023 | 22.8 | 6.6 | 0.78 | 55.8 | 24.88 | 67.82 | 0.20 | 195.26 | 204.60 | 56 |
| 04-01-2023 | 22.1 | 6.1 | 0.77 | 57.7 | 24.88 | 70.30 | 0.21 | 191.50 | 180.14 | 58 |
| 05-01-2023 | 23.3 | 8.6 | 0.75 | 50.5 | 24.89 | 66.49 | 0.17 | 188.64 | 188.18 | 51 |
| 06-01-2023 | 22.4 | 7.2 | 0.80 | 60.5 | 24.92 | 71.85 | 0.17 | 188.80 | 181.95 | 61 |
| 07-01-2023 | 21.4 | 6.4 | 0.82 | 64.1 | 24.95 | 72.14 | 0.14 | 204.97 | 161.42 | 64 |
| 08-01-2023 | 21.4 | 8.7 | 0.79 | 65.3 | 25.02 | 78.90 | 0.14 | 208.40 | 162.79 | 65 |
| 09-01-2023 | 23.7 | 13.5 | 0.79 | 55.4 | 24.94 | 69.29 | 0.16 | 196.69 | 217.95 | 55 |
| 10-01-2023 | 24.4 | 5.7 | 0.80 | 52.1 | 24.92 | 73.90 | 0.16 | 194.29 | 152.35 | 52 |
| 11-01-2023 | 24.8 | 7.1 | 0.82 | 52.7 | 24.92 | 76.44 | 0.25 | 187.51 | 148.61 | 53 |
| 12-01-2023 | 24.3 | 12.1 | 0.76 | 55.7 | 24.94 | 81.23 | 0.21 | 203.42 | 152.97 | 56 |
| 13-01-2023 | 23.8 | 12.7 | 0.81 | 55.2 | 24.97 | 76.37 | 0.22 | 194.14 | 149.24 | 55 |
| 14-01-2023 | 24.1 | 9.1 | 0.86 | 58.9 | 24.93 | 80.41 | 0.21 | 199.89 | 162.36 | 59 |
| 15-01-2023 | 22.9 | 6.6 | 0.81 | 52.9 | 24.93 | 78.64 | 0.21 | 184.95 | 153.64 | 53 |
| 16-01-2023 | 23.6 | 3.3 | 0.84 | 53.2 | 24.96 | 75.24 | 0.22 | 189.23 | 163.12 | 53 |
| 17-01-2023 | 26.3 | 10.6 | 0.82 | 55.7 | 24.89 | 70.61 | 0.29 | 197.26 | 175.73 | 56 |
| 18-01-2023 | 27.1 | 6.1 | 0.83 | 58.3 | 24.66 | 68.52 | 0.33 | 192.81 | 173.19 | 58 |
| 19-01-2023 | 24.2 | 9.3 | 0.82 | 57.9 | 24.73 | 68.48 | 0.33 | 180.46 | 173.03 | 58 |
| 20-01-2023 | 24.6 | 7.0 | 0.80 | 54.1 | 24.96 | 69.22 | 0.46 | 194.64 | 172.32 | 54 |
| 21-01-2023 | 23.6 | 5.3 | 0.77 | 66.1 | 24.65 | 64.99 | 0.22 | 188.70 | 208.70 | 66 |
| 22-01-2023 | 21.6 | 5.7 | 0.78 | 63.1 | 24.73 | 67.11 | 0.20 | 190.71 | 187.74 | 63 |
| 23-01-2023 | 23.4 | 7.6 | 0.80 | 50.8 | 24.76 | 71.87 | 0.21 | 189.73 | 184.43 | 51 |
| 24-01-2023 | 24.9 | 7.9 | 0.81 | 52.9 | 24.86 | 75.08 | 0.24 | 195.71 | 181.70 | 53 |
| 25-01-2023 | 26.2 | 6.7 | 0.83 | 58.2 | 24.86 | 65.79 | 0.23 | 193.59 | 226.08 | 58 |
| 26-01-2023 | 22.3 | 6.2 | 0.78 | 61.0 | 24.88 | 74.26 | 0.22 | 194.11 | 180.06 | 61 |
| 27-01-2023 | 24.8 | 8.7 | 0.83 | 53.1 | 24.90 | 77.34 | 0.20 | 192.21 | 164.44 | 53 |
| 28-01-2023 | 22.2 | 22.2 | 0.80 | 48.5 | 24.86 | 76.03 | 0.20 | 189.13 | 179.64 | 49 |
| 29-01-2023 | 21.5 | 18.8 | 0.81 | 58.0 | 24.90 | 76.11 | 0.20 | 200.82 | 212.70 | 58 |
| 30-01-2023 | 24.3 | 7.9 | 0.82 | 43.8 | 24.94 | 74.05 | 0.15 | 185.69 | 217.79 | 44 |
| 31-01-2023 | 22.8 | 8.5 | 0.85 | 53.9 | 25.15 | 70.85 | 0.15 | 243.60 | 212.08 | 54 |
| Minimum | 21.4 | 3.3 | 0.75 | 43.8 | 24.65 | 64.99 | 0.14 | 180.46 | 148.61 | * |
| Maximum | 27.1 | 22.2 | 0.86 | 66.1 | 25.15 | 81.23 | 0.46 | 243.60 | 226.08 | * |
| Average | 23.6 | 8.6 | 0.81 | 56.0 | 24.89 | 72.71 | 0.22 | 195.40 | 180.90 | * |

| Hebbal-January-2023 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | CO (mg/m³) | Ozone (µg/m³) | NO2 (µg/m³) | NH3 (µg/m³) | SO2 (µg/m³) | PM2.5 (µg/m³) | PM10 (µg/m³) | BEN. (µg/m³) | AT (°C) | RH (%) | WS (m/s) | WD (deg) | BP (mmHg) | AQI |
| 01-01-2023 | 0.45 | 22.0 | 29.3 | 4.9 | 4.0 | 21.7 | 77.7 | 0.1 | 22.8 | 60 | 0.9 | 108 | 714 | 78 |
| 02-01-2023 | 0.48 | 35.3 | 38.5 | 5.8 | 4.2 | 22.4 | 80.4 | 0.1 | 22.0 | 61 | 0.9 | 108 | 714 | 80 |
| 03-01-2023 | 0.47 | 31.5 | 42.5 | 7.0 | 4.3 | 24.0 | 85.1 | 0.2 | 21.7 | 67 | 0.9 | 111 | 715 | 85 |
| 04-01-2023 | 0.44 | 27.8 | 37.1 | 5.7 | 4.2 | 23.3 | 83.0 | 0.1 | 21.4 | 67 | 0.9 | 112 | 713 | 83 |
| 05-01-2023 | 0.42 | 38.0 | 42.3 | 5.9 | 4.3 | 21.2 | 74.9 | 0.1 | 20.4 | 72 | 0.9 | 106 | 715 | 75 |
| 06-01-2023 | 0.37 | 36.0 | 37.0 | 5.9 | 4.4 | 24.9 | 88.8 | 0.1 | 21.3 | 69 | 0.9 | 108 | 715 | 89 |
| 07-01-2023 | 0.41 | 22.5 | 39.1 | 7.5 | 4.8 | 28.3 | 100.8 | 0.1 | 21.4 | 69 | 0.9 | 106 | 715 | 101 |
| 08-01-2023 | 0.37 | 35.8 | 30.6 | 5.8 | 4.6 | 30.7 | 109.7 | 0.1 | 20.8 | 66 | 0.8 | 98 | 714 | 106 |
| 09-01-2023 | 0.51 | 26.6 | 43.7 | 4.8 | 4.6 | 30.7 | 110.0 | 0.2 | 20.4 | 57 | 0.8 | 102 | 715 | 107 |
| 10-01-2023 | 0.49 | 68.3 | 51.0 | 7.5 | 4.0 | 26.5 | 94.3 | 0.2 | 20.5 | 61 | 0.8 | 103 | 714 | 94 |
| 11-01-2023 | 0.52 | 58.6 | 54.8 | 7.4 | 4.3 | 26.8 | 95.5 | 0.2 | 21.2 | 51 | 0.8 | 96 | 712 | 96 |
| 12-01-2023 | 0.64 | 66.5 | 52.8 | 10.9 | 4.1 | 25.0 | 89.5 | 0.2 | 21.9 | 51 | 0.9 | 107 | 711 | 90 |
| 13-01-2023 | 0.52 | 81.1 | 46.9 | 8.5 | 4.4 | 28.3 | 100.8 | 0.2 | 22.0 | 55 | 0.9 | 110 | 712 | 101 |
| 14-01-2023 | 0.58 | 22.7 | 46.6 | 7.1 | 4.2 | 33.1 | 118.6 | 0.2 | 21.2 | 52 | 0.9 | 103 | 711 | 112 |
| 15-01-2023 | 0.51 | 19.4 | 33.4 | 6.8 | 2.9 | 21.6 | 77.2 | 0.2 | 21.9 | 52 | 0.9 | 114 | 710 | 77 |
| 16-01-2023 | 0.45 | 75.1 | 45.7 | 9.1 | 3.3 | 24.0 | 86.3 | 0.2 | 22.1 | 53 | 0.9 | 108 | 711 | 86 |
| 17-01-2023 | 0.49 | 18.3 | 36.3 | 8.2 | 4.5 | 22.8 | 81.3 | 0.2 | 21.9 | 56 | 0.9 | 113 | 712 | 81 |
| 18-01-2023 | 0.45 | 34.1 | 39.3 | 8.0 | - | 23.3 | 83.1 | 0.2 | 21.1 | 62 | 0.9 | 113 | 713 | 83 |
| 19-01-2023 | 0.46 | 46.0 | 38.0 | 7.4 | 19.3 | 24.6 | 88.0 | 0.2 | 21.4 | 57 | 0.9 | 111 | 712 | 88 |
| 20-01-2023 | 0.36 | 24.5 | 35.0 | 9.0 | 3.4 | 27.0 | 95.2 | 0.1 | 20.3 | 71 | 0.9 | 115 | 712 | 95 |
| 21-01-2023 | 0.45 | 35.3 | 36.3 | 9.3 | 22.6 | 26.7 | 95.5 | 0.1 | 21.1 | 67 | 0.9 | 106 | 712 | 96 |
| 22-01-2023 | 0.38 | 11.0 | 28.7 | 6.8 | 3.0 | 22.8 | 81.5 | 0.1 | 21.5 | 62 | 0.9 | 110 | 711 | 82 |
| 23-01-2023 | 0.48 | 22.0 | 37.8 | 6.7 | 7.0 | 24.4 | 87.0 | 0.2 | 20.7 | 74 | 0.9 | 114 | 710 | 87 |
| 24-01-2023 | 0.41 | 29.8 | 32.6 | 6.7 | 7.5 | 19.8 | 70.9 | 0.1 | 21.8 | 69 | 0.9 | 123 | 710 | 71 |
| 25-01-2023 | 0.48 | 41.8 | 41.2 | 7.4 | 8.2 | 25.9 | 92.7 | 0.2 | 21.9 | 62 | 0.9 | 117 | 710 | 93 |
| 26-01-2023 | 0.46 | 15.9 | 32.7 | 6.7 | 7.3 | - | - | 0.1 | 22.2 | 58 | 0.9 | 114 | 673 | * |
| 27-01-2023 | 0.43 | 29.3 | 36.4 | 7.4 | 8.2 | 23.4 | 84.5 | 0.1 | 22.3 | 58 | 0.9 | 108 | 711 | 85 |
| 28-01-2023 | 0.45 | 14.8 | 34.7 | 6.4 | 8.4 | 23.7 | 84.3 | 0.1 | 22.3 | 53 | 0.8 | 101 | 664 | 84 |
| 29-01-2023 | 0.40 | 12.4 | 32.8 | 6.0 | 8.2 | 23.1 | 82.8 | 0.1 | 22.6 | 61 | 0.9 | 113 | 710 | 83 |
| 30-01-2023 | 0.44 | 11.3 | 32.5 | 5.4 | 8.2 | 20.3 | 72.4 | 0.1 | 22.9 | 70 | 0.8 | 102 | 711 | 72 |
| 31-01-2023 | 0.46 | 18.1 | 28.2 | 5.1 | 8.2 | 19.7 | 70.5 | 0.2 | 22.1 | 63 | 0.8 | 102 | 712 | 71 |
| Average | 0.46 | 33.3 | 38.5 | 7.0 | 6.4 | 24.7 | 88.1 | 0.2 | 21.6 | 61 | 0.9 | 108 | 710 | * |
| Minimum | 0.36 | 11.0 | 28.2 | 4.8 | 2.9 | 19.7 | 70.5 | 0.1 | 20.3 | 51 | 0.8 | 96 | 664 | * |
| Maximum | 0.64 | 81.1 | 54.8 | 10.9 | 22.6 | 33.1 | 118.6 | 0.2 | 22.9 | 74 | 0.9 | 123 | 715 | * |

| Jayanagar-January-2023 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | CO mg/m³ | Ozone µg/m³ | NO2 (µg/m³) | NH3 (µg/m³) | SO2 (µg/m³) | PM2.5 (µg/m³) | PM10 (µg/m³) | BEN µg/m³ | AT °C | RH % | WS m/s | WD deg. | BP mmHg | AQI |
| 01-01-2023 | 0.38 | 17.7 | 21.6 | 13.7 | 5.7 | 44.3 | 78.3 | 0.21 | 23.3 | 59 | 0.5 | 122 | 713 | 78 |
| 02-01-2023 | 0.42 | 17.6 | 22.9 | 13.6 | 5.6 | 41.5 | 82.8 | 0.23 | 22.5 | 61 | 0.5 | 134 | 713 | 83 |
| 03-01-2023 | 0.42 | 16.8 | 30.5 | 14.1 | 5.7 | 43.8 | 82.8 | 0.23 | 22.2 | 65 | 0.6 | 125 | 713 | 83 |
| 04-01-2023 | 0.40 | 18.0 | 21.0 | 9.5 | 5.7 | 48.1 | 91.9 | 0.22 | 21.9 | 65 | 0.6 | 122 | 712 | 92 |
| 05-01-2023 | 0.44 | 19.9 | 3.8 | 7.0 | 5.9 | 44.6 | 89.5 | 0.25 | 20.9 | 70 | 0.6 | 114 | 713 | 90 |
| 06-01-2023 | 0.44 | 20.6 | 4.4 | 6.9 | 6.0 | 54.4 | 109.0 | 0.25 | 21.7 | 68 | 0.7 | 117 | 714 | 106 |
| 07-01-2023 | 0.51 | 20.7 | 5.4 | 6.9 | 5.6 | 62.5 | 111.9 | 0.29 | 21.8 | 67 | 0.6 | 112 | 713 | 108 |
| 08-01-2023 | 0.53 | 21.2 | 5.0 | 7.2 | 5.3 | 78.0 | 124.7 | 0.29 | 21.5 | 64 | 0.5 | 107 | 712 | 160 |
| 09-01-2023 | 0.48 | 16.4 | 14.8 | 7.5 | 5.5 | 56.8 | 108.0 | 0.27 | 21.1 | 55 | 0.5 | 114 | 713 | 105 |
| 10-01-2023 | 0.52 | 14.5 | 20.1 | 7.8 | 5.7 | 53.4 | 101.8 | 0.29 | 21.1 | 60 | 0.6 | 103 | 712 | 101 |
| 11-01-2023 | 0.51 | 17.9 | 15.9 | 7.6 | 3.9 | 43.8 | 99.0 | 0.29 | 21.8 | 49 | 0.5 | 112 | 711 | 99 |
| 12-01-2023 | 0.53 | 18.1 | 14.5 | 7.7 | 2.7 | 43.4 | 94.3 | 0.30 | 22.8 | 48 | 0.5 | 110 | 710 | 94 |
| 13-01-2023 | 0.64 | 18.2 | 15.0 | 8.1 | 2.7 | 47.4 | 105.6 | 0.34 | 23.0 | 51 | 0.5 | 108 | 711 | 104 |
| 14-01-2023 | 0.45 | 18.2 | 15.4 | 8.4 | 2.6 | 48.9 | 101.7 | 0.25 | 22.6 | 46 | 0.4 | 121 | 710 | 101 |
| 15-01-2023 | 0.28 | 18.0 | 15.3 | 9.4 | 2.8 | 40.8 | 80.7 | 0.16 | 22.6 | 49 | 0.5 | 116 | 709 | 81 |
| 16-01-2023 | 0.46 | 18.0 | 15.1 | 8.6 | 3.6 | 41.4 | 87.7 | 0.26 | 22.8 | 51 | 0.5 | 123 | 710 | 88 |
| 17-01-2023 | 0.42 | 17.8 | 14.7 | 8.7 | 3.2 | 77.0 | 81.9 | 0.23 | 22.4 | 54 | 0.5 | 113 | 711 | 157 |
| 18-01-2023 | 0.40 | 17.8 | 14.6 | 8.4 | 2.9 | 42.7 | 90.3 | 0.22 | 21.6 | 61 | 0.6 | 111 | 712 | 90 |
| 19-01-2023 | 0.45 | 17.8 | 14.7 | 8.5 | 2.9 | 46.1 | 89.3 | 0.25 | 22.0 | 56 | 0.6 | 117 | 712 | 89 |
| 20-01-2023 | 0.44 | 17.9 | 14.2 | 8.3 | 3.1 | 52.1 | 96.2 | 0.25 | 21.7 | 66 | 0.6 | 121 | 712 | 96 |
| 21-01-2023 | 0.45 | 25.2 | 17.2 | 8.0 | 2.9 | 49.9 | 92.6 | 0.26 | 21.5 | 65 | 0.5 | 117 | 711 | 93 |
| 22-01-2023 | 0.34 | 26.5 | 17.9 | 7.8 | 2.7 | 45.0 | 90.2 | 0.19 | 21.9 | 60 | 0.6 | 115 | 710 | 90 |
| 23-01-2023 | 0.46 | 26.1 | 17.2 | 7.7 | 2.8 | 40.5 | 82.8 | 0.26 | 21.1 | 72 | 0.6 | 115 | 710 | 83 |
| 24-01-2023 | 0.39 | 23.1 | 17.3 | 7.6 | 3.1 | 34.3 | 76.3 | 0.22 | 22.1 | 67 | 0.6 | 116 | 710 | 76 |
| 25-01-2023 | 0.44 | 17.5 | 18.4 | 7.7 | 3.3 | 47.0 | 92.8 | 0.24 | 22.2 | 60 | 0.6 | 112 | 710 | 93 |
| 26-01-2023 | 0.34 | 17.5 | 18.4 | 7.7 | 2.6 | 58.3 | 109.4 | 0.19 | 22.5 | 58 | 0.6 | 117 | 710 | 106 |
| 27-01-2023 | 0.38 | 17.5 | 18.3 | 7.5 | 3.0 | 49.5 | 101.5 | 0.21 | 22.6 | 56 | 0.5 | 115 | 710 | 101 |
| 28-01-2023 | 0.39 | 27.5 | 18.6 | 7.4 | 2.6 | 42.5 | 92.3 | 0.22 | 22.7 | 53 | 0.6 | 112 | 709 | 92 |
| 29-01-2023 | 0.33 | 29.5 | 18.5 | 7.4 | 2.2 | 47.0 | 99.8 | 0.18 | 23.0 | 59 | 0.6 | 113 | 709 | 100 |
| 30-01-2023 | 0.50 | 21.0 | 17.7 | 7.5 | 2.1 | 41.8 | 82.0 | 0.28 | 23.3 | 67 | 0.5 | 107 | 710 | 82 |
| 31-01-2023 | 0.53 | 21.1 | 18.2 | 7.3 | 2.4 | 38.9 | 86.5 | 0.30 | 23.0 | 59 | 0.5 | 113 | 709 | 87 |
| Average | 0.44 | 19.9 | 16.0 | 8.4 | 3.8 | 48.6 | 94.0 | 0.25 | 22.2 | 59 | 0.6 | 115 | 711 | * |
| Maximum | 0.64 | 29.5 | 30.5 | 14.1 | 6.0 | 78.0 | 124.7 | 0.34 | 23.3 | 72 | 0.7 | 134 | 714 | * |
| Minimum | 0.28 | 14.5 | 3.8 | 6.9 | 2.1 | 34.3 | 76.3 | 0.16 | 20.9 | 46 | 0.4 | 103 | 709 | * |

| KAVIKA-January-2023 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | CO mg/m³ | Ozone µg/m³ | NO2 µg/m³ | NH3 µg/m³ | SO2 µg/m³ | PM2.5 µg/m³ | PM10 µg/m³ | BEN µg/m³ | AT °C | RH % | WS m/s | WD deg. | BP mmHg | AQI |
| 01-01-2023 | 1.00 | 20.9 | 67.4 | 44.3 | 6.5 | 72.8 | 100.8 | 0.12 | 24.4 | 44 | 0.8 | 133 | 724 | 143 |
| 02-01-2023 | 0.81 | 21.5 | 67.3 | 43.6 | 5.5 | 55.9 | 90.5 | 0.14 | 23.4 | 44 | 0.9 | 142 | 724 | 93 |
| 03-01-2023 | 0.80 | 21.8 | 64.0 | 45.0 | 6.6 | 52.3 | 85.5 | 0.14 | 22.8 | 44 | 0.9 | 140 | 724 | 87 |
| 04-01-2023 | 0.87 | 21.6 | 61.5 | 39.7 | 6.8 | 56.0 | 89.1 | 0.10 | 22.4 | 44 | 0.9 | 138 | 723 | 93 |
| 05-01-2023 | 0.86 | 22.0 | 64.2 | 34.5 | 6.3 | 46.0 | 86.0 | 0.10 | 21.9 | 44 | 0.9 | 134 | 724 | 86 |
| 06-01-2023 | 0.98 | 22.2 | 66.0 | 34.4 | 6.3 | 61.3 | 110.9 | 0.13 | 22.4 | 44 | 0.9 | 127 | 725 | 107 |
| 07-01-2023 | 1.21 | 21.8 | 73.4 | 31.6 | 6.2 | 75.9 | 123.0 | 0.12 | 22.6 | 44 | 0.9 | 126 | 724 | 153 |
| 08-01-2023 | 1.24 | 21.9 | 73.4 | 32.1 | 4.7 | 91.4 | 142.1 | 0.28 | 22.6 | 44 | 0.8 | 125 | 723 | 205 |
| 09-01-2023 | 1.29 | 22.0 | - | - | 5.7 | 53.6 | 130.3 | 0.18 | 22.1 | 44 | 0.9 | 133 | 724 | 120 |
| 10-01-2023 | 1.07 | 22.1 | - | - | 7.3 | 41.0 | 117.1 | 0.20 | 22.0 | 44 | 0.9 | 131 | 723 | 111 |
| 11-01-2023 | 1.13 | 21.7 | - | - | 6.9 | 39.5 | 112.5 | 0.19 | 22.9 | 44 | 0.9 | 136 | 721 | 108 |
| 12-01-2023 | 1.35 | 22.2 | - | - | 6.9 | 42.1 | 120.3 | 0.20 | 23.9 | 44 | 0.9 | 140 | 720 | 114 |
| 13-01-2023 | 1.25 | 23.0 | - | - | 6.2 | 41.7 | 119.2 | 0.17 | 23.8 | 44 | 0.9 | 146 | 721 | 113 |
| 14-01-2023 | 1.41 | 22.7 | - | - | 5.8 | 51.9 | 148.0 | 1.19 | 23.3 | 44 | 0.9 | 140 | 721 | 132 |
| 15-01-2023 | 0.98 | 23.2 | - | - | 6.2 | 43.2 | 123.3 | 0.82 | 23.8 | 44 | 1.0 | 147 | 720 | 116 |
| 16-01-2023 | 1.13 | 23.1 | - | - | 5.8 | 44.3 | 126.7 | 0.11 | 24.0 | 44 | 1.0 | 151 | 721 | 118 |
| 17-01-2023 | 0.77 | 22.4 | - | - | 5.7 | 30.0 | 85.8 | 0.10 | 23.4 | 44 | 1.0 | 148 | 722 | 86 |
| 18-01-2023 | 0.94 | 21.8 | - | - | 6.0 | 33.6 | 95.9 | 0.10 | 22.4 | 44 | 0.9 | 137 | 723 | 96 |
| 19-01-2023 | 0.80 | 22.3 | - | - | 3.9 | 34.7 | 99.3 | 0.10 | 18.6 | 45 | 0.9 | 139 | 723 | 99 |
| 20-01-2023 | 0.80 | 21.9 | - | - | 4.8 | 35.3 | 100.8 | 0.10 | 22.3 | 44 | 0.9 | 142 | 723 | 101 |
| 21-01-2023 | 1.12 | 21.1 | - | - | 5.1 | 35.1 | 98.7 | - | 21.2 | 44 | 0.9 | 137 | 724 | 99 |
| 22-01-2023 | - | - | - | - | - | - | - | - | - | - | - | - | - | * |
| 23-01-2023 | 0.88 | 22.2 | - | - | 6.0 | 24.2 | 67.8 | - | 22.1 | 44 | 0.9 | 144 | 720 | 68 |
| 24-01-2023 | 0.72 | 21.2 | - | - | 6.3 | 26.6 | 76.0 | - | 22.8 | 44 | 1.0 | 147 | 720 | 76 |
| 25-01-2023 | 0.86 | 22.0 | - | - | 5.5 | 33.6 | 96.0 | 0.12 | 22.9 | 44 | 0.9 | 143 | 720 | 96 |
| 26-01-2023 | 0.81 | 21.7 | - | - | 6.4 | 33.1 | 94.4 | 0.11 | 23.2 | 44 | 0.9 | 144 | 720 | 94 |
| 27-01-2023 | 0.91 | 22.3 | - | - | 6.3 | 35.7 | 102.8 | 0.13 | 23.5 | 44 | 0.9 | 144 | 720 | 102 |
| 28-01-2023 | 0.92 | 21.8 | - | - | 6.7 | 37.5 | 107.3 | 0.13 | 23.7 | 44 | 0.9 | 138 | 720 | 105 |
| 29-01-2023 | 0.83 | 21.6 | - | - | 5.6 | 36.9 | 105.3 | 0.12 | 23.8 | 44 | 0.9 | 131 | 719 | 104 |
| 30-01-2023 | 1.02 | 20.8 | - | - | 6.9 | 35.1 | 100.5 | 0.14 | 24.4 | 44 | 0.9 | 132 | 720 | 100 |
| 31-01-2023 | 1.24 | 20.7 | - | - | 6.8 | 41.4 | 118.5 | 0.17 | 24.3 | 44 | 0.8 | 116 | 720 | 112 |
| Average | 1.00 | 21.9 | 67.2 | 38.2 | 6.1 | 44.7 | 105.8 | 0.20 | 22.9 | 44 | 0.9 | 138 | 722 | * |
| Maximum | 1.41 | 23.2 | 73.4 | 45.0 | 7.3 | 91.4 | 148.0 | 1.19 | 24.4 | 45 | 1.0 | 151 | 725 | * |
| Minimum | 0.72 | 20.7 | 61.5 | 31.6 | 3.9 | 24.2 | 67.8 | 0.10 | 18.6 | 44 | 0.8 | 116 | 719 | * |

| NIMHANS-January-2023 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | CO mg/m³ | Ozone µg/m³ | NO2 µg/m³ | NH3 µg/m³ | SO2 µg/m³ | PM2.5 µg/m³ | PM10 µg/m³ | BEN µg/m³ | AT °C | RH % | WS m/s | WD deg. | BP mmHg | AQI |
| 01-01-2023 | 0.55 | 31.2 | 23.6 | 2.1 | 4.7 | 30.0 | 75.0 | 0.24 | 22.4 | 61 | 0.8 | 126 | 713 | 75 |
| 02-01-2023 | 0.38 | 22.8 | 23.6 | 2.1 | 4.1 | 27.4 | 68.8 | 0.23 | 21.4 | 63 | 0.9 | 139 | 713 | 69 |
| 03-01-2023 | 0.38 | 25.3 | 21.7 | 2.1 | 4.4 | 28.5 | 71.3 | 0.20 | 21.1 | 68 | 1.0 | 130 | 713 | 71 |
| 04-01-2023 | 0.40 | 29.8 | 20.3 | 2.0 | 4.0 | 30.0 | 75.0 | 0.39 | 21.0 | 67 | 1.3 | 130 | 712 | 75 |
| 05-01-2023 | 0.30 | 27.0 | 20.3 | 2.1 | 4.6 | 30.4 | 76.0 | 0.42 | 20.3 | 71 | 1.2 | 131 | 712 | 76 |
| 06-01-2023 | 0.23 | 30.8 | 19.5 | 2.1 | 4.6 | 34.0 | 84.8 | 0.32 | 21.0 | 69 | 1.6 | 128 | 713 | 85 |
| 07-01-2023 | 0.38 | 30.4 | 21.0 | 2.1 | 5.0 | 39.8 | 99.4 | 0.54 | 21.1 | 69 | 1.1 | 126 | 713 | 99 |
| 08-01-2023 | 0.65 | 31.6 | 23.3 | 2.1 | 4.8 | 49.1 | 122.5 | 0.91 | 20.7 | 66 | 0.8 | 124 | 712 | 115 |
| 09-01-2023 | 0.60 | 28.1 | 27.6 | 2.0 | 5.3 | 38.9 | 97.3 | 0.82 | 19.9 | 58 | 0.8 | 136 | 713 | 97 |
| 10-01-2023 | 0.49 | 29.3 | 25.8 | 2.1 | 5.3 | 37.1 | 92.8 | 0.68 | 20.1 | 62 | 0.9 | 129 | 712 | 93 |
| 11-01-2023 | 0.61 | 21.5 | 32.6 | 1.9 | 5.9 | 36.3 | 91.3 | 0.80 | 20.5 | 51 | 0.7 | 138 | 710 | 91 |
| 12-01-2023 | 0.55 | 24.7 | 31.9 | 2.0 | 5.0 | 35.4 | 88.5 | 0.71 | 21.5 | 50 | 0.7 | 137 | 709 | 89 |
| 13-01-2023 | 0.44 | 26.4 | 32.4 | 2.0 | 5.1 | 38.9 | 97.2 | 0.59 | 21.4 | 55 | 0.7 | 130 | 710 | 97 |
| 14-01-2023 | 0.55 | 24.9 | 31.5 | 2.0 | 4.6 | 38.4 | 96.0 | 0.74 | 20.6 | 53 | 0.6 | 144 | 710 | 96 |
| 15-01-2023 | 0.26 | 27.4 | 25.0 | 2.1 | 5.1 | 27.8 | 69.5 | 0.32 | 21.4 | 51 | 0.7 | 139 | 709 | 70 |
| 16-01-2023 | 0.39 | 27.9 | 29.7 | 2.0 | 5.5 | 32.3 | 80.8 | 0.51 | 21.8 | 52 | 0.8 | 143 | 710 | 81 |
| 17-01-2023 | 0.31 | 22.2 | 24.9 | 2.0 | 5.0 | 31.2 | 78.0 | 0.43 | 21.2 | 56 | 1.0 | 138 | 711 | 78 |
| 18-01-2023 | 0.29 | 27.1 | 23.8 | 2.1 | 4.9 | 29.5 | 73.3 | 0.40 | 20.6 | 63 | 1.2 | 131 | 712 | 73 |
| 19-01-2023 | 0.37 | 29.2 | 24.3 | 2.1 | 4.9 | 32.7 | 81.7 | 0.51 | 20.9 | 58 | 1.1 | 127 | 711 | 82 |
| 20-01-2023 | 0.39 | 24.4 | 22.5 | 2.1 | 4.8 | 34.5 | 86.3 | 0.54 | 20.6 | 68 | 1.2 | 137 | 711 | 86 |
| 21-01-2023 | 0.80 | 20.3 | 25.8 | 2.1 | 4.8 | 35.7 | 89.4 | 0.61 | 20.7 | 67 | 1.0 | 128 | 710 | 89 |
| 22-01-2023 | 0.32 | 27.4 | 18.7 | 2.1 | 3.7 | 30.0 | 75.0 | 0.16 | 20.9 | 62 | 1.3 | 128 | 710 | 75 |
| 23-01-2023 | 0.39 | 21.0 | 21.6 | 2.1 | 4.2 | 29.1 | 73.2 | 0.20 | 20.6 | 74 | 1.3 | 131 | 709 | 73 |
| 24-01-2023 | 0.37 | 23.1 | 20.4 | 2.0 | 4.4 | 25.3 | 63.6 | 0.16 | 21.5 | 69 | 1.4 | 136 | 709 | 64 |
| 25-01-2023 | 0.29 | 27.6 | 23.4 | 2.0 | 4.0 | 32.2 | 80.1 | 0.17 | 21.2 | 62 | 1.3 | 133 | 709 | 80 |
| 26-01-2023 | 0.26 | 30.5 | 20.8 | 2.1 | 4.8 | 36.7 | 91.9 | 0.18 | 21.6 | 59 | 1.1 | 133 | 709 | 92 |
| 27-01-2023 | 0.26 | 26.2 | 23.6 | 2.0 | 4.6 | 33.9 | 83.3 | 0.20 | 21.5 | 58 | 1.0 | 137 | 709 | 83 |
| 28-01-2023 | 0.30 | 25.8 | 24.0 | 2.1 | 4.8 | 31.7 | 79.0 | 0.20 | 21.7 | 55 | 1.1 | 130 | 709 | 79 |
| 29-01-2023 | 0.27 | 30.3 | 19.9 | 2.1 | 4.1 | 31.8 | 79.5 | 0.18 | 22.2 | 60 | 1.3 | 128 | 708 | 80 |
| 30-01-2023 | 0.32 | 23.8 | 20.6 | 2.1 | 4.9 | 28.2 | 70.5 | 0.21 | 22.7 | 68 | 1.1 | 127 | 709 | 71 |
| 31-01-2023 | 0.31 | 25.3 | 21.3 | 2.0 | 3.7 | 25.4 | 63.4 | 0.20 | 22.4 | 59 | 1.0 | 112 | 709 | 63 |
| Average | 0.40 | 26.6 | 24.0 | 2.1 | 4.7 | 33.0 | 82.4 | 0.41 | 21.2 | 61 | 1.0 | 132 | 711 | * |
| Maximum | 0.80 | 31.6 | 32.6 | 2.1 | 5.9 | 49.1 | 122.5 | 0.91 | 22.7 | 74 | 1.6 | 144 | 713 | * |
| Minimum | 0.23 | 20.3 | 18.7 | 1.9 | 3.7 | 25.3 | 63.4 | 0.16 | 19.9 | 50 | 0.6 | 112 | 708 | * |

| Silkboard-January-2023 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | CO mg/m³ | Ozone µg/m³ | NO2 µg/m³ | NH3 µg/m³ | SO2 µg/m³ | PM2.5 µg/m³ | PM10 µg/m³ | BEN µg/m³ | AT °C | RH % | WS m/s | WD deg. | BP mmHg | AQI |
| 01-01-2023 | 0.52 | 31.1 | 26.0 | 19.6 | 5.5 | 43.4 | 78.1 | 0.13 | 22.3 | 60 | 0.9 | 92 | 692 | 78 |
| 02-01-2023 | 0.40 | 29.8 | 36.5 | 16.4 | 5.4 | 37.4 | 67.3 | 0.10 | 21.5 | 62 | 1.1 | 89 | 692 | 67 |
| 03-01-2023 | 0.43 | 30.9 | 46.1 | 15.0 | 5.3 | 41.0 | 73.8 | 0.11 | 21.1 | 67 | 1.1 | 91 | 692 | 74 |
| 04-01-2023 | 0.40 | 33.8 | 20.8 | 17.7 | 5.4 | 44.0 | 79.2 | 0.10 | 20.8 | 67 | 1.4 | 89 | 691 | 79 |
| 05-01-2023 | 0.50 | 33.1 | 18.4 | 22.1 | 5.3 | 41.2 | 74.2 | 0.12 | 20.4 | 71 | 1.3 | 93 | 692 | 74 |
| 06-01-2023 | 0.53 | 35.8 | 14.7 | 20.0 | 5.3 | 49.3 | 88.9 | 0.13 | 20.8 | 70 | 1.5 | 87 | 692 | 89 |
| 07-01-2023 | 0.61 | 34.2 | 8.1 | 19.2 | 5.5 | 59.9 | 107.9 | 0.15 | 21.0 | 69 | 1.2 | 88 | 692 | 105 |
| 08-01-2023 | 0.72 | 34.1 | 10.4 | 21.2 | 5.3 | 74.8 | 134.6 | 0.18 | 20.7 | 66 | 1.0 | 99 | 691 | 149 |
| 09-01-2023 | 0.73 | 31.8 | 18.6 | 16.8 | 5.3 | 58.3 | 104.9 | 0.18 | 20.0 | 57 | 1.0 | 86 | 692 | 103 |
| 10-01-2023 | 0.79 | 30.0 | 9.8 | 17.5 | 5.5 | 57.9 | 104.1 | 0.20 | 20.0 | 62 | 1.0 | 107 | 691 | 103 |
| 11-01-2023 | 0.98 | 25.0 | 20.8 | 23.9 | 5.4 | 55.5 | 100.0 | 0.25 | 20.5 | 50 | 0.9 | 88 | 690 | 100 |
| 12-01-2023 | 0.64 | 27.7 | 29.6 | 24.6 | 5.4 | 48.8 | 87.9 | 0.16 | 21.6 | 50 | 0.9 | 89 | 689 | 88 |
| 13-01-2023 | 0.65 | 29.3 | 33.8 | 23.8 | 5.6 | 50.7 | 91.1 | 0.16 | 21.3 | 55 | 0.9 | 89 | 690 | 91 |
| 14-01-2023 | 0.79 | 27.6 | 31.9 | 22.3 | 5.5 | 57.9 | 104.2 | 0.20 | 20.7 | 53 | 0.8 | 87 | 689 | 103 |
| 15-01-2023 | 0.46 | 29.8 | 39.3 | 18.0 | 5.3 | 42.5 | 76.5 | 0.12 | 21.6 | 51 | 0.9 | 90 | 689 | 77 |
| 16-01-2023 | 0.57 | 28.5 | 39.4 | 17.5 | 5.4 | 42.9 | 77.2 | 0.14 | 21.8 | 52 | 0.9 | 98 | 689 | 77 |
| 17-01-2023 | 0.40 | 26.8 | 44.6 | 12.9 | 5.4 | 36.9 | 66.3 | 0.10 | 21.1 | 56 | 1.0 | 91 | 690 | 66 |
| 18-01-2023 | 0.47 | 28.6 | 46.6 | 11.1 | 5.6 | 40.9 | 73.6 | 0.12 | 20.3 | 63 | 1.1 | 92 | 691 | 74 |
| 19-01-2023 | 0.44 | 30.2 | 47.1 | 10.8 | 5.4 | 46.1 | 83.0 | 0.11 | 20.8 | 58 | 1.1 | 92 | 691 | 83 |
| 20-01-2023 | 0.38 | 31.0 | 48.7 | 9.7 | 5.4 | 48.5 | 87.4 | 0.09 | 20.6 | 69 | 1.2 | 96 | 691 | 87 |
| 21-01-2023 | 0.50 | 28.2 | 32.2 | 10.5 | 5.3 | 47.2 | 85.0 | 0.12 | 20.5 | 67 | 1.1 | 91 | 690 | 85 |
| 22-01-2023 | 0.39 | 32.2 | 30.6 | 9.0 | 5.4 | 42.0 | 75.6 | 0.10 | 20.8 | 63 | 1.3 | 93 | 689 | 76 |
| 23-01-2023 | 0.49 | 28.7 | 30.4 | 9.3 | 5.3 | 37.3 | 67.2 | 0.12 | 20.3 | 75 | 1.2 | 95 | 689 | 67 |
| 24-01-2023 | 0.35 | 31.3 | 24.4 | 8.5 | 5.4 | 30.4 | 54.5 | 0.09 | 21.2 | 69 | 1.3 | 96 | 689 | 55 |
| 25-01-2023 | 0.41 | 32.6 | 12.4 | 8.3 | 5.5 | 41.5 | 74.6 | 0.10 | 21.1 | 63 | 1.3 | 95 | 689 | 75 |
| 26-01-2023 | 0.44 | 33.0 | 12.0 | 8.6 | 5.4 | 50.8 | 91.3 | 0.11 | 21.5 | 60 | 1.2 | 95 | 689 | 91 |
| 27-01-2023 | 0.46 | 28.8 | 10.2 | 9.9 | 5.4 | 48.2 | 86.7 | 0.11 | 21.5 | 58 | 1.1 | 113 | 689 | 87 |
| 28-01-2023 | 0.49 | 28.0 | 10.0 | 10.0 | 5.5 | 44.2 | 79.5 | 0.12 | 21.6 | 55 | 1.1 | 108 | 689 | 80 |
| 29-01-2023 | 0.44 | 31.1 | 10.7 | 9.7 | 5.3 | 43.2 | 77.8 | 0.11 | 22.1 | 60 | 1.3 | 97 | 688 | 78 |
| 30-01-2023 | 0.56 | 26.2 | 9.8 | 10.9 | 5.3 | 39.4 | 71.0 | 0.14 | 22.6 | 69 | 1.1 | 94 | 689 | 71 |
| 31-01-2023 | 0.61 | 23.4 | 8.0 | 11.8 | 5.3 | 40.0 | 72.0 | 0.15 | 22.2 | 61 | 1.0 | 133 | 689 | 72 |
| Average | 0.53 | 30.1 | 25.2 | 15.0 | 5.4 | 46.5 | 83.7 | 0.13 | 21.1 | 62 | 1.1 | 95 | 690 | * |
| Maximum | 0.98 | 35.8 | 48.7 | 24.6 | 5.6 | 74.8 | 134.6 | 0.25 | 22.6 | 75 | 1.5 | 133 | 692 | * |
| Minimum | 0.35 | 23.4 | 8.0 | 8.3 | 5.3 | 30.4 | 54.5 | 0.09 | 20.0 | 50 | 0.8 | 86 | 688 | * |

Possible Health Impacts

| Good | Minimal Impact |
| --- | --- |
| Satisfactory | Minor breathing discomfort to sensitive people |
| Moderate | Breathing discomfort to the people with lungs, asthma and heart diseases |
| Poor | Breathing discomfort to most people on prolonged exposure |
| Very Poor | Respiratory illness on prolonged exposure |
| Severe | Affects healthy people and seriously impacts those with existing diseases |

e

u

l

a

V

I

Q

A

AQI in CAAQMs in Bengaluru- January 2023

400

350

300

City Railway Station, Majestic

250

Nisarga Bhavan, Saneguruvanahalli

200

Veterinary College, Hebbal

150

Shalini Ground, Jayanagar

100

KAVIKA, Mysore Road

50

NIMHANS

0

H.S.R Layout, Near Central Silkboard

3 2

0

2

/

1

/

1

3 2

0

2

/

3

/

1

3 2

0

2

/

5

/

1

3 2

0

2

/

7

/

1

3 2

0

2

/

9

/

1

3 2

0

2

/

1

1

/

1

3 2

0

2

/

3

1

/

1

3 2

0

2

/

5

1

/

1

3 2

0

2

/

7

1

/

1

3 2

0

2

/

9

1

/

1

3 2

0

2

/

1

2

/

1

3 2

0

2

/

3

2

/

1

3 2

0

2

/

5

2

/

1

3

2

0

2

/

7

2

/

1

3 2

0

2

/

9

2

/

1

3 2

0

2

/

1

3

/

1

INFERENCE:

The Air Quality monitored through the 7 CAAQMs (Continuous Ambient Air  Quality Monitoring Stations) installed in Bengaluru at City  Railway Station, Hebbal, Saneguruvanahalli, NIMHANS, Jayanagar, KAVIKA Mysore Road and Central Silk Board-HSR Layout, for the month of January-2023 reveals that the Air quality is almost Satisfactory in almost all days over the number  of days monitored.

-----*****-----