KARNATAKA STATE POLLUTION CONTROL BOARD No.49, Parisara Bhavan, Church Street, Bengaluru-560 001.

Air Quality Monitoring Cell

Air Quality Index of CAAQM Stations monitored in Bengaluru city-March-2023• Air Quality Index-(AQI)

Air Quality Index is a tool for effective communication of air quality status to people in  terms, which are easy to understand. It transforms complex air quality data of various  pollutants into a single number (index value), nomenclature and colour.

There are six AQI categories, namely Good, Satisfactory, Moderate, Poor,  Very Poor, and Severe. Each of these categories is decided based on ambient concentration  values of air pollutants and their likely health impacts (known as health breakpoints). AQ sub index and health breakpoints are evolved for eight pollutants (PM10, PM2.5, NO2, SO2, CO, O3,  NH3, and Pb) for which short-term (upto 24-hours) National Ambient Air Quality Standards are prescribed.

Based on the measured ambient concentrations of a pollutant, sub-index is calculated,  which is a linear function of concentration (e.g. the sub-index for PM2.5 will be 51 at  concentration 31 µg/m3, 100 at concentration 60 µg/m3, and 75 at concentration of 45(µg/m3).  The worst sub-index determines the overall AQI. AQI categories and health breakpoints for the  eight pollutants are as follow:

The Karnataka State Pollution Control Board has installed 7 Continuous Ambient Air  Quality Monitoring Stations (CAAQMs) covering Residential, Commercial, Industrial and  Silence Zones. The Air Quality is being monitored round the clock, 24 hours a day for the  Primary and secondary pollutants as well as Meteorological parameters. The Air Quality  monitored during the month of March-2023 is as shown below:

| Daily AQI Values of Bengaluru (March-2023) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date/ Station Name | City Railway Station, Majestic | Nisarga Bhavan, Saneguruvanahalli | Veterinary College, Hebbal | Shalini Ground, Jayanagar | KAVIKA, Mysore Road | NIMHANS | H.S.R Layout, Near Central Silkboard |
| 01-03-2023 | 86 | 54 | 91 | 92 | 96 | 94 | 100 |
| 02-03-2023 | 91 | 47 | 97 | 101 | 103 | 93 | 98 |
| 03-03-2023 | 97 | 52 | 105 | 114 | 110 | 114 | 116 |
| 04-03-2023 | 91 | 49 | 100 | 108 | 103 | 95 | 106 |
| 05-03-2023 | 90 | 45 | 100 | 111 | 94 | 87 | 104 |
| 06-03-2023 | 87 | 49 | 98 | 98 | 102 | 103 | 97 |
| 07-03-2023 | 87 | 43 | 96 | 94 | 98 | 95 | 94 |
| 08-03-2023 | 80 | 41 | 84 | 93 | 96 | 88 | 93 |
| 09-03-2023 | 84 | 57 | 91 | 92 | 97 | 87 | 101 |
| 10-03-2023 | 85 | 55 | 88 | 99 | 97 | 89 | 107 |
| 11-03-2023 | 88 | 59 | 90 | 108 | 104 | 91 | 109 |
| 12-03-2023 | 83 | 58 | 71 | 89 | 77 | 64 | 75 |
| 13-03-2023 | 83 | 59 | 86 | 110 | 81 | 81 | 100 |
| 14-03-2023 | 89 | 51 | 93 | 110 | 99 | 86 | 106 |
| 15-03-2023 | 98 | 40 | 105 | 117 | * | 98 | 124 |
| 16-03-2023 | 97 | 59 | 105 | 99 | 80 | 97 | 128 |
| 17-03-2023 | 80 | 52 | 60 | 76 | 68 | 69 | 97 |
| 18-03-2023 | 82 | 51 | 61 | 59 | 49 | 52 | 78 |
| 19-03-2023 | 94 | 50 | 51 | 53 | 54 | 49 | 82 |
| 20-03-2023 | 105 | 46 | 64 | 51 | 61 | 45 | 83 |
| 21-03-2023 | 87 | 57 | 75 | 92 | * | 67 | 100 |
| 22-03-2023 | 101 | 37 | 90 | 96 | * | 85 | 100 |
| 23-03-2023 | 90 | 46 | 70 | 81 | * | 73 | 90 |
| 24-03-2023 | 84 | 49 | 61 | 72 | * | 61 | 78 |
| 25-03-2023 | 89 | 43 | 68 | 82 | 50 | 60 | 75 |
| 26-03-2023 | 92 | 41 | 65 | 78 | 69 | 62 | 79 |
| 27-03-2023 | 95 | 50 | 68 | * | 93 | 73 | 96 |
| 28-03-2023 | 86 | 74 | 90 | * | 107 | 98 | 109 |
| 29-03-2023 | 84 | 69 | 87 | 79 | 74 | 80 | 108 |
| 30-03-2023 | 80 | 53 | 60 | 69 | 72 | 60 | 87 |
| 31-03-2023 | 90 | 58 | 64 | 77 | 71 | 63 | 84 |
| Min | 80 | 37 | 51 | 51 | 49 | 45 | 75 |
| Max | 105 | 74 | 105 | 117 | 110 | 114 | 128 |

* Data Not available

| Good | Satisfactory | Moderate | Poor | Very Poor | Severe |
| --- | --- | --- | --- | --- | --- |
| (0-50) | (51-100) | (101-200) | (201-300) | (301-400) | (>401) |

| City Railway Station-March-2023 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Date | NO2 ug/m3 | SO2 ug/m3 | CO mg/m3 | PM10 ug/m3 | AQI |
| 01-03-2023 | 26.6 | 19.6 | 0.88 | 85.5 | 86 |
| 02-03-2023 | 26.6 | 19.6 | 0.89 | 91.4 | 91 |
| 03-03-2023 | 26.5 | 19.6 | 0.85 | 96.8 | 97 |
| 04-03-2023 | 26.5 | 19.6 | 0.90 | 91.1 | 91 |
| 05-03-2023 | 26.6 | 19.6 | 0.89 | 90.3 | 90 |
| 06-03-2023 | 26.6 | 19.5 | 1.21 | 86.5 | 87 |
| 07-03-2023 | 26.5 | 19.6 | 1.21 | 87.0 | 87 |
| 08-03-2023 | 26.5 | 19.5 | 1.19 | 80.4 | 80 |
| 09-03-2023 | 26.6 | 19.6 | 1.33 | 83.6 | 84 |
| 10-03-2023 | 26.5 | 19.6 | 1.15 | 84.9 | 85 |
| 11-03-2023 | 26.5 | 19.6 | 1.25 | 88.4 | 88 |
| 12-03-2023 | 26.6 | 19.5 | 1.18 | 82.9 | 83 |
| 13-03-2023 | 26.5 | 19.5 | 1.29 | 83.3 | 83 |
| 14-03-2023 | 26.6 | 19.6 | 1.19 | 89.2 | 89 |
| 15-03-2023 | 26.5 | 19.5 | 1.24 | 97.6 | 98 |
| 16-03-2023 | 26.6 | 19.6 | 1.18 | 97.1 | 97 |
| 17-03-2023 | 26.7 | 19.5 | 1.28 | 79.7 | 80 |
| 18-03-2023 | 26.6 | 19.6 | 1.19 | 82.0 | 82 |
| 19-03-2023 | 26.5 | 19.6 | 1.09 | 94.3 | 94 |
| 20-03-2023 | 26.6 | 19.6 | 1.13 | 107.7 | 105 |
| 21-03-2023 | 26.6 | 19.6 | 1.25 | 86.7 | 87 |
| 22-03-2023 | 26.5 | 19.6 | 1.20 | 100.5 | 101 |
| 23-03-2023 | 26.6 | 19.6 | 1.24 | 89.9 | 90 |
| 24-03-2023 | 26.6 | 19.6 | 1.16 | 83.7 | 84 |
| 25-03-2023 | 26.5 | 19.6 | 1.09 | 88.8 | 89 |
| 26-03-2023 | 26.6 | 19.6 | 1.11 | 91.6 | 92 |
| 27-03-2023 | 26.6 | 19.5 | 1.29 | 95.1 | 95 |
| 28-03-2023 | 26.6 | 19.6 | 1.27 | 86.4 | 86 |
| 29-03-2023 | 26.6 | 19.6 | 1.19 | 83.6 | 84 |
| 30-03-2023 | 26.5 | 19.6 | 1.54 | 79.6 | 80 |
| 31-03-2023 | 26.5 | 19.6 | 1.30 | 89.6 | 90 |
| Minimum | 26.5 | 19.5 | 0.85 | 79.6 | * |
| Maximum | 26.7 | 19.6 | 1.54 | 107.7 | * |
| Average | 26.6 | 19.6 | 1.17 | 88.9 | * |

| Saneguruvanahalli-March-2023 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | NO2 ug/m3 | SO2 ug/m3 | CO mg/m3 | PM10 ug/m3 | TEMP degree C | HR % | WS m/s | WD degree | SR W/m2 | AQI |
| 01-03-2023 | 23.7 | 6.0 | 0.86 | 54.3 | 24.89 | 61.73 | 0.56 | 149.18 | 140.36 | 54 |
| 02-03-2023 | 22.9 | 8.4 | 0.85 | 47.4 | 25.18 | 46.92 | 0.45 | 162.88 | 125.69 | 47 |
| 03-03-2023 | 23.8 | 11.0 | 0.88 | 52.0 | 25.73 | 44.93 | 0.42 | 165.77 | 145.60 | 52 |
| 04-03-2023 | 23.4 | 9.3 | 0.93 | 49.0 | 25.47 | 53.37 | 0.46 | 178.73 | 120.79 | 49 |
| 05-03-2023 | 22.6 | 4.8 | 0.76 | 44.5 | 25.33 | 48.45 | 0.42 | 186.07 | 136.66 | 45 |
| 06-03-2023 | 25.6 | 6.5 | 0.80 | 48.6 | 25.66 | 47.55 | 0.44 | 175.11 | 150.94 | 49 |
| 07-03-2023 | 25.4 | 6.8 | 0.86 | 43.3 | 25.78 | 52.11 | 0.43 | 179.73 | 145.41 | 43 |
| 08-03-2023 | 21.3 | 4.4 | 0.86 | 41.3 | 25.36 | 57.98 | 0.46 | 171.92 | 147.13 | 41 |

09-03-2023 21.4 8.4 0.95 57.1 23.76 65.72 0.60 201.29 151.80 57

| 10-03-2023 | 21.6 | 5.7 | 0.97 | 55.4 | 22.07 | 74.56 | 0.57 | 164.26 | 113.15 | 55 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11-03-2023 | 20.2 | 7.4 | 0.93 | 59.0 | 22.09 | 72.62 | 0.73 | 197.99 | 110.80 | 59 |
| 12-03-2023 | 20.6 | 9.3 | 0.99 | 57.8 | 22.29 | 65.75 | 0.82 | 203.10 | 140.85 | 58 |
| 13-03-2023 | 22.4 | 7.6 | 0.90 | 58.5 | 21.50 | 82.54 | 0.65 | 192.90 | 88.20 | 59 |
| 14-03-2023 | 22.8 | 3.9 | 0.96 | 50.6 | 23.13 | 75.42 | 0.72 | 197.36 | 111.42 | 51 |
| 15-03-2023 | 23.2 | 8.1 | 0.93 | 39.9 | 23.41 | 66.76 | 0.60 | 167.46 | 148.14 | 40 |
| 16-03-2023 | 21.4 | 7.6 | 1.08 | 59.2 | 23.59 | 64.28 | 0.62 | 158.21 | 155.02 | 59 |
| 17-03-2023 | 22.8 | 5.6 | 0.91 | 52.3 | 23.22 | 62.15 | 0.59 | 136.44 | 146.28 | 52 |
| 18-03-2023 | 20.6 | 2.6 | 0.90 | 50.6 | 23.06 | 58.00 | 0.56 | 145.33 | 148.54 | 51 |
| 19-03-2023 | 19.3 | 4.1 | 0.88 | 50.0 | 23.32 | 64.54 | 0.49 | 136.88 | 148.96 | 50 |
| 20-03-2023 | 19.7 | 3.8 | 0.85 | 45.9 | 23.17 | 76.16 | 0.66 | 167.25 | 103.29 | 46 |
| 21-03-2023 | 20.7 | 4.9 | 0.86 | 56.5 | 22.29 | 65.29 | 0.73 | 161.13 | 117.10 | 57 |
| 22-03-2023 | 20.0 | 5.4 | 0.89 | 36.8 | 23.01 | 58.38 | 0.70 | 211.57 | 124.11 | 37 |
| 23-03-2023 | 20.1 | 5.0 | 0.91 | 46.2 | 21.92 | 52.41 | 0.77 | 223.02 | 97.30 | 46 |
| 24-03-2023 | 22.5 | 5.4 | 0.88 | 49.3 | 22.11 | 66.93 | 0.92 | 133.15 | 83.57 | 49 |
| 25-03-2023 | 20.4 | 6.4 | 0.87 | 43.0 | 23.64 | 53.89 | 0.95 | 152.32 | 137.40 | 43 |
| 26-03-2023 | 21.3 | 6.7 | 0.88 | 41.2 | 23.98 | 51.10 | 0.94 | 157.61 | 147.70 | 41 |
| 27-03-2023 | 22.5 | 5.0 | 0.88 | 50.2 | 23.65 | 47.97 | 0.96 | 145.11 | 125.69 | 50 |
| 28-03-2023 | 21.7 | 7.0 | 0.89 | 74.4 | 24.03 | 52.99 | 0.93 | 170.48 | 136.86 | 74 |
| 29-03-2023 | 20.3 | 4.5 | 0.83 | 68.9 | 23.52 | 58.10 | 0.97 | 173.28 | 127.43 | 69 |
| 30-03-2023 | 18.8 | 7.7 | 0.80 | 52.9 | 23.05 | 51.15 | 1.03 | 176.58 | 135.02 | 53 |
| 31-03-2023 | 14.3 | 8.1 | 0.82 | 57.8 | 23.54 | 45.03 | 0.97 | 172.08 | 135.17 | 58 |
| Minimum | 14.3 | 2.6 | 0.76 | 36.8 | 21.50 | 44.93 | 0.42 | 133.15 | 83.57 | * |
| Maximum | 25.6 | 11.0 | 1.08 | 74.4 | 25.78 | 82.54 | 1.03 | 223.02 | 155.02 | * |
| Average | 21.5 | 6.4 | 0.89 | 51.4 | 23.64 | 59.51 | 0.68 | 171.43 | 130.53 | * |

Hebbal-March-2023

| Date | CO (mg/m³) | Ozone (µg/m³) | NO2 (µg/m³) | NH3 (µg/m³) | SO2 (µg/m³) | PM2.5 (µg/m³) | PM10 (µg/m³) | BEN. (µg/m³) | AT (°C) | RH (%) | WS (m/s) | WD (deg) | BP (mmHg) | AQI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01-03-2023 | 0.45 | 25.0 | 36.2 | 10.2 | 11.0 | 52.0 | 90.6 | 0.2 | 24.6 | 57 | 0.8 | 106 | 643 | 91 |
| 02-03-2023 | 0.37 | 19.0 | 34.6 | 8.9 | 10.9 | 44.0 | 97.3 | 0.1 | 24.1 | 44 | 0.7 | 99 | 699 | 97 |
| 03-03-2023 | 0.45 | 20.7 | 39.5 | 11.8 | 10.5 | 48.8 | 106.9 | 0.2 | 24.2 | 35 | 0.7 | 95 | 714 | 105 |
| 04-03-2023 | 0.39 | 18.3 | 40.4 | 12.3 | 10.4 | 46.5 | 100.2 | 0.1 | 25.0 | 34 | 0.7 | 99 | 714 | 100 |
| 05-03-2023 | 0.44 | 17.2 | 33.6 | 12.5 | 10.4 | 46.4 | 100.0 | 0.2 | 25.5 | 35 | 0.7 | 99 | 714 | 100 |
| 06-03-2023 | 0.39 | 22.5 | 42.8 | 13.4 | 10.7 | 42.7 | 98.4 | 0.1 | 25.2 | 38 | 0.7 | 99 | 713 | 98 |
| 07-03-2023 | 0.46 | 23.6 | 43.4 | 14.2 | 11.1 | 44.0 | 96.1 | 0.2 | 24.6 | 40 | 0.7 | 94 | 695 | 96 |
| 08-03-2023 | 0.38 | 27.0 | 33.8 | 13.5 | 11.0 | 41.4 | 83.7 | 0.1 | 25.1 | 47 | 0.7 | 101 | 710 | 84 |
| 09-03-2023 | 0.42 | 34.9 | 53.4 | 16.3 | 10.8 | 46.9 | 91.2 | 0.1 | 27.0 | 37 | 0.8 | 104 | 708 | 91 |
| 10-03-2023 | 0.41 | 23.2 | 38.4 | 12.2 | 11.1 | 42.5 | 88.0 | 0.1 | 24.6 | 47 | 0.7 | 98 | 711 | 88 |
| 11-03-2023 | 0.43 | 26.6 | 36.9 | 14.3 | 11.0 | 45.7 | 90.4 | 0.1 | 24.8 | 50 | 0.7 | 93 | 711 | 90 |
| 12-03-2023 | 0.41 | 16.6 | 27.0 | 11.0 | 10.7 | 36.0 | 70.7 | 0.1 | 25.2 | 53 | 0.8 | 110 | 710 | 71 |
| 13-03-2023 | 0.46 | 25.8 | 36.1 | 12.9 | 11.0 | 46.8 | 85.7 | 0.2 | 26.1 | 52 | 0.8 | 120 | 654 | 86 |
| 14-03-2023 | 0.51 | 17.5 | 32.1 | 15.7 | 10.5 | 44.8 | 92.6 | 0.2 | 26.7 | 44 | 0.8 | 109 | 601 | 93 |
| 15-03-2023 | 0.40 | 23.6 | 29.8 | 18.9 | 10.1 | 57.9 | 107.0 | 0.1 | 26.8 | 50 | 0.9 | 129 | 625 | 105 |
| 16-03-2023 | 0.37 | 23.0 | 21.7 | 19.4 | 10.1 | 61.5 | 105.0 | 0.1 | 25.5 | 64 | 1.0 | 144 | 657 | 105 |
| 17-03-2023 | 0.17 | 16.0 | 21.0 | 15.7 | 10.6 | 37.9 | 59.8 | 0.1 | 24.7 | 73 | 0.9 | 132 | 715 | 60 |
| 18-03-2023 | 0.33 | 18.6 | 22.6 | 19.0 | 9.3 | 29.5 | 60.8 | 0.1 | 27.5 | 57 | 1.0 | 157 | 708 | 61 |
| 19-03-2023 | 0.32 | 15.1 | 14.8 | 16.3 | 9.0 | 32.0 | 50.5 | 0.1 | 24.8 | 58 | 1.1 | 173 | 708 | 51 |
| 20-03-2023 | 0.69 | 18.2 | 16.4 | 8.2 | 7.2 | 30.1 | 63.9 | 0.2 | 25.9 | 49 | 1.4 | 202 | 695 | 64 |
| 21-03-2023 | 0.55 | 20.7 | 18.4 | 9.5 | 7.8 | 28.1 | 74.5 | 0.2 | 26.0 | 42 | 1.0 | 185 | 698 | 75 |
| 22-03-2023 | 0.38 | 19.5 | 21.1 | 16.3 | 9.4 | 49.6 | 90.1 | 0.1 | 26.6 | 55 | 0.8 | 112 | 709 | 90 |
| 23-03-2023 | 0.44 | 20.3 | 17.2 | 13.5 | 10.0 | - | 70.2 | 0.1 | 26.6 | 53 | 0.8 | 123 | 710 | 70 |
| 24-03-2023 | 0.36 | 23.2 | 16.2 | 12.7 | 10.1 | 16.8 | 60.8 | 0.1 | 27.3 | 54 | 0.9 | 141 | 702 | 61 |
| 25-03-2023 | 0.45 | 21.8 | 20.4 | 10.8 | 10.3 | 17.8 | 68.3 | 0.2 | 26.7 | 60 | 0.9 | 141 | 709 | 68 |
| 26-03-2023 | 0.39 | 18.3 | 18.3 | 9.1 | 10.4 | 17.0 | 65.3 | 0.1 | 27.1 | 55 | 0.9 | 131 | 709 | 65 |
| 27-03-2023 | 0.44 | 18.4 | 17.1 | 8.3 | 10.5 | 17.8 | 68.3 | 0.2 | 26.7 | 49 | 0.9 | 132 | 709 | 68 |
| 28-03-2023 | 0.53 | 25.3 | 28.4 | 11.0 | 10.1 | 23.5 | 90.1 | 0.2 | 28.0 | 47 | 0.8 | 115 | 708 | 90 |
| 29-03-2023 | 0.43 | 26.2 | 18.6 | 12.6 | 9.9 | 23.7 | 87.3 | 0.1 | 26.9 | 63 | 1.0 | 147 | 709 | 87 |
| 30-03-2023 | 0.31 | 27.2 | 15.6 | 10.3 | 10.4 | 15.8 | 60.1 | 0.1 | 26.7 | 59 | 0.9 | 144 | 709 | 60 |
| 31-03-2023 | 0.42 | 32.5 | 19.2 | 11.0 | 10.6 | 25.6 | 63.8 | 0.1 | 27.3 | 57 | 0.9 | 141 | 708 | 64 |
| Average | 0.42 | 22.1 | 27.9 | 13.0 | 10.2 | 37.1 | 81.9 | 0.1 | 25.9 | 50 | 0.9 | 125 | 696 | * |
| Minimum | 0.17 | 15.1 | 14.8 | 8.2 | 7.2 | 15.8 | 50.5 | 0.1 | 24.1 | 34 | 0.7 | 93 | 601 | * |
| Maximum | 0.69 | 34.9 | 53.4 | 19.4 | 11.1 | 61.5 | 107.0 | 0.2 | 28.0 | 73 | 1.4 | 202 | 715 | * |

Jayanagar-March-2023

| Date | CO mg/m³ | Ozone µg/m³ | NO2 µg/m³ | NH3 µg/m³ | SO2 µg/m³ | PM2.5 µg/m³ | PM10 µg/m³ | BEN µg/m³ | AT °C | RH % | WS m/s | WD deg. | BP mmHg | AQI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01-03-2023 | 0.46 | 28.2 | 25.2 | 7.7 | 2.8 | 48.5 | 91.6 | 0.26 | 24.7 | 56 | 0.7 | 116 | 714 | 92 |
| 02-03-2023 | 0.29 | 22.9 | 28.9 | 7.6 | 2.7 | 43.2 | 101.4 | 0.16 | 24.3 | 45 | 0.8 | 122 | 714 | 101 |
| 03-03-2023 | 0.32 | 22.8 | 32.3 | 7.6 | 2.7 | 48.1 | 121.0 | 0.18 | 24.3 | 34 | 0.7 | 130 | 715 | 114 |
| 04-03-2023 | 0.40 | 30.1 | 34.4 | 7.5 | 2.8 | 47.5 | 112.3 | 0.21 | 25.2 | 32 | 0.7 | 126 | 714 | 108 |
| 05-03-2023 | 0.34 | 35.8 | 37.7 | 7.4 | 2.3 | 47.8 | 115.9 | 0.19 | 25.8 | 33 | 0.7 | 118 | 714 | 111 |
| 06-03-2023 | 0.34 | 42.7 | 35.3 | 7.2 | 2.9 | 40.5 | 98.1 | 0.19 | 25.4 | 37 | 0.7 | 114 | 713 | 98 |
| 07-03-2023 | 0.36 | 31.3 | 38.4 | 7.2 | 3.2 | 40.7 | 94.1 | 0.20 | 25.1 | 38 | 0.7 | 126 | 712 | 94 |
| 08-03-2023 | 0.32 | 21.4 | 35.5 | 7.8 | 2.8 | 44.0 | 93.0 | 0.18 | 25.0 | 46 | 0.8 | 114 | 712 | 93 |
| 09-03-2023 | 0.46 | 29.6 | 36.5 | 7.9 | 2.5 | 48.5 | 91.5 | 0.26 | 24.4 | 45 | 0.6 | 108 | 711 | 92 |
| 10-03-2023 | 0.38 | 28.9 | 38.7 | 7.8 | 2.8 | 41.8 | 99.3 | 0.21 | 24.8 | 44 | 0.7 | 117 | 711 | 99 |
| 11-03-2023 | 0.44 | 32.0 | 40.9 | 7.7 | 2.7 | 47.9 | 112.4 | 0.25 | 25.1 | 48 | 0.6 | 119 | 711 | 108 |
| 12-03-2023 | 0.36 | 31.5 | 41.2 | 7.5 | 3.1 | 39.9 | 88.6 | 0.20 | 25.5 | 52 | 0.7 | 114 | 711 | 89 |
| 13-03-2023 | 0.43 | 33.8 | 35.3 | 7.8 | 2.2 | 54.3 | 115.6 | 0.24 | 26.2 | 52 | 0.7 | 122 | 710 | 110 |
| 14-03-2023 | 0.46 | 28.9 | 36.8 | 7.6 | 2.6 | 48.5 | 115.6 | 0.26 | 26.9 | 43 | 0.6 | 126 | 708 | 110 |
| 15-03-2023 | 0.33 | 29.2 | 35.9 | 8.1 | 3.0 | 59.2 | 125.7 | 0.19 | 27.1 | 48 | 0.7 | 150 | 708 | 117 |
| 16-03-2023 | 0.31 | 30.9 | 32.9 | 8.2 | 3.1 | 58.3 | 98.6 | 0.17 | 25.5 | 63 | 0.9 | 173 | 709 | 99 |
| 17-03-2023 | 0.41 | 35.3 | 35.4 | 7.5 | 3.0 | 40.2 | 76.3 | 0.23 | 24.8 | 68 | 0.6 | 155 | 709 | 76 |
| 18-03-2023 | 0.26 | 34.9 | 34.4 | 7.4 | 2.5 | 29.1 | 58.5 | 0.15 | 24.7 | 68 | 0.8 | 166 | 710 | 59 |
| 19-03-2023 | 0.20 | 35.1 | 29.9 | 7.4 | 2.7 | 32.1 | 53.0 | 0.11 | 25.7 | 55 | 1.4 | 218 | 707 | 53 |
| 20-03-2023 | 0.19 | 34.9 | - | - | 2.8 | 27.6 | 51.0 | 0.10 | 26.0 | 48 | 1.2 | 221 | 707 | 51 |
| 21-03-2023 | 0.46 | 34.7 | - | - | 3.1 | 30.9 | 91.5 | 0.26 | 25.8 | 42 | 0.9 | 223 | 707 | 92 |
| 22-03-2023 | 0.26 | 35.0 | - | - | 2.3 | 44.2 | 96.3 | 0.14 | 27.2 | 46 | 0.6 | 191 | 708 | 96 |
| 23-03-2023 | 0.51 | 28.9 | 6.1 | 4.9 | 3.5 | 35.8 | 81.4 | 0.28 | 27.2 | 51 | 0.8 | 194 | 708 | 81 |
| 24-03-2023 | 0.25 | 33.2 | 7.2 | 5.4 | 1.6 | 31.1 | 71.6 | 0.14 | 27.6 | 54 | 0.8 | 184 | 708 | 72 |
| 25-03-2023 | 0.24 | 26.5 | 7.6 | 7.0 | 3.3 | 31.2 | 81.9 | 0.13 | 26.8 | 58 | 0.9 | 174 | 709 | 82 |
| 26-03-2023 | 0.16 | 36.9 | 7.4 | 7.0 | 2.6 | 32.9 | 77.9 | 0.09 | 27.4 | 52 | 1.0 | 204 | 709 | 78 |
| 27-03-2023 | - | - | - | - | - | - | - | - | - | - | - | - | - | * |
| 28-03-2023 | - | - | - | - | - | - | - | - | - | - | - | - | - | * |
| 29-03-2023 | 0.30 | 21.7 | 7.6 | 6.9 | 4.3 | 33.3 | 79.0 | 0.17 | 29.4 | 49 | 1.1 | 192 | 709 | 79 |
| 30-03-2023 | 0.38 | 21.6 | 7.5 | 7.0 | 3.2 | 31.3 | 68.8 | 0.21 | 27.3 | 54 | 1.0 | 201 | 709 | 69 |
| 31-03-2023 | 0.33 | 21.7 | 8.7 | 7.6 | 2.9 | 34.3 | 76.8 | 0.18 | 27.3 | 55 | 1.0 | 184 | 708 | 77 |
| Average | 0.34 | 30.4 | 27.6 | 7.3 | 2.8 | 41.1 | 91.0 | 0.19 | 25.9 | 49 | 0.8 | 155 | 710 | * |
| Maximum | 0.51 | 42.7 | 41.2 | 8.2 | 4.3 | 59.2 | 125.7 | 0.28 | 29.4 | 68 | 1.4 | 223 | 715 | * |
| Minimum | 0.16 | 21.4 | 6.1 | 4.9 | 1.6 | 27.6 | 51.0 | 0.09 | 24.3 | 32 | 0.6 | 108 | 707 | * |

| KAVIKA-March-2023 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | CO mg/m³ | Ozone µg/m³ | NO2 µg/m³ | NH3 µg/m³ | SO2 µg/m³ | PM2.5 µg/m³ | PM10 µg/m³ | BEN µg/m³ | AT °C | RH % | WS m/s | WD deg. | BP mmHg | AQI |
| 01-03-2023 | 0.98 | 21.0 | 62.3 | 75.6 | 5.8 | 33.7 | 96.3 | 0.14 | 25.4 | 43 | 0.3 | 129 | 725 | 96 |
| 02-03-2023 | 0.84 | 21.4 | 75.2 | 70.1 | 5.9 | 36.8 | 105.1 | 0.12 | 25.3 | 43 | 0.3 | 124 | 725 | 103 |
| 03-03-2023 | 0.89 | 22.1 | 84.2 | 77.8 | 6.9 | 40.3 | 114.8 | 0.12 | 25.5 | 43 | 0.3 | 131 | 726 | 110 |
| 04-03-2023 | 0.98 | 21.8 | 86.0 | 96.1 | 6.5 | 36.7 | 104.5 | 0.13 | 26.2 | 43 | 0.3 | 127 | 725 | 103 |
| 05-03-2023 | 0.82 | 21.3 | 74.7 | 87.4 | 6.0 | 32.9 | 93.8 | 0.11 | 26.8 | 43 | 0.3 | 130 | 725 | 94 |
| 06-03-2023 | 0.91 | 20.6 | 77.0 | 88.8 | 5.8 | 35.9 | 102.8 | 0.13 | 26.4 | 43 | 0.3 | 131 | 723 | 102 |
| 07-03-2023 | 0.91 | 20.3 | 74.1 | 86.8 | 6.7 | 34.5 | 97.8 | 0.13 | 25.9 | 43 | 0.3 | 131 | 723 | 98 |
| 08-03-2023 | 0.84 | 20.2 | 70.9 | 79.7 | 6.4 | 33.6 | 95.8 | 0.12 | 25.7 | 43 | 0.3 | 126 | 723 | 96 |
| 09-03-2023 | 0.91 | 21.1 | 81.5 | 80.2 | 5.9 | 34.5 | 97.1 | 0.31 | 25.4 | 43 | 0.3 | 128 | 722 | 97 |
| 10-03-2023 | 0.82 | 20.6 | 71.3 | 79.9 | 5.9 | 36.3 | 96.9 | 0.11 | 25.7 | 43 | 0.3 | 133 | 722 | 97 |
| 11-03-2023 | 1.01 | 20.7 | 76.3 | 100.9 | 6.5 | 37.3 | 106.6 | 0.14 | 25.9 | 43 | 0.3 | 126 | 721 | 104 |
| 12-03-2023 | 0.73 | 20.0 | 49.9 | 82.3 | 5.6 | 26.8 | 76.5 | 0.10 | 26.1 | 43 | 0.3 | 129 | 721 | 77 |
| 13-03-2023 | 0.87 | 19.2 | 49.0 | 83.7 | 5.0 | 28.3 | 80.5 | 0.12 | 27.2 | 43 | 0.3 | 143 | 720 | 81 |
| 14-03-2023 | 0.82 | 20.8 | 45.9 | 70.4 | 5.4 | 107.8 | 98.9 | 0.12 | 23.6 | 44 | 0.3 | 129 | 720 | 99 |
| 15-03-2023 | - | - | - | - | - | - | - | - | - | - | - | - | - | * |
| 16-03-2023 | 1.08 | 8.5 | 49.4 | 90.9 | 5.0 | 28.7 | 79.6 | 0.15 | 28.4 | 42 | 0.3 | 179 | 718 | 80 |
| 17-03-2023 | 0.52 | 17.8 | 45.7 | 73.7 | 5.1 | 25.0 | 68.2 | 0.03 | 26.2 | 43 | 0.3 | 168 | 719 | 68 |
| 18-03-2023 | 0.47 | 17.7 | 33.9 | 62.0 | 5.7 | 17.2 | 49.0 | - | 26.1 | 43 | 0.3 | 167 | 718 | 49 |
| 19-03-2023 | 0.30 | 18.2 | 34.5 | 61.7 | 5.9 | 47.0 | 53.8 | 0.43 | 26.8 | 43 | 0.3 | 238 | 717 | 54 |
| 20-03-2023 | 0.79 | 19.6 | 29.6 | 53.0 | 5.6 | 21.3 | 60.8 | 0.03 | 23.1 | 43 | 0.3 | 247 | 717 | 61 |
| 21-03-2023 | 1.03 | - | - | - | 6.0 | - | - | 0.14 | - | 43 | 0.3 | 239 | 717 | * |
| 22-03-2023 | 0.88 | - | - | - | 5.9 | - | - | 0.12 | 0.0 | 43 | 0.3 | 204 | 718 | * |
| 23-03-2023 | 1.03 | - | - | - | 5.7 | - | - | 0.14 | - | 43 | 0.3 | 228 | 718 | * |
| 24-03-2023 | 0.67 | - | - | - | 5.5 | - | - | 0.09 | 5.5 | 43 | 0.3 | 224 | 718 | * |
| 25-03-2023 | 0.24 | 24.0 | 32.0 | 77.0 | 4.9 | 23.7 | 49.5 | 0.04 | 10.8 | 43 | 0.3 | 212 | 719 | 50 |
| 26-03-2023 | 2.00 | 25.2 | 38.0 | 63.6 | 4.5 | 25.8 | 68.8 | 0.28 | 28.4 | 43 | 0.3 | 228 | 719 | 69 |
| 27-03-2023 | 1.27 | 21.9 | 60.9 | 79.2 | 5.1 | 33.4 | 93.0 | 0.18 | 28.9 | 43 | 0.3 | 211 | 719 | 93 |
| 28-03-2023 | 1.04 | 23.5 | 72.3 | 100.6 | 5.0 | 38.5 | 110.0 | 0.15 | 30.3 | 43 | 0.3 | 174 | 718 | 107 |
| 29-03-2023 | 2.88 | 22.2 | 38.2 | 83.4 | 5.1 | 54.0 | 73.8 | 0.40 | 28.8 | 43 | 0.3 | 214 | 719 | 74 |
| 30-03-2023 | 0.98 | 22.3 | 49.6 | 75.4 | 5.5 | 25.3 | 72.1 | 0.14 | 28.1 | 43 | 0.3 | 231 | 719 | 72 |
| 31-03-2023 | 0.97 | 23.1 | 46.4 | 72.4 | 5.4 | 24.8 | 70.9 | 0.14 | 28.5 | 43 | 0.3 | 220 | 718 | 71 |
| Average | 0.95 | 20.6 | 58.0 | 79.0 | 5.7 | 35.4 | 85.3 | 0.15 | 24.3 | 43 | 0.3 | 173 | 720 | * |
| Maximum | 2.88 | 25.2 | 86.0 | 100.9 | 6.9 | 107.8 | 114.8 | 0.43 | 30.3 | 44 | 0.3 | 247 | 726 | * |
| Minimum | 0.24 | 8.5 | 29.6 | 53.0 | 4.5 | 17.2 | 49.0 | 0.03 | 0.0 | 42 | 0.3 | 124 | 717 | * |

| NIMHANS-March-2023 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | CO mg/m³ | Ozone µg/m³ | NO2 µg/m³ | NH3 µg/m³ | SO2 µg/m³ | PM2.5 µg/m³ | PM10 µg/m³ | BEN µg/m³ | AT °C | RH % | WS m/s | WD deg. | BP mmHg | AQI |
| 01-03-2023 | 0.50 | 31.7 | 22.6 | 2.0 | 4.6 | 37.7 | 94.4 | 0.06 | 23.7 | 59 | 1.4 | 129 | 713 | 94 |
| 02-03-2023 | 0.30 | 34.7 | 22.5 | 2.0 | 4.7 | 37.0 | 92.5 | 0.03 | 23.5 | 47 | 1.5 | 129 | 714 | 93 |
| 03-03-2023 | 0.34 | 33.6 | 26.9 | 2.0 | 5.7 | 48.5 | 121.2 | 0.04 | 23.5 | 37 | 1.3 | 131 | 714 | 114 |
| 04-03-2023 | 0.30 | 30.8 | 27.1 | 1.9 | 5.4 | 38.0 | 94.9 | 0.03 | 24.2 | 34 | 1.2 | 131 | 716 | 95 |
| 05-03-2023 | 0.38 | 36.1 | 23.3 | 2.0 | 5.6 | 34.9 | 87.0 | 0.04 | 24.9 | 35 | 1.2 | 126 | 713 | 87 |
| 06-03-2023 | 0.44 | 27.7 | 27.1 | 2.0 | 5.0 | 41.8 | 104.1 | 0.05 | 24.3 | 40 | 1.2 | 131 | 714 | 103 |
| 07-03-2023 | 0.30 | 26.3 | 25.4 | 2.0 | 5.4 | 38.0 | 95.2 | 0.03 | 24.1 | 42 | 1.1 | 137 | 714 | 95 |
| 08-03-2023 | 0.47 | 30.5 | 23.2 | 2.0 | 5.1 | 35.0 | 87.7 | 0.05 | 24.0 | 49 | 1.3 | 127 | 711 | 88 |
| 09-03-2023 | 0.46 | 33.0 | 24.7 | 2.0 | 4.1 | 34.9 | 87.3 | 0.05 | 23.6 | 47 | 1.0 | 128 | 713 | 87 |
| 10-03-2023 | 0.50 | 28.6 | 24.8 | 2.0 | 4.0 | 35.4 | 88.6 | 0.06 | 23.8 | 49 | 1.1 | 134 | 710 | 89 |

11-03-2023 0.31 29.6 24.0 2.0 5.2 36.5 91.0 0.03 24.1 50 1.1 128 710 91

| 12-03-2023 | 0.90 | 23.8 | 19.9 | 2.0 | 3.8 | 25.6 | 64.0 | 0.10 | 24.4 | 55 | 1.2 | 131 | 710 | 64 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13-03-2023 | 0.53 | 30.0 | 21.7 | 2.0 | 3.6 | 32.2 | 80.7 | 0.06 | 25.4 | 54 | 1.3 | 140 | 709 | 81 |
| 14-03-2023 | 0.40 | 27.4 | 26.5 | 2.0 | 3.7 | 34.3 | 85.8 | 0.12 | 26.0 | 47 | 0.8 | 144 | 710 | 86 |
| 15-03-2023 | 0.41 | 35.0 | 22.4 | 2.0 | 4.0 | 39.5 | 98.4 | 0.05 | 26.2 | 53 | 0.9 | 157 | 707 | 98 |
| 16-03-2023 | 0.58 | 31.1 | 19.6 | 2.1 | 4.7 | 38.9 | 97.0 | 0.07 | 25.3 | 65 | 1.1 | 183 | 708 | 97 |
| 17-03-2023 | 0.49 | 21.6 | 24.5 | 2.0 | 3.8 | 27.5 | 68.8 | 0.05 | 24.5 | 69 | 0.8 | 159 | 708 | 69 |
| 18-03-2023 | 0.32 | 20.2 | 20.9 | 2.0 | 4.3 | 20.6 | 51.7 | 0.03 | 24.6 | 67 | 1.1 | 184 | 707 | 52 |
| 19-03-2023 | 0.24 | 21.3 | 18.1 | 2.0 | 3.1 | 20.9 | 49.0 | 0.03 | 25.8 | 55 | 2.0 | 279 | 707 | 49 |
| 20-03-2023 | 0.28 | 20.5 | 19.3 | 2.0 | 3.2 | 18.0 | 45.2 | 0.03 | 26.1 | 49 | 1.7 | 278 | 709 | 45 |
| 21-03-2023 | 0.48 | 21.0 | 23.6 | 1.9 | 3.9 | 26.6 | 66.7 | 0.05 | 25.6 | 44 | 1.2 | 239 | 709 | 67 |
| 22-03-2023 | 0.30 | 34.4 | 24.4 | 2.0 | 3.9 | 34.0 | 84.9 | 0.03 | 26.7 | 50 | 0.6 | 188 | 707 | 85 |
| 23-03-2023 | 0.40 | 30.8 | 22.2 | 2.0 | 4.6 | 29.0 | 72.6 | 0.05 | 26.9 | 53 | 1.0 | 218 | 707 | 73 |
| 24-03-2023 | 0.26 | 36.6 | 18.7 | 2.0 | 4.2 | 24.3 | 60.8 | 0.03 | 27.7 | 54 | 1.1 | 209 | 707 | 61 |
| 25-03-2023 | 0.37 | 32.0 | 18.9 | 2.0 | 4.1 | 23.9 | 59.7 | 0.04 | 26.8 | 58 | 1.1 | 187 | 709 | 60 |
| 26-03-2023 | 0.25 | 31.5 | 19.2 | 2.0 | 3.8 | 24.8 | 62.1 | 0.03 | 27.5 | 53 | 1.1 | 225 | 708 | 62 |
| 27-03-2023 | 0.48 | 26.2 | 24.7 | 1.9 | 3.3 | 29.1 | 72.8 | 0.06 | 27.5 | 44 | 1.3 | 207 | 708 | 73 |
| 28-03-2023 | 0.33 | 31.1 | 28.8 | 1.9 | 4.4 | 39.1 | 97.7 | 0.04 | 28.2 | 45 | 0.9 | 149 | 708 | 98 |
| 29-03-2023 | 0.52 | 32.3 | 19.8 | 2.0 | 4.4 | 31.8 | 79.7 | 0.05 | 27.5 | 59 | 1.3 | 203 | 709 | 80 |
| 30-03-2023 | 0.33 | 36.4 | 18.4 | 2.0 | 3.7 | 24.1 | 60.1 | 0.04 | 27.1 | 55 | 1.4 | 228 | 708 | 60 |
| 31-03-2023 | 0.48 | 34.1 | 22.6 | 2.0 | 3.8 | 25.2 | 63.3 | 0.05 | 27.2 | 56 | 1.2 | 221 | 707 | 63 |
| Average | 0.41 | 29.7 | 22.8 | 2.0 | 4.3 | 31.8 | 79.5 | 0.05 | 25.5 | 51 | 1.2 | 173 | 710 | * |
| Maximum | 0.90 | 36.6 | 28.8 | 2.1 | 5.7 | 48.5 | 121.2 | 0.12 | 28.2 | 69 | 2.0 | 279 | 716 | * |
| Minimum | 0.24 | 20.2 | 18.1 | 1.9 | 3.1 | 18.0 | 45.2 | 0.03 | 23.5 | 34 | 0.6 | 126 | 707 | * |

| Silkboard-March-2023 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | CO mg/m³ | Ozone µg/m³ | NO2 µg/m³ | NH3 µg/m³ | SO2 µg/m³ | PM2.5 µg/m³ | PM10 µg/m³ | BEN µg/m³ | AT °C | RH % | WS m/s | WD deg. | BP mmHg | AQI |
| 01-03-2023 | 0.57 | 38.6 | 12.8 | 3.2 | 5.4 | 49.1 | 99.7 | 0.15 | 23.7 | 57 | 1.6 | 86 | 692 | 100 |
| 02-03-2023 | 0.45 | 39.1 | 13.6 | 3.6 | 5.4 | 44.6 | 98.1 | 0.11 | 23.4 | 46 | 1.5 | 91 | 692 | 98 |
| 03-03-2023 | 0.59 | 35.6 | 17.1 | 3.2 | 5.8 | 56.5 | 124.3 | 0.15 | 23.2 | 36 | 1.4 | 127 | 693 | 116 |
| 04-03-2023 | 0.60 | 34.0 | 16.5 | 3.7 | 5.8 | 49.6 | 109.1 | 0.15 | 24.2 | 34 | 1.3 | 124 | 693 | 106 |
| 05-03-2023 | 0.72 | 39.7 | 14.6 | 3.2 | 5.9 | 48.0 | 105.7 | 0.18 | 24.8 | 35 | 1.4 | 113 | 692 | 104 |
| 06-03-2023 | 0.60 | 34.8 | 13.8 | 2.2 | 5.7 | 43.8 | 96.5 | 0.15 | 24.3 | 38 | 1.4 | 123 | 692 | 97 |
| 07-03-2023 | 0.59 | 32.0 | 14.9 | 3.7 | 5.7 | 42.5 | 93.5 | 0.15 | 24.3 | 40 | 1.3 | 87 | 691 | 94 |
| 08-03-2023 | 0.44 | 36.5 | 12.1 | 3.0 | 5.8 | 42.2 | 92.8 | 0.11 | 24.0 | 47 | 1.5 | 88 | 691 | 93 |
| 09-03-2023 | 0.62 | 36.3 | 13.6 | 3.2 | 5.6 | 46.4 | 102.0 | 0.15 | 23.7 | 47 | 1.2 | 96 | 690 | 101 |
| 10-03-2023 | 0.56 | 27.6 | 14.7 | 2.9 | 5.5 | 50.6 | 110.8 | 0.14 | 23.0 | 49 | 1.2 | 101 | 691 | 107 |
| 11-03-2023 | 0.58 | 30.5 | 15.3 | 3.2 | 5.7 | 51.3 | 112.8 | 0.14 | 24.1 | 48 | 1.3 | 95 | 690 | 109 |
| 12-03-2023 | 0.46 | 29.4 | 11.6 | 2.9 | 5.7 | 34.1 | 75.1 | 0.12 | 24.4 | 53 | 1.4 | 91 | 690 | 75 |
| 13-03-2023 | 0.59 | 35.9 | 11.2 | 2.8 | 5.5 | 43.7 | 100.0 | 0.15 | 25.4 | 53 | 1.3 | 98 | 689 | 100 |
| 14-03-2023 | 0.61 | 33.3 | 12.7 | 2.5 | 5.6 | 43.7 | 109.4 | 0.15 | 26.2 | 44 | 1.1 | 92 | 688 | 106 |
| 15-03-2023 | 0.58 | 38.8 | 12.7 | 2.9 | 5.4 | 54.5 | 136.5 | 0.13 | 26.5 | 48 | 1.2 | 111 | 688 | 124 |
| 16-03-2023 | 0.71 | 32.9 | 12.2 | 2.6 | 5.7 | 56.5 | 141.4 | 0.18 | 25.0 | 64 | 1.4 | 145 | 688 | 128 |
| 17-03-2023 | 0.58 | 28.5 | 13.3 | 3.4 | 5.4 | 39.0 | 97.1 | 0.15 | 24.4 | 67 | 1.0 | 107 | 689 | 97 |
| 18-03-2023 | 0.56 | 24.2 | 14.9 | 4.4 | 5.5 | 31.1 | 78.0 | 0.14 | 24.2 | 67 | 1.2 | 140 | 688 | 78 |
| 19-03-2023 | 0.75 | 22.3 | 17.3 | 4.3 | 5.2 | 32.5 | 81.5 | 0.19 | 26.0 | 54 | 1.6 | 227 | 687 | 82 |
| 20-03-2023 | 0.80 | 21.1 | 18.1 | 3.8 | 5.5 | 33.0 | 82.8 | 0.20 | 26.4 | 46 | 1.5 | 232 | 687 | 83 |
| 21-03-2023 | 1.03 | 21.0 | 19.2 | 3.1 | 5.5 | 40.0 | 100.4 | 0.26 | 26.3 | 40 | 1.2 | 206 | 687 | 100 |
| 22-03-2023 | 0.54 | 25.8 | 13.9 | 2.2 | 5.7 | 41.4 | 99.6 | 0.13 | 25.9 | 49 | 1.0 | 141 | 688 | 100 |

23-03-2023 0.88 24.0 15.2 1.9 5.8 37.6 90.3 0.21 27.3 50 1.3 174 688 90

| 24-03-2023 | 0.67 | 34.5 | 12.5 | 1.8 | 5.5 | 32.3 | 77.5 | 0.16 | 27.5 | 53 | 1.3 | 169 | 687 | 78 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25-03-2023 | 0.54 | 34.6 | 10.8 | 1.7 | 5.7 | 31.1 | 74.7 | 0.14 | 26.7 | 57 | 1.3 | 145 | 689 | 75 |
| 26-03-2023 | 0.70 | 50.1 | 12.4 | 1.9 | 5.6 | 32.8 | 78.5 | 0.17 | 27.5 | 51 | 1.2 | 182 | 688 | 79 |
| 27-03-2023 | 1.09 | 39.8 | 15.9 | 1.5 | 5.8 | 39.8 | 95.6 | 0.28 | 27.4 | 44 | 1.3 | 176 | 688 | 96 |
| 28-03-2023 | 0.86 | 33.4 | 14.8 | 2.0 | 5.8 | 47.3 | 113.3 | 0.21 | 28.3 | 45 | 1.0 | 122 | 688 | 109 |
| 29-03-2023 | 0.77 | 22.7 | 11.2 | 1.3 | 5.7 | 46.9 | 112.6 | 0.20 | 27.4 | 58 | 1.4 | 158 | 689 | 108 |
| 30-03-2023 | 0.80 | 27.7 | 13.1 | 1.5 | 5.8 | 36.1 | 86.5 | 0.20 | 27.2 | 53 | 1.4 | 202 | 688 | 87 |
| 31-03-2023 | 0.73 | 28.9 | 12.6 | 1.5 | 5.5 | 35.0 | 84.1 | 0.18 | 27.2 | 53 | 1.3 | 172 | 687 | 84 |
| Average | 0.66 | 32.0 | 14.0 | 2.7 | 5.6 | 42.4 | 98.7 | 0.17 | 25.5 | 49 | 1.3 | 136 | 689 | * |
| Maximum | 1.09 | 50.1 | 19.2 | 4.4 | 5.9 | 56.5 | 141.4 | 0.28 | 28.3 | 67 | 1.6 | 232 | 693 | * |
| Minimum | 0.44 | 21.0 | 10.8 | 1.3 | 5.2 | 31.1 | 74.7 | 0.11 | 23.0 | 34 | 1.0 | 86 | 687 | * |

Possible Health Impacts

| Good | Minimal Impact |
| --- | --- |
| Satisfactory | Minor breathing discomfort to sensitive people |
| Moderate | Breathing discomfort to the people with lungs, asthma and heart diseases |
| Poor | Breathing discomfort to most people on prolonged exposure |
| Very Poor | Respiratory illness on prolonged exposure |
| Severe | Affects healthy people and seriously impacts those with existing diseases |

INFERENCE:

The Air Quality monitored through the 7 CAAQMs (Continuous Ambient Air

Quality Monitoring Stations) installed in Bengaluru at City  Railway Station, Hebbal, Saneguruvanahalli, NIMHANS, Jayanagar, KAVIKA Mysore Road and Central Silk Board-HSR Layout, for the month of March-2023 reveals that the Air quality is Good to Moderate in almost all days over the number of  days monitored.

-----*****-----